3GPP TSG-RAN WG4 Meeting # 107											                R4-2310259

Incheon Korea, 22 May –26 May, 2023

Agenda item:			8.7.3

Source:	Nokia

Title:	WF on beam correspondence in initial access and RRC_INACTIVE

Document for:	Approval

# Topic #1: Beam correspondence requirement applicability

Issue 1-1-1: minimum peak EIRP for msg1 + spherical coverage package

- Proposals

  - Option 1: Do not specify the min peak EIRP requirements but Same spherical coverage as RRC_Connected mode (Huawei, Apple, MediaTek)

  - Option 2: Lower than the min peak EIRP of RRC_Connected mode  + Same spherical coverage as RRC_Connected mode (5-7 dB lower by Xiaomi)

  - Option 3: Same as min peak EIRP of RRC_Connected mode  + Same spherical coverage as RRC_Connected mode (Sony, CMCC, Nokia, ZTE)

  - Option 4: Develop RF requirements based on different UE capabilities for beam correspondence in initial access:

- For UE support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, same beam correspondence requirement in RRC_CONNECTED is applied.

- For UE doesn’t support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, 7 dB relaxation is applied to both min peak EIRP and spherical coverage. (vivo)

- Agreement:

  - msg1 spherical coverage requirement will be further discussed

- Enable multiple PRACH transmission in testing mode, including holding RAR.

- The accumulative period of measurement for PRACH transmission shall be at least 1 ms.

- The requirement is verified with the test metric of EIRP (Link=spherical coverage grid, Meas=Link angle). (Vivo, Huawei, mediaTek, Apple, xiaomi, Ericsson, Samsung, ZTE)

  - No minimum peak EIRP requirement will be specified for initial access and RRC INACTIVE STATE

- WF

  - Power tolerance due to open loop power control in initial access is for further discussion.

    - Option 1: No tolerance is introduced.

    - Option 2: The power tolerance should be introduced according to TS 38.101-2 clause 6.3.4.2, 6.3.4.3, or 6.3.4.4 in the following.

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n259 | 5.8 |
| n260 | 8 |
| n261 | 11.5 |
| n262 | 2.9 |
| n263 | 2.3 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance for RRC_CONNECTED STATE NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 4: For initial access and RA/CG-SDT in RRC_INACTIVE STATE, the power tolerance specified in 6.3.4.2, 6.3.4.3 or 6.3.4.4 should be applied on top of minimum EIRP at 50%-tile CDF. The applicability rules of the power tolerance are defined in 6.3.4.2, 6.3.4.3 and 6.3.4.4, respectively. |  |

    - Option 3: Relaxation X dB is introduced to min EIRP at 50%-tile CDF.

  - FFS for Beam lock in the core requirement.

  - FFS whether RAR reception-based BC test is needed.

Sub-topic 1-4 UE capability

- Proposals

  - Option 1: Only the UE support both beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered can support Rel-18 msg1 beam correspondence in RRC_INACTIVE and initial access. (Huawei, MediaTek, ZTE)

  - Option 2: The R-15 and forward release new UE supporting beamCorrespondenceWithoutUL-BeamSweeping can support msg1 beam correspondence. (Xiaomi)

  - Option 3: The feature of supporting beam correspondence in initial access is only introduced for R18 onward UE. (vivo)

  - Option 4: RAN4 to further discuss if Rel-18 msg1 beam correspondence requirements are mandatory or optional. If it is mandatory for all UE regardless of UE capability, then it should not be release independent to previous releases. (Samsung)

- WF

  - Apply for Rel-18 onward UE.

  - Optional or mandatory is FFS

.

Sub-topic 1-5 UE side condition

- Proposals

  - Option 1: RAN4 shall also determine the side condition of SSB for the EIRP spherical coverage test of Msg. 1, to match the condition in the field. (Sony)

  - Option 2: The side condition for SSB based beam correspondence could be reused for spherical coverage requirements. (Huawei)

  - Option 3: Rel-16 side condition of SSB can be reused for BC in IA, if it doesn’t need to relax EIS spherical coverage and SSB Ês/Iot in initial access compared to connected mode. (Xiaomi)

  - Option 4: Different side conditions are applied based on the UE capability during the verification:

    - For UE support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, SSB SNR≥ 6 dB can be reused.

    - For UE doesn’t support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, SSB SNR should be adjusted to ≥ 13 dB (vivo)

- Agreement

  - the side condition in 38.101-2 table 6.6.4.3.1-1 (Apple, Huawei, Vivo, Xiaomi, MediaTek, Nokia, Samsung)

- Table 6.6.4.3.1-1: Conditions for SSB based L1-RSRP measurements for beam correspondence

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -96.2 | ≥6 |
|  | n258 | -96.2 |  |
|  | n259 | -90.7 |  |
|  | n260 | -91.9 |  |
|  | n261 | -96.2 |  |
|  | n262 | -88.5 |  |
|  | n263 | -88.0 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum SSB_RP values for all angles are increased by MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1. NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |