3GPP TSG-RAN WG4 Meeting # 107	R4-2310278

Incheon, KR, May 22nd – May 26th, 2023

Title: 	WF on 4Tx UE RF requirements

Agenda Item:	8.5.4

Source: 	vivo

Document for:	Approval

Way Forward

Stage-2 Test Configurations

Power class and Reference architecture for Stage-2 configurations.

Agreement:

- In Rel-18, focus on power class 1.5.

- In Stage-2 of Rel-18 WI, only consider 4x23, 4x26 and 2x23 + 2x26.

Verification of full power rank 2 UL operation

Agreement:

- Verify full power rank2 UL operation for:

  - 4 X 23 Tx architecture in mode 1 operation

  - 2 X 23 + 2 X 26 Tx architecture in mode 1 and mode 2 operation

  - 4 X 26 Tx architecture in mode 1 and mode 2 operation

New Test configurationsFull power Mode 1, 2-layer

Agreement:

- In Rel-18, specify the requirements for

  - Full power Mode 1, 2-layer with TPMI index 6 and other parameters as shown below

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | CP-OFDM | 2 | 4 | 6 |

Note: This configuration would be applied to reference architectures: 4x23, 2x23+2x26, 4x26;

  - Full power Mode 2, 1-layer and other parameters as shown below

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 4 | 4, 5, 6 ,7 or 4, 5, 6 ,7, 8, 9, 10, 11NOTE |
| NOTE: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. |  |  |  |  |  |  |

Note: These configurations would applied to reference architectures: 2x23+2x26, 4x26.

  - Full power Mode 2, 2-layer and other parameters as shown below

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-2 | Codebook based uplink | DCI format 0_1 | CP-OFDM | 2 | 4 | 1 or 0, 1, 2, 3, 4 ,5NOTE |
| NOTE: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. |  |  |  |  |  |  |

Note: These configurations would applied to reference architectures: 2x23+2x26, 4x26.

Applicability of Stage-2 UE architecture’s conformance to Stage-1 test configurations.

Agreement:

- Confirm stage-2 reference architectures can also satisfy the stage-1 test configurations (rank = 4 and 1-layer ULFPTx mode 1).

Other requirements

UL-MIMO coherence for 4Tx

- Proposals

  - Option 1: Extend the UL-MIMO coherence requirements for 4Tx requirements. (As in R4-2308238)

  - Option 2: Others

- WF

  - Further study this issue in next meeting.

PCMAX,c tolerance for 4Tx

Agreement:

- Use the following table as a as a starting point of PCMAX tolerance for 4Tx-based UL-MIMO / TxDiversity. and further discuss the values to be specified

- Table 4: PCMAX,c tolerance for 4 Tx UL-MIMO

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 26 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 25 ≤ PCMAX,c < 26 | 5.0 | 2.0 |
| 24 ≤ PCMAX,c < 25 | 5.0 | 3.0 |
| 23 ≤ PCMAX,c < 24 | 5.0 | 4.0 |
| 19 ≤ PCMAX,c < 23 | 5.0 |  |
| 14 ≤ PCMAX,c < 19 | 6.0 |  |
| -40 ≤ PCMAX,c < 14 | 7.0 |  |

EVM

Agreement:

- Allow the use of the EVM formula defined in 1-stage draft CR obtain the total EVM for Stage-2 Tx architectures (2x23 dBm + 2x26 dBm and 4x26 dBm)

Whether non-full power cases is need to be verified or not?

Agreement:

- Non-full power cases do not need be verified.

Power Scaling and other non-full power cases

Whether sending LS to ask RAN1’s view about the possible confusion or misalignment of PHR calculation related to power scaling issue, and possible impact to Pcmax?

- WF:

  - Postpone the LS and further discuss this issue in next meeting