3GPP TSG-RAN WG4 Meeting #107															R4-2310290

Seoul, South Korea, 22 – 26th May 2023

Agenda item:			7.32

Source:	Apple Inc.

Title:	WF on NTN L-/S-band

Document for:	Information

# Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments that use mixed L-/S-band pairing, whereupon the UL part is one the L-band, while the DL part is one the S-band. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band where the UL is on the L-band and the DL is on the S-band.

This document presents a way forward and agreements made during the RAN#107 meeting.

# Topic #1: System parameters

Agreements for the system parameters:

-	15MHz channel bandwidths is specified for the band n254

FFS: On whether it should be optional or mandatory

# Topic #2: UE RF requirements

Agreements for the UE RF requirements:

-	FCC out-of-band emission requirements, as presented in the table below, shall be applied for band n254.

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -50 | 700 Hz | Discreet emissions averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -50 + 60/5 (f-1605) | 700 Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

# Topic #3: RRM requirements

No new agreements.

# Topic #4: SAN

Agreements from the UE RF system parameters are applied to SAN:

-	15MHz channel bandwidths are specified for the band.

- Agreements from RAN4#106bis meeting

### Topic #1: System parameters

Agreements for the system parameters:

-	5MHz, 10MHz and [15MHz] channel bandwidths are specified for the band

NOTE: The 15MHz channel bandwidth may need further technical discussions.

-	The following channel raster points are specified for the band, UL channel raster points at 322000 – 325300 and the DL channel raster points at 496700 –500000.

-	"Case A" sync pattern is specified for the band with the sync raster points at 6215 – 6244 in steps of <1>.

### Topic #2: UE RF requirements

Agreements for the UE RF requirements:

-	Existing MPR (same as for other NTN bands) is re-used for this band.

-	UE co-existence requirements:

-	Use L-band UE co-existence requirements as a baseline considering additional bands as follows:

-	NR band n54, n77, n78, n105; E-UTRA band 31, 54, 72, 73, 87, 88, 103;

-	Other bands can be considered during next meetings.

-	Reference sensitivity:

-	n53 REFSENS + "insertion loss" dB; where potential options for the insertion loss are as follows

-	Option 1: 0dB (resulting in same REFSENS as band n53);

-	Option 2: 0.5dB (resulting in same REFSENS as band n256);

-	Other options are not excluded.

### Topic #3: RRM requirements

No new agreements.

### Topic #4: SAN

Agreements from the UE RF system parameters are applied to SAN:

-	5MHz, 10MHz and [15MHz] channel bandwidths are specified for the band.

-	The following channel raster points are specified for the band, UL channel raster points at 322000 – 325300 and the DL channel raster points at 496700 –500000.

-	"Case A" sync pattern is specified for the band with the sync raster points at 6215 – 6244 in steps of <1>.