3GPP TSG-RAN WG4 Meeting # 107												R4-2310293

Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	TP for test related issue of beam correspondence in initial access

Agenda Item:	8.7.3.3

Document for:	Approval

- Introduction

The test related issue has been discussed for a long time and in the last meeting, most companies agreed that the details of the test setup can be left to RAN5. In this contribution, we try to summarize the discussion about test related issues so far.

- References

[1] 	3GPP TS 38.213

[2]	3GPP TS 38.321

[3]	R5-231834	Response LS on testability for beam correspondence in initial access, Apple

[4]	R4-2216131	Discussion on test related issue of beam correspondence, vivo

[5]	R4-2215856	On beam correspondence test issues in RRC_IDLE or RRC_INACTIVE for Rel-18 NR FR2, Huawei, HiSilicon

[6]	R4-2215702	FR2 beam correspondence requirement and test for random access, Samsung

[7]	R4-2307670, 	On testing for beam correspondence, Apple

- Text proposal

## <<Start of Change for TR 38.891>>

## 6.4	Testing impact

Unlike traditional RF requirements defined in the RRC_CONNECTED state, beam correspondence for initial access needs to be verified in RRC_IDLE, and several potential testability issues were raised during the discussion. The following aspects of test related issues were identified:

- Verification in RRC_IDLE

In general, the power test needs to be averaged over a period, which requires the UE to remain in RRC_IDLE and multiple PRACH transmission need to be triggered, so testing mode including holding RAR should be considered. The accumulative period of measurement for PRACH transmission shall be at least 1 ms.

- Maximum output power for PRACH

To guarantee stable test results, the requirement is defined and verified under maximum output power (MOP). Since the open loop power control is used for PRACH, how to ensure that the UE reaches and maintains MOP is a problem that has to be faced. During the initial access, the UE measures the RSRP and estimates the PL based on the broadcasted DL RS power at first. The required power for PRACH transmission will be calculated based on the estimated PL and the target Rx power at gNB and compares with the Pcmax if configured. The power control for PRACH is defined in [1] as:

[dBm],

where  is a pathloss calculated by the UE in dB as referenceSignalPower – higher layer filtered RSRP in dBm, and referenceSignalPower is provided by ss-PBCH-BlockPower. The parameter  is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers as specified in clause 5.1.3 of [2]:

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

UE can ramp up its power when RAR is held since it will trigger PRACH retransmission, and adjusting each individual RA response window may affect the overall test time. UE may also achieve MOP quickly by setting the preambleReceivedTargetPower properly with as few as possible PRACH re-transmission. Based on the analysis above, The PRACH can achieve MOP by holding RAR with well-defined parameters. In addition, during the initial access, UE might start transmitting preamble once it detects the SSB RSRP higher than preconfigured threshold on one RX beam, even if this RX beam is not the optimal one that UE could form, so the test function need to be considered. The details of test function and the details of the parameter configured in the test can be left to RAN5.

- Beam lock function

Currently, the beam lock function is only available in the RRC_CONNECTED state, and if the beam lock function is absent during the verification in RRC_IDLE, many complex issues will appear, e.g., when the RAR is held to trigger PRACH retransmission, UE may attempt other beams since the RAR is not received which may lead to unstable test. It also will be difficult to measure the θ (or φ) component EIRP under φ (or θ) link polarization without beam lock function. In [3], the LS from RAN5 concludes it is feasible to introduce beam lock function for RRC_IDLE, so the concern above can be solved. RAN4 concludes that dedicated beam lock function for RRC_IDLE state is needed, and the details of new beam lock function design can be left to RAN5.

- Impact of power control accuracy

TBA

## <<End of Change>>