3GPP TSG-RAN WG4 Meeting # 107	R4-2310304

Incheon Meeting, May 22nd – 27th, 2023

Title:	LS on MPR reduction

Response to:	LS R1-2302081 on LS to RAN4 for results of the LLS performance evaluation of MPR/PAR reduction solutions from TSG-RAN WG1

Release:	Rel-18

Work Item:	NR_cov_enh2

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Johannes Hejselbaek

johannes (dot) hejselbaek (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 were asked by RAN1 to assess if candidates for MPR/PAR reduction, as also studied form a link-level perspective in RAN1, could provide positive net-gains when paired with RF simulations performed by RAN4.

RAN4 have completed the performance evaluation and found that positive net-gain can be observed for agreed transparent and non-transparent schemes, at least when power-boosting is enabled.

For transparent schemes FDSS shows positive net-gains in some scenarios. For transparent schemes other than FDSS, some companies also indicated positive net-gains which are higher and close to FDSS-SE.

RAN4 has below observations:

    - Best case gain for FDSS-SE over transparent schemes:

      - 0.1-0.9 dB for inner PRBs

      - 0.4-1.3 dB for outer PRBs

    - Best case gain for Transparent schemes over baseline:

      - 0.3-1.5 dB for inner PRBs

      - 0.6-1.0 dB for outer PRBs,

    - FDSS-SE gains over transparent schemes vary with parameter settings: RB size, inner/outer, MCS index, filter type

    - FDSS-SE performs better than transparent scheme at outer allocation with certain parameter settings while transparent scheme performs better than FDSS-SE at inner allocation with certain parameter settings.

RAN4 will further consider optimized MPR performance based on achievable gain for outer and inner allocations using FDSS-SE and/or Transparent Schemes respectively.

The obtainable gains for different scenarios and schemes are further captured in the RAN4 WF R4-2310303

# 2	Actions

To RAN1

ACTION: 	RAN4 kindly requests RAN1 to take the above information into consideration for their future work and complete remaining RAN1 aspects, if any, of the schemes mentioned in Section 1.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #108	August 21 – August 25, 2023	Toulouse, France

TSG RAN WG4 Meeting #108bis	October 09 – October 13, 2023	Xiamen, China