3GPP TSG-RAN WG4 Meeting # 107												 R4-2310461

Incheon, South Korea, 22 May –26 May, 2023

Agenda item:			8.13.5

Source:	Moderator (Samsung)

Title:	Topic summary for [107][324] NR_HST_FR2_enh_Demod

Document for:	Information

# Introduction

In RAN#97-e meeting, the updated WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2) was approved with the following objective as

- Core requirement - Only train roof-mounted high-power devices with target applicable carrier frequency up to 30GHz and up to 350km/h velocity are considered in this WI - Specify the RF requirements for intra-band carrier aggregation (CA) scenario, and investigate and specify the RRM requirements for intra-band carrier aggregation (CA) scenario [RAN4] - Specify the requirement for simultaneous multi-panel operation for train roof-mounted FR2 high power devices [RAN4]: - Maximum 2 active panels supporting the multi-panel simultaneous reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception - Study on reference tunnel deployment scenario for FR2 HST and specify the channel model and corresponding core requirements if any [RAN4] - Specify UL timing adjustment solution, including explicit NW signaling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2]. - Note: RAN1/RAN2 work can be triggered by RAN4 LS. - Performance requirement - Specify the necessary RRM test cases based on the outcome on corresponding core part. - Investigate and if needed specify the demodulation performance requirements for intra-band carrier aggregation (CA) HST scenario - Specify the necessary demodulation performance requirements for simultaneous multi-panel reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception - Specify the other necessary RRM and demodulation performance requirements depending on the outcome of core part.

In practical, the scope of this email discussion is indicated as follows agenda:

- General and channel modelling (8.13.5.1)

- PDSCH requirements with CA (8.13.5.2)

- PDSCH requirements with multi-Rx Chain DL reception (8.13.5.3)

- Demodulation aspects for tunnel deployment scenario (8.13.5.4)

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Discussion the test scope and test setup for UE and BS demodulation requirement for open space and tunnel scenario deployment

- 2nd round: Discussion the test scope and test setup for UE and BS demodulation requirement for open space and tunnel scenario deployment

# Topic #1: Deployment and Channel Modelling

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2308036 | Nokia | Observation 1: Bidirectional Scenario-A and Scenario-B deployments are both possible when simultaneous two-panel reception is supported but have different beam orientation and coverage. It might be hard to conclude about the pefromance difference in those scenarios before channel model is clarified. Proposal 1: Continue the discussion of the open-space channel model both for Scenario-A and Scenario-B. Proposal 2: Consider symmetrical beam coverage areas both in Scenario-A and Scenario-B. Proposal 3: In Scenario-A, the absolute value of Ds_offset before and after RRH can be kept equal to 10m as in Rel-17. Observation 2: Bidirectional Scenario-B deployment where the next closes RRH is used as serving does not provide the best possible RRH strength. This scenario is more feasible in Rel-18 when simultaneous multi-pen reception is considered. Proposal 4: Consider bidirectional Scenario-B deployment where CPE is served by the closes RRH (i.e., not by the next closest) as one of the references for propagation conditions for PDSCH requirements. Observation 3: Neither DPS channel profile introduced for HST FR2 (Clause B.3.4) not SFN channel profiles introduced for HST FR1 (Clause B4.5/B.3.6) cannot be re-used directly for HST FR2 deployment with simultaneous two-panel reception. Proposal 5: Consider relative power trajectory and absolute delay trajectory as a part of HST FR2 channel profile for simultaneous two-panel reception in addition to the Doppler shift trajectory. Proposal 6: For Doppler shift trajectories, re-used existing DPS model from Clause B.3.4.1 for the panel moving in the direction opposite to the serving beam orientation. Additionally, introduce Doppler profile for the panel moving in same direction with the serving beam orientation. Observation 4: Relative power trajectories considered in HST FR1 SFN models cannot be re-used directly because they assume omni-directional coverage and do not take beamforming into account both on UE and RRH sides. Proposal 7: Design power trajectories for HST FR2 simultaneous reception propagation conditions taking beamforming pattern on the UE and RRH sides. |
| R4-2308037 | Nokia | Observation 1: The scenario where the UE is moving in the direction opposite to the serving beam orientation is much more challenging from the mobility point of view. Therefore, bi-directional deployment in the tunnel could be useful to address this issue. Proposal 1: RAN4 to consider both uni- and bi-directional deployments for the evaluation of FR2 HST-DPS performance requirement in the tunnel pending on the outcomes of RRM discussion. Observation 2: The contribution of multi-path components (MPCs) to the received signal might be significant (e.g., up to 2nd order). The delay spread could be small and the excess delay between different order of MPCs might not be separable. Observation 3: Per ray tracing simulation result, AoD/AoA concentrated in few angular directions, and maximum AoD/AoA are bigger compared to the urban channel. Observation 4: The effect of the multi-path channel is expected to be more significant when the train is nearby the source RRH Proposal 2: RAN 4 to discuss further muti-path fading model (e.g., 2-tap multi-path components). Observation 5: NLoS channel condition under the RRH may exist, and bi-directional deployment may see stronger impact on the performance because such NLoS paths may bring higher interference power. Proposal 3: RAN4 to consider the NLoS channel condition under the RRH for performance evaluation in tunnel bi-directional deployment. |
| R4-2308432 | Ericsson | Proposal 1: For simultaneous multi-Rx reception requirements in FR2 HST, RAN4 extend the existing FR2 HST-DPS uni-directional model to evaluate the UE demodulation performance. Proposal 2: Channel model(s) used for the simultaneous multi-Rx reception should ensure UE always receives data signals from two TRPs regardless of the location to avoid unnecessary switching between single-Rx reception and multi-Rx reception during the test. Proposal 3: RAN4 focus on the Bi-directional deployment with Scenario B with smaller Ds_offset, e.g., Ds_offset=10m. |
| R4-2308433 | Ericsson | Proposal 1: RAN4 should consider the LOS-based single-tap model(s) to study UE/BS demodulation performance for tunnel deployment scenario. Proposal 2: RAN4 reuse the uni-directional HST-DPS channel model to study the tunnel deployment scenario with parameters: • Ds=700m • Dmin=2m • Ds_offset=10m Proposal 3: For the tunnel deployment scenario, RAN4 should assume PC6 UE receives signals with single Rx panel. |
| R4-2308831 | Samsung | Proposal 1: The following value of Ds_offset can be considered to determine the beam serving coverage for each RRH in Bi-directional scenario, for scenario A and B, separately Scenario; Ds (meter); Dmin (meter); Ds_offset (meter). A; 700; 10; 10. B; 700; 150; 100. The related beam serving coverage for left panel and right panel for each RRH can be considered as - Left panel: [(k-1) *Ds- Ds_offset, k*Ds- Ds_offset] for the kth RRH, k=0, 1, … - Right panel: [k *Ds+ Ds_offset, (k+1) *Ds+ Ds_offset] for the kth RRH, k=0,1, … Proposal 2: No need to model the relative propagation delay and power for tap from the visible RRH Proposal 3: The following channel model can be considered for PDSCH requirement with multi-Rx reception in Bi-directional deployment scenario. - Doppler shift fst=fd cosθt (Hz) from the left panel of RRH for PDSCH received at right panel of UE is given by cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v - Doppler shift fst=fd cosθt (Hz) from the right panel of RRH for PDSCH received at left panel of UE is given by cosθt=-(Ds-Dsoffset +vt)Dmin2+Ds-Dsoffset +vt2, 0<t≤2*Ds_offsetv cosθt=-(Dsoffset +vt)Dmin2+Dsoffset +vt2, 2*Ds_offsetv<t≤Dsv cosθt=cosθt modDsv, t>Ds/v. |
| R4-2308833 | Samsung | Proposal 1: RAN4 considers both Uni-directional and Bi-directional deployment for demodulation requirement for FR2 HST-DPS in tunnel scenario. For Uni-directional deployment, FR2 HST UE moves in the same direction as the RRH serving beam direction Proposal 2: Deprioritized the demodulation requirement for two-panel simultaneous reception in Bi-directional scenario in the tunnel scenario. Proposal 3: No new channel model (multi-tap) is needed pending on UE location with faraway or closed Proposal 4: Not consider multipath fading channel for demodulation performance Propose 5: Reuse the channel model in RAN4 spec 38.101-4, i.e, single path with LoS propagation, for performance requirement study of FR2 HST-DPS in tunnel deployment with updated Ds, Dmin and Ds_offset. Uni-directional scenario cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v Parameter Value Parameter; Value. TBD. Ds; 700 m. Ds_offset; [5m]. Dmin; 1 m. v; 350 km/h. fd; 9722 Hz. Bi-directional scenario cosθt=Ds-vtDmin2+Ds-vt2, 0<t≤(0.5*Ds)/v cosθt=-vtDmin2+vt2, (0.5*Ds)/v<t≤Ds/v cosθt modDsv, t>Ds/v Parameter; Value. TBD. 700 m. 1 m. 350 km/h. 9722 Hz. Proposal 6: The value of Ds_offset as [5]m can be considered for channel model of Uni-directional scenario in Tunnel scenario. Proposal 7: Reuse Rel-17 FR2 38.104 Bi-directional scenario B channel model with updated the value of Dmin as 1m to align the deployment scenario for demodulation requirement in tunnel scenario. |
| R4-2308889 | Huawei | Proposal 1: Consider both scenario A and scenario B for PDSCH demodulation requirements. If one set of requirements are defined, select scenario B with high priority for Bi-directional deployment scenario for PDSCH demodulation requirements. Proposal 2: Consider above beam serving coverage for FR2 HST simultaneous Rx reception. Keep the previous agreements that the Ds_offset value is 10m and 100m for Scenario-A and Scenario-B respectively considering the worst case. Proposal 3: Consider above channel model for FR2 HST simultaneous Rx reception. Proposal 4: Single-panel reception assumption should be prioritized for tunnel scenario deployment. Proposal 5: Use Uni-directional deployment for tunnel scenario for HST FR2. |
| R4-2309663 | Qualcomm | Observation 1: HST-SFN Channel Model is not applicable to FR2 HST with Simultaneous reception; Proposal 1: RAN4 to apply the existing FR2 HST DPS Channel model to Simultaneous reception, with updated beam coverage areas as necessary; Observation 2: RAN4 agreed previously not to model propagation delay in existing FR2 HST UE channel models for legacy Demod requirement; Proposal 2: If independent FFT windows for each panel are assumed, RAN4 should not consider modelling propagation delay from RRHs to the train tracks as part of the channel model; Proposal 3: If a single FFT window is assumed, RAN4 should further consider modelling the propagation delay from RRHs to the train tracks as part of the channel model; Proposal 4: RAN4 to assume independent FFT Windows for each panel, and to not consider propagation delay in the channel model; Observation 3: RAN4 specified FR2 HST Requirements for bidirectional deployment with Scenario B only; Proposal 5: For Bidirectional requirements with simultaneous multiRX reception, RAN4 should reuse the approach followed for existing requirements, and specify test cases for Bidirectional deployments with Scenario B only; |
| R4-2309665 | Qualcomm | Observation 1: Propagation in the case of Tunnel Deployment for FR2 HST follows mainly a single path; Proposal 1: RAN4 should not consider multipath channel to model propagation in the case of Tunnel Deployment for FR2 HST; Proposal 2: To study the scope of FR2 HST Tunnel Deployments, RAN4 should consider the existing FR2 HST DPS channel model as baseline, Dmin = 1m to model reduced distance between Track and RRH; Observation 2: A smaller Dmin results in a smaller ‘jump’ in Doppler shift when switching across RRH during the train path along the tracks; |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

List of open issues

- Sub-topic 1-1 Deployment and Channel Model for Demodulation requirement with simultaneous Rx reception

  - Issue 1-1-1: Deployment Scenario for FR2 HST simultaneous multi-Rx reception requirements for Open Space Scenario

  - Issue 1-1-2: Beam serving coverage for multi-Rx reception in Bi-directional scenario

  - Issue 1-1-3: Channel Model for FR2 HST simultaneous multi-panel reception performance evaluation

- Sub-topic 1-2 Deployment and Channel Model for Demodulation requirement with simultaneous Rx reception

  - Issue 1-2-1: Deployment for Demodulation requirements in Tunnel Scenario

  - Issue 1-2-2: RRH parameters for Channel Model in tunnel scenario

  - Issue 1-2-3: Channel Model for Demodulation requirements in Tunnel Scenario for UE demod

  - Issue 1-2-4: Channel Model for Demodulation requirements in Tunnel Scenario for BS demod

### Sub-topic 1-1 Deployment and Channel Model for Demodulation requirement with simultaneous Rx reception in open space scenario

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1-1: Deployment Scenario for FR2 HST simultaneous multi-Rx reception requirements for Open Space Scenario

- Observations

  - Observation 1 (Nokia):

    - Bidirectional Scenario-A and Scenario-B deployments are both possible when simultaneous two-panel reception is supported but have different beam orientation and coverage. It might be hard to conclude about the performance difference in those scenarios before channel model is clarified.

Figure 1: HST FR2 bidirectional deployments in Scenario-B (top) and in Scenario-A (bottom).

  - Observation 2 (QC):

    - RAN4 specified FR2 HST Requirements for bidirectional deployment with Scenario B only;

- Proposals

  - Option 1 (Ericsson): RAN4 focus on the Bi-directional deployment with Scenario B with smaller Ds_offset, e.g., Ds_offset=10m.

  - Option 2 (Huawei): Consider both scenario A and scenario B for PDSCH demodulation requirements. If one set of requirements are defined, select scenario B with high priority for Bi-directional deployment scenario for PDSCH demodulation requirements.

  - Option 3(QC): For Bidirectional requirements with simultaneous multiRX reception, RAN4 should reuse the approach followed for existing requirements, and specify test cases for Bidirectional deployments with Scenario B only;

- Recommended WF

  - TBA

Issue 1-1-2: Beam serving coverage for multi-Rx reception in Bi-directional scenario

- Observations

  - Observation 1 (Nokia):

    - Bidirectional Scenario-B deployment where the next closes RRH is used as serving does not provide the best possible RRH strength. This scenario is more feasible in Rel-18 when simultaneous multi-pen reception is considered.

- Proposals

  - Option 1 (Huawei):

    - Consider above beam serving coverage for FR2 HST simultaneous Rx reception. Keep the previous agreements that the Ds_offset value is 10m and 100m for Scenario-A and Scenario-B respectively considering the worst case.

  - Option 2 (Samsung):

    - The following value of Ds_offset can be considered to determine the beam serving coverage for each RRH in Bi-directional scenario, for scenario A and B, separately

| Scenario | Ds (meter) | Dmin (meter) | Ds_offset (meter) |
| --- | --- | --- | --- |
| A | 700 | 10 | 10 |
| B | 700 | 150 | 100 |

    - The related beam serving coverage for left panel and right panel for each RRH can be considered as

      - Left panel: [(k-1) *Ds- Ds_offset, k*Ds- Ds_offset] for the kth RRH, k=0, 1, …

      - Right panel: [k *Ds+ Ds_offset, (k+1) *Ds+ Ds_offset] for the kth RRH, k=0,1, …

  - Option 3 (Nokia):

    - Consider symmetrical beam coverage areas both in Scenario-A and Scenario-B

    - In Scenario-A, the absolute value of Ds_offset before and after RRH can be kept equal to 10m as in Rel-17.

    - Consider bidirectional Scenario-B deployment where CPE is served by the closes RRH (i.e., not by the next closest) as one of the references for propagation conditions for PDSCH requirements.

- Figure 4: Bidirectional scenario-B where the closest RRH is used as serving.

- Recommended WF

  - TBA

Issue 1-1-3: Channel model for PDSCH requirements with Multi-Rx Reception

- Observations

  - Observation 1 (Nokia):

    - Neither DPS channel profile introduced for HST FR2 (Clause B.3.4) not SFN channel profiles introduced for HST FR1 (Clause B4.5/B.3.6) cannot be re-used directly for HST FR2 deployment with simultaneous two-panel reception.

    - Relative power trajectories considered in HST FR1 SFN models cannot be re-used directly because they assume omni-directional coverage and do not take beamforming into account both on UE and RRH sides.

  - Observation 2 (QC):

    - RAN4 agreed previously not to model propagation delay in existing FR2 HST UE channel models for legacy Demod requirement;

- Proposals

  - Option 1 (Huawei): Consider above channel model for FR2 HST simultaneous Rx reception.

    - Visible rang

      - (k - 1)*Ds-Ds_offset≤a<k*Ds- Ds_offset (For the left panel)

      - k*Ds+ Ds_offset≤a<(k + 1)*Ds+Ds_offset (For the right panel)

    - Doppler shift

fst=fd cosθt

      - For the left panel:

cosθt=Ds-vtDmin2+Ds-vt2,  -Ds_offsetv<t≤Ds-Ds_offsetv

cosθt=cosθ(t+Ds_offsetv) modDsv-Ds_offsetv,  t>Ds-Ds_offsetv

      - For the right panel:

cosθt=-Ds+vtDmin2+Ds+vt2,  -Ds-Dsoffsetv<t≤Dsoffsetv

cosθt=cosθ(t-Ds_offsetv) modDsv-Ds-Ds_offsetv,  t>Ds_offsetv

    - Delay

      - For the left panel:

Tt=Dmin2+Ds-vt2C,  -Ds_offsetv<t≤Ds-Ds_offsetv

Tt=T(t+Ds_offsetv) modDsv-Ds_offsetv,  t>Ds-Ds_offsetv

      - For the right panel:

Tt=Dmin2+Ds+vt2C,  -Ds-Ds_offsetv<t≤Ds_offsetv

Tt=T(t-Ds_offsetv) modDsv-Ds-Ds_offsetv,  t>Ds_offsetv

    - Power

      - Power level Pk (dB) for the signal from kth RRH, normalized to the total power received from all visible RRHs.

  - Option 2 (Samsung):

    - No need to model the relative propagation delay and power for tap from the visible RRH

    - The following channel model can be considered for PDSCH requirement with multi-Rx reception in Bi-directional deployment scenario.

- Doppler shift fst=fd cosθt (Hz) from the left panel of RRH for PDSCH received at right panel of UE is given by cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθt=cosθt modDsv, t>Ds/v - Doppler shift fst=fd cosθt (Hz) from the right panel of RRH for PDSCH received at left panel of UE is given by cosθt=-(Ds-Dsoffset +vt)Dmin2+Ds-Dsoffset +vt2, 0<t≤2*Ds_offsetv cosθt=-(Dsoffset +vt)Dmin2+Dsoffset +vt2, 2*Ds_offsetv<t≤Dsv cosθt=cosθt modDsv, t>Ds/v

  - Option 3 (Ericsson):

    - For simultaneous multi-Rx reception requirements in FR2 HST, RAN4 extend the existing FR2 HST-DPS uni-directional model to evaluate the UE demodulation performance.

    - Channel model(s) used for the simultaneous multi-Rx reception should ensure UE always receives data signals from two TRPs regardless of the location to avoid unnecessary switching between single-Rx reception and multi-Rx reception during the test.

      - Rx Panel 1 (Same as TS38.101-4 B.3.4.1):

fst=fdcosθ(t)

cosθt=Ds-vtDmin2+Ds-vt2,  0<t≤Ds-Ds_offsetv2Ds-vtDmin2+2Ds-vt2,  Ds-Ds_offsetv<t≤Dsv

cosθ(t)=cost modDsv,  t>Dsv

      - Rx Panel 2:

fst=-fdcosθ(t)

cosθt=Ds+vtDmin2+Ds+vt2,  0<t≤Ds_offsetvvtDmin2+vt2,  Ds_offsetv<t≤Dsv

cosθ(t)=cost modDsv,  t>Dsv

  - Option4 (Nokia):

    - Consider relative power trajectory and absolute delay trajectory as a part of HST FR2 channel profile for simultaneous two-panel reception in addition to the Doppler shift trajectory.

    - For Doppler shift trajectories, re-used existing DPS model from Clause B.3.4.1 for the panel moving in the direction opposite to the serving beam orientation. Additionally, introduce Doppler profile for the panel moving in same direction with the serving beam orientation.

    - Design power trajectories for HST FR2 simultaneous reception propagation conditions taking beamforming pattern on the UE and RRH sides.

  - Option5 (QC):

    - If independent FFT windows for each panel are assumed, RAN4 should not consider modelling propagation delay from RRHs to the train tracks as part of the channel model

    - If a single FFT window is assumed, RAN4 should further consider modelling the propagation delay from RRHs to the train tracks as part of the channel model;

    - RAN4 to assume independent FFT Windows for each panel, and to not consider propagation delay in the channel model;

- Recommended WF

  - TBA

### Sub-topic 1-2 Deployment and Channel Model for Demodulation requirement with simultaneous Rx reception in tunnel scenario

Issue 1-2-1: Deployment for Demodulation requirements in Tunnel Scenario

- Observations

  - Observation 1 (Nokia):

    - The scenario where the UE is moving in the direction opposite to the serving beam orientation is much more challenging from the mobility point of view. Therefore, bi-directional deployment in the tunnel could be useful to address this issue.

- Proposals

  - Option 1 (Samsung):

    - RAN4 considers both Uni-directional and Bi-directional deployment for demodulation requirement for FR2 HST-DPS in tunnel scenario. For Uni-directional deployment, FR2 HST UE moves in the same direction as the RRH serving beam direction.

    - Deprioritized the demodulation requirement for two-panel simultaneous reception in Bi-directional scenario in the tunnel scenario.

  - Option 2 (Huawei):

    - Single-panel reception assumption should be prioritized for tunnel scenario deployment.

    - Use Uni-directional deployment for tunnel scenario for HST FR2.

  - Option 3 (Ericsson):

    - For the tunnel deployment scenario, RAN4 should assume PC6 UE receives signals with single Rx panel.

  - Option 4 (Nokia):

    - RAN4 to consider both uni- and bi-directional deployments for the evaluation of FR2 HST-DPS performance requirement in the tunnel pending on the outcomes of RRM discussion

- Recommended WF

  - TBA

Issue 1-2-2: RRH parameters for Channel Model in tunnel scenario

- Proposals

  - Option 1 (Samsung):

    - The value of Ds_offset as [5]m can be considered for channel model of Uni-directional scenario in Tunnel scenario.

      - Ds=700m

      - Dmin=1m

      - Ds_offset=5m

  - Option 2 (Ericsson):

    - RAN4 reuse the uni-directional HST-DPS channel model to study the tunnel deployment scenario with parameters:

      - Ds=700m

      - Dmin=2m

      - Ds_offset=10m

- Recommended WF

  - TBA

Issue 1-2-3: Channel Model for Demodulation requirements in Tunnel Scenario for UE demod

- Observations

  - Observation 1 (Nokia):

    - The contribution of multi-path components (MPCs) to the received signal might be significant (e.g., up to 2nd order). The delay spread could be small and the excess delay between different order of MPCs might not be separable.

    - Per ray tracing simulation result, AoD/AoA concentrated in few angular directions, and maximum AoD/AoA are bigger compared to the urban channel.

    - The effect of the multi-path channel is expected to be more significant when the train is nearby the source RRH

    - NLoS channel condition under the RRH may exist, and bi-directional deployment may see stronger impact on the performance because such NLoS paths may bring higher interference power.

  - Observation 2 (QC):

    - Propagation in the case of Tunnel Deployment for FR2 HST follows mainly a single path;

    - A smaller Dmin results in a smaller ‘jump’ in Doppler shift when switching across RRH during the train path along the tracks;

- Proposals

  - Option 1 (Samsung):

    - No new channel model (multi-tap) is needed pending on UE location with faraway or closed

    - Reuse the channel model in RAN4 spec 38.101-4, i.e, single path with LoS propagation, for performance requirement study of FR2 HST-DPS in tunnel deployment with updated Ds, Dmin and Ds_offset.

    - Uni-directional scenario

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv

cosθt=cosθt modDsv,  t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
| Ds | 700 m |
| Ds_offset | [5m] |
| Dmin | 1 m |
| v | 350 km/h |
| fd | 9722 Hz |

    - Uni-directional scenario

cosθt=Ds-vtDmin2+Ds-vt2,  0<t≤(0.5*Ds)/v

cosθt=-vtDmin2+vt2,  (0.5*Ds)/v<t≤Ds/v 
cosθt modDsv,    t>Ds/v

| Parameter | Value |
| --- | --- |
|  | TBD |
|  | 700 m |
|  | 1 m |
|  | 350 km/h |
|  | 9722 Hz |

  - Option 2 (Ericsson):

    - RAN4 should consider the LOS-based single-tap model(s) to study UE/BS demodulation performance for tunnel deployment scenario.

    - RAN4 reuse the uni-directional HST-DPS channel model to study the tunnel deployment scenario with parameters:

      - Ds=700m

      - Dmin=2m

      - Ds_offset=10m

  - Option 3 (Nokia):

    - RAN 4 to discuss further muti-path fading model (e.g., 2-tap multi-path components).

    - RAN4 to consider the NLoS channel condition under the RRH for performance evaluation in tunnel bi-directional deployment.

  - Option 4 (QC):

    - RAN4 should not consider multipath channel to model propagation in the case of Tunnel Deployment for FR2 HST;

    - To study the scope of FR2 HST Tunnel Deployments, RAN4 should consider the existing FR2 HST DPS channel model as baseline, Dmin = 1m to model reduced distance between Track and RRH;

- Recommended WF

  - TBA

Issue 1-2-4: Channel Model for Demodulation requirements in Tunnel Scenario for BS demod

- Observations

  - Observation 1 (Nokia):

    - The contribution of multi-path components (MPCs) to the received signal might be significant (e.g., up to 2nd order). The delay spread could be small and the excess delay between different order of MPCs might not be separable.

    - Per ray tracing simulation result, AoD/AoA concentrated in few angular directions, and maximum AoD/AoA are bigger compared to the urban channel.

    - The effect of the multi-path channel is expected to be more significant when the train is nearby the source RRH

    - NLoS channel condition under the RRH may exist, and bi-directional deployment may see stronger impact on the performance because such NLoS paths may bring higher interference power.

- Proposals

  - Option 1 (Samsung):

    - Reuse Rel-17 FR2 38.104 Bi-directional scenario B channel model with updated the value of Dmin as 1m to align the deployment scenario for demodulation requirement in tunnel scenario.

  - Option 2 (Ericsson):

    - RAN4 should consider the LOS-based single-tap model(s) to study UE/BS demodulation performance for tunnel deployment scenario.

    - RAN4 reuse the uni-directional HST-DPS channel model to study the tunnel deployment scenario with parameters:

      - Ds=700m

      - Dmin=2m

      - Ds_offset=10m

  - Option 3 (Nokia):

    - RAN 4 to discuss further muti-path fading model (e.g., 2-tap multi-path components).

    - RAN4 to consider the NLoS channel condition under the RRH for performance evaluation in tunnel bi-directional deployment.

- Recommended WF

  - TBA

# Topic #2: PDSCH requirement with CA

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2308431 | Ericsson | Proposal 1: Rel-18 FR2 CA PDSCH demodulation requirements for HST are only applicable for intra-band contiguous CA and intra-band non-contiguous CA combinations. Proposal 2: Apply the existing applicability and test rules for FR2 CA PDSCH demodulation (TS38.101-4 clause 7.1.1.5) to Rel-18 CA PDSCH demodulation requirements for HST. Proposal 3: RAN4 should consider additional margin of 0.7dB for Rel-18 CA PDSCH demodulation requirements for HST as far as the span of alignment results is less than 2.5dB. |
| R4-2308830 | Samsung | Proposal 1: Consider TDD 120KHz SCS + TDD 120KHz SCS for carrier combination Proposal 2: it is proposed that Rel-18 FR2 HST PDSCH CA requirements are release independent from Rel-17. Proposal 3: Not consider additional margin for CA requirement based on single carrier. The performance requirements specified in the section of CA requirement do not apply for UE single carrier test. |
| R4-2308834 | Samsung | Simulation results summary |
| R4-2308891 | Huawei | Proposal 1: Only consider TDD 120 kHz + TDD 120 kHz case for FR2 HST CA scenario. Proposal 2: Reuse SNR for 200MHz case from Rel-17 FR2 HST single carrier case, and run simulation for other bandwidths, i.e. 50MHz, 100MHz and 200MHz. Proposal 3: Do not consider additional margin for CA requirement based on single carrier simulation with considering imperfection model. |
| R4-2308892 | Huawei | Simulation results for CA |
| R4-2309330 | Nokia | Observation 1: There are already existing minimum requirements for 200 MHZ CBW for HST FR2 as tabulated in 38.101-4 Table 7.2.2.2.4-3: Minimum performance for HST-DPS. Observation 2: There is still an open issue from RAN4#106bis-e related to the possibility of adding noise margin and/or imperfection model to be discussed. Proposal 1: In case there is no additional noise margin or imperfection model being introduced to the single carrier link level simulation, the minimum requirements for 200 MHz CBW can be reused. Observation 3: For UE demodulation, only TDD 120 KHz + TDD 120 KHz is considered in 38.101-4. Observation 4: For testing in UE demodulation, largest aggregated CA bandwidth combination should be the priority. Proposal 2: For HST FR2 with CA, TDD 120 KHz + TDD 120 KHz shall be considered, with the largest aggregated CA bandwidth combination shall be the highest priority. Observation 5: The noise power level in CA case is coming from the noise level of single carrier with some additional noise from ΔRIB, which is the allowed reference sensitivity relaxation due to support for inter-band CA operation. Proposal 3: In defining PDSCH performance requirements for HST FR2, when using single carrier link level simulation, RAN4 to introduce additional noise to the single carrier noise in a similar fashion as defined in section 4.5.3.2 of 38.101-4. Proposal 4: If neither additional noise nor imperfection model is introduced in the single carrier link level simulation, RAN4 to define HST FR2 requirements for single carrier for the whole considered CBW in FR2. The requirements for HST FR2 with CA can then simply refer to the requirements for single carrier. Proposal 5: RAN4 to consider imperfection model to be introduced in single carrier link level simulation if there is a proper model proposed by any company. Observation 6: The reference values for test cases 1-4 and 2-4 as documented in R4-2305893 are taken from 38.101-4 for single carrier Rel-17 HST FR2. Observation 7: The final values as specified in 38.101-4 for single carrier Rel-17 HST FR2 were defined after some iterations of simulation alignment process and the introduction of impairments. Proposal 6: In the simulation alignment for Rel-18 HST FR2 with CA, RAN4 shall consider the reference values as in R4-2208074 rather than the already specified values in 38.101-4. |
| R4-2309664 | Qualcomm | Proposal 1: For FR2 HST CA UE Demod, RAN4 should not reuse single CC PDSCH requirements for CA; Proposal 2: For FR2 HST CA Demod requirements, RAN4 should define requirements for the agreed BW carrier configuration based on the collected impairment results; Proposal 3: For FR2 HST CA Demod tests, the same RRH is assumed to transmit all CCs, Channel and Doppler trajectories should be the same for all CCs; |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

List of open issues

- Sub-topic 2-1 Intra-band CA requirement

  - Issue 2-1-1: Carrier combination for requirements

  - Issue 2-1-2: Whether existing SNR requirement or 200MHz CBW defined in single carrier can be reused for CA case with 200MHz

  - Issue 2-1-3: RRH and Channel model assumption for CA

  - Issue 2-1-4:  imperfection model

  - Issue 2-1-5: Additional margin for CA requirement

  - Issue 2-1-6: Simulation results alignment and requirement derivation

  - Issue 2-1-7: Test applicability rule

  - Issue 2-1-8: Release independent

### Sub-topic 2-1: Intra-band CA requirement

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1-1:  Carrier combination for requirements

- Proposals

  - Option 1 (Samsung, Huawei, Nokia): only TDD 120 KHz SCS + TDD 120 KHz SCS

  - Option 2 (Ericsson): Rel-18 FR2 CA PDSCH demodulation requirements for HST are only applicable for intra-band contiguous CA and intra-band non-contiguous CA combinations.

- Recommended WF

  - TBA

Issue 2-1-2:  Whether existing SNR requirement or 200MHz CBW defined in single carrier can be reused for CA case with 200MHz

- Observations

  - Observation 1 (Nokia):

    - There are already existing minimum requirements for 200 MHZ CBW for HST FR2 as tabulated in 38.101-4 Table 7.2.2.2.4-3: Minimum performance for HST-DPS.

    - There is still an open issue from RAN4#106bis-e related to the possibility of adding noise margin and/or imperfection model to be discussed.

- Proposals

  - Option 1 (Huawei): Reuse SNR for 200MHz case from Rel-17 FR2 HST single carrier case, and run simulation for other bandwidths, i.e. 50MHz, 100MHz and 400MHz.

  - Option 2 (QC):

    - For FR2 HST CA UE Demod, RAN4 should not reuse single CC PDSCH requirements for CA;

    - For FR2 HST CA Demod requirements, RAN4 should define requirements for the agreed BW carrier configuration based on the collected impairment results;

  - Option 3 (Nokia):

    - In case there is no additional noise margin or imperfection model being introduced to the single carrier link level simulation, the minimum requirements for 200 MHz CBW can be reused.

- Recommended WF

  - TBA

Issue 2-1-3:  RRH and Channel model assumption for CA

- Proposals

  - Option 1 (QC): For FR2 HST CA Demod tests, the same RRH is assumed to transmit all CCs, Channel and Doppler trajectories should be the same for all CCs;

- Recommended WF

  - TBA

Issue 2-1-4:  imperfection model

- Proposals

  - Option 1 (Nokia): RAN4 to consider imperfection model to be introduced in single carrier link level simulation if there is a proper model proposed by any company.

- Recommended WF

  - TBA

Issue 2-1-5:  Additional noise margin for CA requirement based on single carrier

- Observations

  - Observation 1 (Nokia):

    - The noise power level in CA case is coming from the noise level of single carrier with some additional noise from ΔRIB, which is the allowed reference sensitivity relaxation due to support for inter-band CA operation.

- Proposals

  - Option 1 (Samsung): Not consider additional margin for CA requirement based on single carrier. The performance requirements specified in the section of CA requirement do not apply for UE single carrier test.

  - Option 2 (Huawei): Do not consider additional margin for CA requirement based on single carrier simulation with considering imperfection model

  - Option 3 (Nokia): In defining PDSCH performance requirements for HST FR2, when using single carrier link level simulation, RAN4 to introduce additional noise to the single carrier noise in a similar fashion as defined in section 4.5.3.2 of 38.101-4.

  - Ooption4 (Nokia): If neither additional noise nor imperfection model is introduced in the single carrier link level simulation, RAN4 to define HST FR2 requirements for single carrier for the whole considered CBW in FR2. The requirements for HST FR2 with CA can then simply refer to the requirements for single carrier.

- Recommended WF

  - TBA

Issue 2-1-6:  Simulation results alignment and requirement derivation

- Observations

  - Observation 1 (Nokia):

    - The reference values for test cases 1-4 and 2-4 as documented in R4-2305893 are taken from 38.101-4 for single carrier Rel-17 HST FR2.

    - The final values as specified in 38.101-4 for single carrier Rel-17 HST FR2 were defined after some iterations of simulation alignment process and the introduction of impairments.

- Proposals

  - Option 1 (Nokia):  In the simulation alignment for Rel-18 HST FR2 with CA, RAN4 shall consider the reference values as in R4-2208074 rather than the already specified values in 38.101-4.

  - Option 2 (Ericsson): RAN4 should consider additional margin of 0.7dB for Rel-18 CA PDSCH demodulation requirements for HST as far as the span of alignment results is less than 2.5dB.

- Recommended WF

  - TBA

Issue 2-1-7: Test applicability rule

- Observations

  - Observation 1 (Nokia):

    - For HST FR2 with CA, TDD 120 KHz + TDD 120 KHz shall be considered, with the largest aggregated CA bandwidth combination shall be the highest priority.

    - For testing in UE demodulation, largest aggregated CA bandwidth combination should be the priority.

- Proposals

  - Option 1 (Ericsson): Apply the existing applicability and test rules for FR2 CA PDSCH demodulation (TS38.101-4 clause 7.1.1.5) to Rel-18 CA PDSCH demodulation requirements for HST.

  - Option 2(Nokia): For HST FR2 with CA, TDD 120 KHz + TDD 120 KHz shall be considered, with the largest aggregated CA bandwidth combination shall be the highest priority.

- Recommended WF

  - TBA

Issue 2-1-8: Release independent

- Proposals

  - Option 1 (Samsung): it is proposed that Rel-18 FR2 HST PDSCH CA requirements are release independent from Rel-17.

- Recommended WF

  - TBA

# Topic #3: PDSCH requirement with multi-Rx reception

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2308432 | Ericsson | Proposal 4: RAN4 only evaluate the mDCI-based transmission scheme. Proposal 5: RAN4 will evaluate both the rank combination options: 1+1 and 2+2. Proposal 6: Simultaneous multi-Rx reception requirements are appliable for UE with the following capabilities at least: • UE power class 6 (PC6) • simultaneousReceptionDiffTypeD-r16 • maxNumberActiveTCI-PerBWP > 1 |
| R4-2308831 | Samsung | Proposal 4: Independent FFT window for each panel for the scenario the reception time difference between different TRPs is larger than one CP. Proposal 5: Consider 2+2-layer combination with full-overlapping scheduling based on mDCI transmission scheme for requirement definition of multi-Rx reception |
| R4-2308890 | Huawei | Observation 1: For rank 1+1 case and 2+2 case respectively, there is about 0.5dB and 3.5dB performance difference between different UE processing assumption for the FFT window. Observation 2: For both UE processing assumption for the FFT window, the maximum throughput can be achieved. Proposal 1: Do not specify baseline UE processing assumption for the FFT window and leave it to UE implementation for FR2 HST performance requirements definition. In case large span is observed, additional margin should be added. Proposal 2: Do not consider single-DCI based multi-TRP scheduling. |
| R4-2309331 | Nokia | Observation 1: CPE is having more capabilities than a normal UE and is not power constrained. Proposal 1: RAN4 shall consider that the CPE will have independent FFT per panel. Observation 2: In HST FR2, the train is moving, which will cause power imbalance between the received signals from the TRPs. Observation 3: mDCI allows to have different MCSs transmitted from the two TRPs. Observation 4: sDCI is bounded by the lowest MCS and requires more stringent time difference requirements than mDCI. Observation 5: It is assumed that there is no inter-TRP interference and the time difference between TRPs can be more than CP. Proposal 2: RAN4 to consider mDCI in defining the PDSCH requirements for HST FR2 with multi-Rx chain DL reception. Proposal 3: sDCI can be considered if it can be shown that it has advantages than mDCI in HST FR2 with multi-Rx chain DL reception, and interested companies are encouraged to provide their studies. |
| R4-2309663 | Qualcomm | Proposal 6: RAN4 to introduce FR2 HST Demodulation tests for simultaneous reception with sDCI, and related applicability rule to skip sDCI testing if the device supports mDCI; |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

List of open issues

- Sub-topic 3-1 General for PDSCH requirement with multi-Rx reception

  - Issue 3-1-1: UE processing assumption for the FFT window

- Sub-topic 3-2: Test setup for PDSCH requirement with multi-Rx reception

  - Issue 3-2-1: Applicability rule for PDSCH requirement with simultaneous multi-Rx reception

  - Issue 3-2-2: Transmission schemes

  - Issue 3-2-3: Layer combination

  - Issue 3-2-4: Test applicability rule of PDSCH with mDCI and sDCI if introduced both

### Sub-topic 3-1: General for PDSCH requirement with multi-Rx reception

Sub-topic description:

Open issues and candidate options before meeting:

Issue 3-1-1: UE processing assumption for the FFT window

- Observations

  - Observation 1 (Nokia): CPE is having more capabilities than a normal UE and is not power constrained.

  - Observation 2 (Huawei):

    - For rank 1+1 case and 2+2 case respectively, there is about 0.5dB and 3.5dB performance difference between different UE processing assumption for the FFT window.

    - For both UE processing assumption for the FFT window, the maximum throughput can be achieved.

- Proposals

  - Option 1 (Nokia): RAN4 shall consider that the CPE will have independent FFT per panel

  - Option 2 (Huawei):

    - Do not specify baseline UE processing assumption for the FFT window and leave it to UE implementation for FR2 HST performance requirements definition. In case large span is observed, additional margin should be added.

  - Option 3(Samsung):

    - Independent FFT window for each panel for the scenario the reception time difference between different TRPs is larger than one CP.

- Recommended WF

  - TBA

### Sub-topic 3-2: Test setup for PDSCH requirement with multi-Rx reception

Sub-topic description:

Open issues and candidate options before meeting:

Issue 3-2-1: Applicability rule for PDSCH requirement with simultaneous multi-Rx reception

- Proposals

  - Option 1 (Ericsson):

    - Simultaneous multi-Rx reception requirements are appliable for UE with the following capabilities at least:

      - UE power class 6 (PC6)

      - simultaneousReceptionDiffTypeD-r16

      - maxNumberActiveTCI-PerBWP > 1

- Recommended WF

  - TBA

Issue 3-2-2: Transmission schemes

- Observations

  - Observation 1 (Nokia):

    - In HST FR2, the train is moving, which will cause power imbalance between the received signals from the TRPs.

    - mDCI allows to have different MCSs transmitted from the two TRPs

    - sDCI is bounded by the lowest MCS and requires more stringent time difference requirements than mDCI.

    - It is assumed that there is no inter-TRP interference and the time difference between TRPs can be more than CP.

- Proposals

  - Option 1 (Nokia):

    - RAN4 to consider mDCI in defining the PDSCH requirements for HST FR2 with multi-Rx chain DL reception.

    - sDCI can be considered if it can be shown that it has advantages than mDCI in HST FR2 with multi-Rx chain DL reception, and interested companies are encouraged to provide their studies.

  - Option 2 (Huawei, Samsung, Ericsson):

    - Do not consider single-DCI based multi-TRP scheduling.

- Recommended WF

  - TBA

Issue 3-2-3: Layer combination

- Proposals

  - Option 1 (Ericsson):

    - RAN4 will evaluate both the rank combination options: 1+1 and 2+2.

  - Option 2 (Samsung):

    - Consider 2+2-layer combination with full-overlapping scheduling based on mDCI transmission scheme for requirement definition of multi-Rx reception.

- Recommended WF

  - TBA

Issue 3-2-4: Test applicability rule of PDSCH with mDCI and sDCI if introduced both

- Proposals

  - Option 1 (QC): RAN4 to introduce FR2 HST Demodulation tests for simultaneous reception with sDCI, and related applicability rule to skip sDCI testing if the device supports mDCI;

- Recommended WF

  - TBA

# Topic #4: Demoduation requirements for tunnel scenario

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2308037 | Nokia | Observation 6: If the same channel model as in open space scenario is considered in the tunnel deployment, then the Doppler channel profile in tunnel is getting even less challenging than that in the open space. Proposal 4: RAN4 to define new demodulation requirements for tunnel deployment scenario only if a new channel model in the tunnel is introduced. |
| R4-2308832 | Samsung | Observation 1: The maximum Doppler shift experienced by UE in tunnel scenario is equal to 9722Hz, high than that in Open space scenario. Proposal 1: Introduce the UE demodulation requirements with DPS transmission scheme with 1a and 1b for tunnel scenario. - Uni-directional with DPS scheme 1b - Bi-directional with DPS scheme 1a Proposal 2: Introduce single set requirement for PUSCH in tunnel scenario based on Bi-directional scenario in the tunnel scenario. |
| R4-2309665 | Qualcomm | Proposal 3: RAN4 should agree to not introduce dedicated Demodulation requirements for Tunnel Deployment, considering this scenario covered by existing test cases; |
| R4-2308888 | Huawei | Observation 1: Large frequency jump is not observed if the existing FR2 HST-DPS channel model Scenario is reused with some modification based on agreed Ds and Dmin. Proposal 1: BS demodulation requirements can be defined only if new channel model with multi-path propagation introduced. Proposal 2: UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

List of open issues

- Sub-topic 4-1 Test scope for tunnel deployment scenario

  - Issue 4-1-1: Whether to define UE demodulation requirements for tunnel deployment scenario in FR2 HST

  - Issue 4-1-2: Whether to define BS demodulation requirements for tunnel deployment scenario in FR2 HST

### Sub-topic 4-1 Test Scope

Sub-topic description:

Open issues and candidate options before meeting:

Issue 4-1-1: Whether to define UE demodulation requirements for tunnel deployment scenario in FR2 HST

- Observations

  - Observation 1 (Samsung):

    - The maximum Doppler shift experienced by UE in tunnel scenario is equal to 9722Hz, high than that in Open space scenario

  - Observation 2 (Nokia):

    - If the same channel model as in open space scenario is considered in the tunnel deployment, then the Doppler channel profile in tunnel is getting even less challenging than that in the open space.

  - Observation 3 (Huawei):

    - Large frequency jump is not observed if the existing FR2 HST-DPS channel model Scenario is reused with some modification based on agreed Ds and Dmin.

- Proposals

  - Option 1 (Samsung):

    - Introduce the UE demodulation requirements with DPS transmission scheme with 1a and 1b for tunnel scenario.

      - Uni-directional with DPS scheme 1b

      - Bi-directional with DPS scheme 1a

  - Option 2 (Nokia):

    - RAN4 to define new demodulation requirements for tunnel deployment scenario only if a new channel model in the tunnel is introduced.

  - Option 3 (QC):

    - RAN4 should agree to not introduce dedicated Demodulation requirements for Tunnel Deployment, considering this scenario covered by existing test cases;

  - Option 4 (Huawei)

    - UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

- Recommended WF

  - TBA

Issue 4-1-2: Whether to define BS demodulation requirements for tunnel deployment scenario in FR2 HST

- Observations

  - Observation 1 (Nokia):

    - If the same channel model as in open space scenario is considered in the tunnel deployment, then the Doppler channel profile in tunnel is getting even less challenging than that in the open space.

  - Observation 2 (Huawei):

    - Large frequency jump is not observed if the existing FR2 HST-DPS channel model Scenario is reused with some modification based on agreed Ds and Dmin.

- Proposals

  - Option 1 (Samsung):

    - Introduce single set requirement for PUSCH in tunnel scenario based on Bi-directional scenario in the tunnel scenario.

  - Option 2 (Nokia):

    - RAN4 to define new demodulation requirements for tunnel deployment scenario only if a new channel model in the tunnel is introduced.

  - Option 3 (Huawei)

    - UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

- Recommended WF

  - TBA