3GPP TSG-RAN WG4 Meeting # 107																								R4-2310497

Incheon, KR, May 22nd – May 26th , 2023

Title: 	WF on eRedCap UE RF requirements

Agenda Item:	8.32.3

Source: 	Ericsson

Document for:	Approval

# eRedcap UE RF requriement

Issue 1-1-1: New FRC for maximum input level test

- Proposals

  - Option 1: (CT)When considering specifying one new FRC with 256QAM modulation order for eRedcap, we should wait for a consensus on peak rate, that is, wait for the interpretation of RAN#100.

  - Option 2: (Vivo)RAN4 should not update FRC until RAN1 or RAN-P has clear conclusions on ~10Mbps data rate definition.

  - Option 3: (Ericsson) Reduce the PRB size for maximum input level requirement while keep the 64QAM modulation, this applies to both types eRedCap UE, FFS on the specific RB size for either type of the eRedCap.

  - Option 4: (Huawei), according to CR R4-2308580

- Recommended WF

  - Wait RAN1/RAN-P to further clarify the peak data rate, delay any discussion around FRC relating to the peak data reduction.

Agreement:

- Wait RAN1/RAN-P to further clarify the peak data rate, delay any discussion around FRC relating to the peak data reduction.

Issue 1-2-1: Separate requirement or same requirement

- Proposals:

  - Option 1: Define separate requirements for the two different types of eRedCap devices. (Nokia, CT, Huawei, Xiaomi, Ericsson)

  - Option 2: (Vivo) RAN4 further study the Rx requirement impacts before making decision on whether different requirement should be defined for Rel-18 RedCap UEs with different capability.

  - Option 3: TBA

- Recommended WF

  - Define separate requirement for two types of eRedCap UE.

Agreement:

- To define separate requirements for two types of eRedCap UE could be considered in RAN4

  - Assuming the different types of eRedCap UE has different hardware implementations

Issue 1-2-2: requirement for eRedCap (20MHz + PR1)

- Proposals:

  - Option 1: (QC, Xiaomi, Ericsson, Huawei) Apply Rel-17 RedCap RF requirements.

  - Option 2: (Vivo) RAN4 further study the Rx requirement impacts before making decision on whether different requirement should be defined for Rel-18 RedCap UEs with different capability.

- Recommended WF

  - Optoin 1

Agreement:

- Apply Rel-17 RedCap RF requirements to eRedCap (20MHz+PR1)

  - FFS on FRC issue for maximum input level requirements

- Issue 1-3-1: REFSENS of eRedcap UE ( BW3/PR3 + PR1) for wider channel BW (FDD band)

  - There is no scheduling restriction on DL and UL RB position, the scheduling restriction on RB number and position is according to eRedCap type (BW3/PR3 + PR1)

  - The RF performance is the same with legacy RedCap UE when scheduling is the same

  - For REFSENS of eRedCap (BW3/PR3 + PR1) FDD band

    - Option 1: Investigate if new REFSENS of eRedCap (BW3/PR3 + PR1) is needed:

      - FFS on how study on UE REFSENS performance with below parameters helps to decide whether new REFSENS test point is needed

      - The band to be investigated as starting point: n71.

      - UL configuration is the same with legacy RedCap test condition

      - 25 RB is placed at the channel edge with the closest distance to the UL allocation

      - Bands that would need to be studied if option 2 is not followed :

        - At least n5, n8, n12, n20, n26, n28, n71, n85, and n105, other bands not excluded

        - TBD on study and impact on all concerned bands

    - Option 2: Derive the eRedCap REFSENS based on legacy REFSENS with additional modification on test conditions. Options for modification are below

      - 2A: Interlace FRC : Distribute the 25 RB within 106 RB grid

      - 2B: 25 contiguous RB placed in middle of channel BW both in UL and DL

      - 2C: UL on edge, DL with nominal duplex distance away

      - 2D: other option not excluded

. Issue 1-3-4: REFSENS of eRedcap UE ( BW3/PR3 + PR1) for wider channel BW (TDD band)

- Proposals:

  - Option 1 :

    - For TDD bands, the REFSENS requirements currently specified for RedCap UE at 5MHz channel BW can be directly applied to eRedCap UE for all RF channel BWs up to 20MHz, including both 2Rx and 1Rx requirements.

    - For TDD bands with minimum channel bandwidth at 10MHz, the REFSENS requirements for eRedCap UE can be scaled by the DL PRB ratio between eRedCap UE and RedCap UE at 10MHz channel bandwidth.

  - Option 2: TBA