3GPP TSG-RAN WG4 Meeting # 107      	R4-2310057

Incheon, KR, May 22 – May 26, 2023

Title: 	WF on NR ATG RRM requirements

Agenda Item:	8.14.6

Source: 	CMCC

Document for:	Approval

# General

Issue 1-1: BS location accuracy

Agreement:

- Base station is allowed to not provide the exact base station location

- BS location accuracy

  - Note: Nokia will provide the BS accuracy table with different PRACH formats this week.

  - Do not specify the BS location accuracy in RAN4 specs.

  - Capture the BS location accuracy in reply LS to assist RAN2 signaling design.

Issue 1-2: Other information included in LS

- Introduce the mechanism of Koffset in ATG system, whether and how to configure Koffset is up to network implementation. (last meeting agreement)

Issue 1-4: How to involve ATG RRM core requirements in TS38.133

Agreement:

- ATG specific RRM requirements are defined in separate sections like NTN.

# Mobility

# Mobility in RRC_IDLE/INACTIVE

Issue 2-1-1: Cell re-selection mechanism

Agreement

- Distance-based solution

  - Option 1: Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation, provided that existing NTN signalling is reused.

  - Option 2: Do not introduce distance-based triggering measurement.

- Altitude-based solution

  - Common understanding is that ATG UE is turned off lower 3km altitude.

  - The ATG RRM requirements apply for ATG UE operating higher than 3km altitude.

Issue 2-1-3: Cell re-selection measurement requirements

Agreement:

- 	Set 2 applies only to cell detection delays.

Issue 2-1-4: Signalling for Cell re-selection measurement requirements

Agreement:

- Introduce optional UE capability for supporting set2 requirements for ATG

Issue 2-1-5: How to switch between the two sets of cell re-selection requirements

Agreement:

- RAN4 to request RAN2 to define ATG indicator (via system information) which is used to determine which set (set 1 or set 2) of requirements to meet.

  - UE meets set 2 requirements when ATG indicator is received.

  - Otherwise, UE meets set 1 requirement.

- For a particular cell, network can indicate set 1(Legacy R15 cell-reselection requirement) or set 2 (R17 HST cell-reselection requirement) based on cell configuration, for example, the ISD, DRX configuration and so on, which is up to BS implementation.

# Mobility in RRC_CONNECTED

Issue 2-2-1: location-based CHO mechanism

Agreements:

- Option 1: For the location-based CHO, reusing the procedure in R17 NTN for R18 ATG (combines RSRP and location based triggering condition) with existing NTN signaling.

- Option 2: Do not introduce location-based CHO.

Issue 2-2-3: Requirement for location-based CHO if agreed to be introduced

- If location-based CHO is introduced, requirements are as follows:

  - For the requirement of location-based CHO, the definition of Tmeasure should be revised, others can reuse the legacy.

    - DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

  - For Tmeasure of intra-frequency handover,

    - If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index.

    - If both condition D1-1 and condition D1-2 are fulfilled is later than TEvent_DU plus Tidentify_intra_with_index or Tidentify_intra_without_index for intra-frequency handover, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition D1-1 and condition D1-2 are fulfilled.

  - For Tmeasure of inter-frequency handover,

    - If both condition D1-1 and condition D1-2 are fulfilled earlier than TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equal to Tidentify_inter_with_index or Tidentify_inter_without_index.

    - If both condition D1-1 and condition D1-2 are fulfilled later than TEvent_DU plus Tidentify_inter_with_index or Tidentify_inter_without_index, then the measurement time delay equal to the time from the end of Tevent_DU until time of both condition D1-1 and condition D1-2 are fulfilled.

# Timing

# Timing requirements

Issue 3-1-1: Initial transmit timing requirements Te

Agreement:

- For ATG network, it is suggested that the ideal UE-specific TA is based on the indicated BS location and the actual UE location, where the UE pre-compensation timing error does not consider the timing error due to the misalignment between the indicated BS location and the actual BS location.

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_ATG |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 20*64*Tc |
|  |  | 30 | 18*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 16*64*Tc |
|  |  | 30 | 16*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Issue 3-1-2: Gradual timing adjustment

Agreement:

- For ATG network, the maximum autonomous time adjustment step (Tq_ATG) and the minimum aggregate adjustment rate (Tp_ATG) for gradual timing adjustment requirments can be defined as follows:

| Frequency Range | SCS of uplink signals (kHz) | Tq_ATG | Tp_ATG |
| --- | --- | --- | --- |
| 1 | 15 | 9.5*64*Tc | 9.5*64*Tc |
|  | 30 | 9.5*64*Tc | 9.5*64*Tc |
|  | 60 | N/A | N/A |

# Guard period as Large TDD cell impaction

Issue 3-2-1: Guard period issue due to large TDD cell and coexistence demand

Agreement:

- Further discuss the following proposals in next meeting

  - Introduce NTN TA reporting for ATG UE.

  - Whether and how to use the reported TA to avoid UL and DL collision is FFS.

# Measurement

Issue 4-1: Measurement mechanism

Agreement:

- Altitude-based solution

  - Common understanding is that ATG UE is turned off lower 3km altitude.

  - The ATG RRM requirements apply for ATG UE operating higher than 3km altitude.

Issue 4-2: Directional antenna impaction

Agreement:

- Further discuss the scaling factor in next meeting

  - Option 1: Do not introduce scaling factor N

    - Existing measurement requirements can be reused, provided that beam steering based on the positioning is capable for the ATG UE.

    - Introduce scheduling restriction due to different Rx beam applies in FR1

  - Option 2: For UEs with directional antennas operating in 4 GHz, use a scaling N for ATG UEs measurement requirements. FFS on the value of ‘N’.

Issue 4-3: Scheduling restriction-for intra-frequency

Tentative Agreement:

- The legacy scheduling restriction requirement can be reused for ATG.

  - When it is enabled, the restriction is on SSB symbols and 1 data symbol before and after.

  - When it is disabled, the restriction is on all symbols in SMTC.

Issue 4-4: Scheduling restriction-for inter-frequency

Tentative Agreement:

- For ATG inter-frequency measurement without gap, the legacy scheduling restriction can be reused.

# CR split

| Section | Sub-section | Company |
| --- | --- | --- |
| 3 Definitions, symbols and abbreviations | 3.3 Abbreviations |  |
|  | 3.6.x Applicability of requirements for ATG |  |
| 4 RRC_IDLE state mobility | 4.2D Cell Re-selection for ATG |  |
|  | 4.3D Minimization of Drive Tests (MDT) |  |
| 5 RRC_INACTIVE state mobility | 5.1.D Cell Re-selection for ATG |  |
|  | 5.3D Minimization of Drive Tests (MDT) |  |
|  | 5.5D Configured Grant based Small Data Transmissions (CG-SDT) for ATG |  |
|  | 5.7D Random access based Small Data Transmissions (RA-SDT) for ATG |  |
| 6 RRC_CONNECTED state mobility | 6.1D Handover for ATG |  |
|  | 6.2D RRC Connection Mobility Control for ATG |  |
| 7 Timing | 7.1D UE transmit timing for ATG |  |
|  | 7.2D UE timer accuracy for ATG |  |
|  | 7.3D Timing advance for ATG |  |
|  | 7.4D Cell phase synchronization accuracy for ATG |  |
|  | 7.7D deriveSSB-IndexFromCell tolerance for ATG |  |
|  | 7.9D deriveSSB-IndexFromcell tolerance for ATG |  |
| 8 Signaling characteristics | 8.1D Radio Link monitoring for ATG |  |
|  | 8.4D UE UL carrier RRC reconfiguration delay for ATG |  |
|  | 8.5D Link Recovery Procedures for ATG |  |
|  | 8.6D Active BWP switch delay for ATG |  |
|  | 8.13D UE-specific CBW change for ATG |  |
|  | 8.15D Active downlink TCI state switching delay for unified TCI for ATG |  |
|  | 8.16D Active uplink TCI state switching delay for unified TCI for ATG |  |
|  | 8.19D Pre-configured measurement gap activation/deactivation delay for ATG |  |
| 9 Measurement Procedure | 9.1D General measurement requirement for ATG |  |
|  | 9.2D NR intra-frequency measurements for ATG |  |
|  | 9.3D NR inter-frequency measurements for ATG |  |
|  | 9.5D L1-RSRP measurements for Reporting |  |
|  | 9.8D L1-SINR measurements for Reporting |  |
|  | 9.10D CSI-RS based L3 measurements |  |
|  | 9.11D NR measurements with autonomous gaps |  |