3GPP TSG-RAN4 Meeting #108	R4-2313549

# Toulouse, France, 21 - 25 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1802 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_PC2_CA_R17_2BDL_2BUL-Core] CR to 38.101-1, Rel-17 Cat F Powerclass indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_CA_R17_2BDL_2BUL-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Minimum requirements for HPUE are unclear as RAN4 specification does not specify which minimum requirements apply when power class signaling is present and which power class applies when some signaling parameters are absent. Relationship with power class definition for CA in clause 6.2A.1 and CA configurations in clause 5.5A is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 6.2A.1 general section is added to define how power class is indicated and modified by signaling, making minimum requirements clear both in the presence and absence of signaling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HPUE minimum requirements and their relationship to signaling remain unclear and unspecified in RAN4, resulting in high likelyhood of misunderstanding the requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.0, 6.2A.1.1, 6.2A.1.2, 6.2A.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of Changes>

### 6.2A.1	UE maximum output power for CA

#### 6.2A.1.0	General

By default, for each band of the CA configuration, UE shall meet the power class indicated by ue-Powerclass IE or its extensions. Power class of a band cannot be higher than the power class of the band combination, which can be indicated by PowerClass IE. If PowerClass is not indicated, default power class applies to the band combination. ue-PowerClassPerBandPerBC-r17 can be used to indicate lower power class for a band in band combination than set by ue-PowerClass and PowerClass. This does not change the power class of the band combination. If ue-PowerClassPerBandPerBC-r17 is not signalled the earlier rules apply.  The CA configurations for which support of other power class than power class 3 has been evaluated are indicated in clauses 5.5A.1, 5.5A.2 and 5.5A.3.

#### 6.2A.1.1	UE maximum output power for Intra-band contiguous CA

For uplink intra-band contiguous carrier aggregation, the maximum output power is specified in Table 6.2A.1.1-1. For downlink intra-band contiguous carrier aggregation with a single uplink component carrier configured in the NR band, the maximum output power is specified in Table 6.2.1-1.

Table 6.2A.1.1-1: UE Power Class for intra-band contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B |  |  |  |  | 23 | +2/-2 |  |  |
| CA_n7B |  |  |  |  | 23 | +2/-2 |  |  |
| CA_n40B |  |  |  |  | 23 | +2/-2 |  |  |
| CA_n41B |  |  |  |  | 23 | +2/-21 |  |  |
| CA_n41C |  |  | 26 | +2/-3 | 23 | +2/-21 |  |  |
| CA_n48B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n77C |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n78C |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n79C |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: An uplink CA configuration in which the band has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of the band are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For intra-band contiguous carrier aggregation the maximum power requirement shall apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

#### 6.2A.1.2	UE maximum output power for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier on the PCC, the requirements in clause 6.2.1 apply. For intra-band non-contiguous carrier aggregation with two uplink carriers the maximum output power is specified in Table 6.2A.1.2-1.

Table 6.2A.1.2-1: UE Power Class for intraband non-contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41(2A) |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n77(2A) |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78(2A) |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: An uplink CA configuration in which the band has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of the band are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For intra-band non-contiguous carrier aggregation the maximum power requirement shall apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

#### 6.2A.1.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in Table 6.2.1-1 apply.

For inter-band carrier aggregation with two uplink contiguous carrier assigned to one NR band, the transmitter power requirements specified in subclause 6.2A.1.1 apply.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the transmitter power requirements specified in subclause 6.2A.1.2 apply. For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.3-1.

< End of Changes>