3GPP TSG-RAN WG4 Meeting #108	R4-2313876

Toulouse, France, 21 August ‒ 25 August 2023

Title: 	WF for NonCol_intraB_ENDC_NR_CA_Demod

Agenda Item:	8.11.5

Source: 	Ericsson

Document for:	Approval

# Topic 1:	Type 2 UE demodulation requirements

## Sub-topic 1-1: Test setup for Type 2 UE NR-CA PDSCH demodulation requirements

Test setup and parameters

Agreement:

- Reuse the existing PDSCH CA demodulation requirements (e.g., TS 38.101-4 5.2A.2.1)

Other parameter configurations

Agreement:

- Reusing test set-up from existing CA demodulation requirements for TRS and SSB configuration.

- Configure 33us received time difference (RTD) between PCell and SCell.

|  | Type 2 UE NR-CA demodulation requirements |
| --- | --- |
| Received power difference | 25 dB |
| Received time difference | 33 µs |
| Channel model | [AWGN] |
| Maximum number of HARQ transmission | [1] |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |
| Throughput measurement procedure | Measure both carriers |
| Tx antenna configuration and rank | Option1: 2Tx, rank2 for both carriers Option 2: 1Tx with rank1 for carrier with lower power 2Tx with rank2 for another carrier Option 3: rank 1 for both carriers with 1Tx |
| MCS table and MCS index | FFS |
| Test metric | 70% of the maximum throughput |

MCS pair and SNR test point pair

Way forward: Interested companies are encouraged to bring the simulation results in RANM4#108bis to decide the MCS index pair and corresponding SNR test point pair according to the agreed test setup and test metrics.

Note the required SNR difference between the carriers with lower power and higher power should not exceed 25dB.

|  | Tx antenna and rank | MCS table | Test metric |
| --- | --- | --- | --- |
| Carrier with lower power | Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1 | Option 1: MCS index 1 Option 2: MCS index 2 | 70% of the maximum throughput |
| Carrier with higher power | Option 1: 2Tx, Rank 2 Option 2: 1Tx, Rank 1 | Option 1: MCS index 1 Option 2: MCS index 2 | 70% of the maximum throughput |