3GPP TSG-RAN WG4 Meeting # 108												R4-231xxxx

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

Agenda item:			5.1

Source:	Moderator (Ericsson)

Title:	Topic summary for [108][102] R17_spectrum_maintenance

Document for:	Information

# Introduction

This document is a summary of the proposals made in the contributions submitted under AI 5.1 for the RAN4 #108 meeting.

# Topic #1: RMR bands n100 and n101.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2313584 | Huawei, HiSilicon | Proposal 1: Not to remove additional RMR-specific BS output power requirements defined in TS 38.104 for BS type 1-C and bands n100/n101, and instead modify them to provide the intended (deployment) flexibility as outlined in ECC/DEC/(20)02. Proposal 2: Implement corrections for additional RMR-specific BS output power requirement in TS 38.104 and TS 38.141-1 by using manufacturer declarations for n100/n101 antenna gain values. Proposal 3: Apply the agreeable solution to all n100/n101 BS RF requirements defined based on the fixed antenna gain, i.e. BS output power, Tx spur emissions, OBUE, where applicable. Proposal 4: Add a clarification note in TS 38.104 and TS 38.141-1, clarifying applicability of additional BS output power requirements for n100 and n101 for the uncoordinated deployment. Proposal 5: Not to remove additional OBUE limits for n100 from TS 38.104 (clause 6.6.4.2.5.7) and TS 38.141-1 (clause 6.6.4.5.6.7), and provide related feedback to ECC WG FM. Proposal 6: Send related LS back to ECC WG FM to inform them on the approach taken by RAN4 for the additional RMR-specific BS output power requirements. Draft LSout was submitted in [5] under AI 10.2.3. |

## Open issues summary

### Sub-topic 1-1

Sub-topic description: ECC Decision(20)02 specified EIRP limits for non-AAS BS and uncoordinated deployment. As RAN4 specifies conducted limits, the ECC Decision limits were converted in conducted ones assuming a certain antenna gain and losses. The proposal would be to use another approach to handle those EIRP limits specified in ECC Decision, keeping the consideration of uncoordinated deployment.

Issue 1-1-1: BS max output power for n100 and n101

- Proposals: Not to remove additional RMR-specific BS output power requirements defined in TS 38.104 for BS type 1-C and bands n100/n101, and instead modify them to provide the intended (deployment) flexibility as outlined in ECC/DEC/(20)02.

  - Agree

  - Disagree

- Recommended WF

  - TBA

Issue 1-1-2: Introduce manufacturer declaration approach to convert EIRP limits

- Proposals: Implement corrections for additional RMR-specific BS output power requirement in TS 38.104 and TS 38.141-1 by using manufacturer declarations for n100/n101 antenna gain values.

  - Agree

  - Disagree

- Recommended WF

  - TBA

Issue 1-1-3: Similar approach for other requirements specified as EIRP limits in ECC Decision

- Proposals: Apply the agreeable solution to all n100/n101 BS RF requirements defined based on the fixed antenna gain, i.e. BS output power, Tx spur emissions, OBUE, where applicable.

  - Agree

  - Disagree

- Recommended WF

  - TBA

Issue 1-1-4: Clarify the BS maximum output power should be considered for uncoordinated deployment

- Proposals: Add a clarification note in TS 38.104 and TS 38.141-1, clarifying applicability of additional BS output power requirements for n100 and n101 for the uncoordinated deployment.

  - Agree

  - Disagree

- Recommended WF

  - TBA

### Sub-topic 1-2

Sub-topic description: The LS received from WG FM recommended RAN4 to remove the additional OBUE requirements specified for band n100, arguing it was assumed equivalent to the OBUE requirements for WA BS.

Issue 1-2: Additional OBUE requirements for band n100

- Proposals: Not to remove additional OBUE limits for n100 from TS 38.104 (clause 6.6.4.2.5.7) and TS 38.141-1 (clause 6.6.4.5.6.7), and provide related feedback to ECC WG FM.

  - Agree

  - Disagree

- Recommended WF

  - TBA

### Sub-topic 1-3

Sub-topic description LS reply to WG FM

Issue 1-3: LS Reply to WG FM

- Proposals: Send related LS back to ECC WG FM to inform them on the approach taken by RAN4 for the additional RMR-specific BS output power requirements.

  - Agree

  - Disagree

- Recommended WF

  - This topic is already handled in thread #304 and should not be discussed here.

# Topic #2: New UE capability for intra-band EN-DC BCS

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2313499 | Google Inc. | Proposal 1: Introduce a new UE capability signaling to indicate intra-band EN-DC BCS for the second intra-band EN-DC component in inter-band EN-DC band combination with two additional intra-band EN-DC components. Proposal 2: Send LS in the Annex to RAN2. |

## Open issues summary

### Sub-topic 2-1

Sub-topic description: The contribution relates an ambiguous issue by indicating one IE for intra-band EN-DC BCS reporting in inter-band EN-DC band combination with two additional intra-band EN-DC components, this could be solved by introducing a new capability signalling to indicate intra-band EN-DC BCS for the second intra-band EN-DC component

Issue 2-1: New UE capability

- Proposals: Introduce a new UE capability signaling to indicate intra-band EN-DC BCS for the second intra-band EN-DC component in inter-band EN-DC band combination with two additional intra-band EN-DC components

  - Agree

  - Disagree

- Recommended WF

  - TBA

### Sub-topic 2-2

Sub-topic description Pending on issue 2-1, send LS to RAN2.

Issue 2-2: LS to RAN2

- Proposals: Agree on the LS to RAN2 (proposed in Annex)

  - Agree

  - Disagree

- Recommended WF

  - TBA

# Topic #2: CRs or Draft CRs

## Companies’ contributions summary

The following contributions give detailed rationale on the correponding CRs, proposing to approve those CRs.

| T-doc number | Source | Proposals | Corresponding CRs |
| --- | --- | --- | --- |
| R4-2312175 | MediaTek Inc. | Observation 1: For MSD due to 4th order receiver harmonic mixing for a low-band + C band CA/DC, a few dB MSD may be needed for typical UE receiver design. Proposal 1: Agree on the R4-2309346 CR to R15 TS38.101-3 for addition of missing MSD as is Proposal 2: MSD due to 4th order receiver harmonic mixing for CA_n5/n26-n77 and CA_n8-n77 are proposed to be added as below: Moderator’s note: R4-2309346 was agreed in RAN4#107, proposal1 doesn’t have to be discussed in this meeting then. | R4-2312176 |
| R4-2311142 | Skyworks Solutions, Inc. | Observation: For CA_n41-n79, and CA_n78-n79, the PC2, PC1.5 MSD test points due to cross-band isolation are specified for n79 40MHz DL CBW. According to WF [1], the request for BCS4/5 introduces a new band n79 lowest CBW of 10MHz. These MSD test points shall be updated. Proposal: For Rel-18, endorse the Rel-18 and Rel-17 CRs [2,3] which capture the corrections to the band n79 MSD test point highlighted in green in Table 5 and Table 6. | R4-2311131 R4-2311132 |

The following table lists all CRs submitted under AI 5.1. This list is also available in NWM to capture any early flag.

| T-doc number | Title | Source |
| --- | --- | --- |
| R4-2312751 | [NR_n53-Core] CR on 38.101-1 Correction on the range of GSCN in 30 KHz SCS for Sync Raster of Band n53 | Huawei, Hisilicon. |
| R4-2312752 | [NR_n53-Core] CR on 38.104: Correction on the range of GSCN in 30 KHz SCS for Sync Raster of Band n53 | Huawei, Hisilicon. |
| R4-2312318 | [NR_6GHz-Core] CR to 38.104: applicability note for n104 | Huawei, HiSilicon, CMCC, China Telecom, China Unicom, CBN, CATT, OPPO, Vivo, Xiaomi |
| R4-2312319 | [NR_6GHz-Core] CR to 38.104: applicability note for n104 | Huawei, HiSilicon, CMCC, China Telecom, China Unicom, CBN, CATT, OPPO, Vivo, Xiaomi |
| R4-2312320 | [NR_6GHz-Core] CR to 38.101-1: applicability note for n104 | Huawei, HiSilicon, CMCC, China Telecom, China Unicom, CBN, CATT, OPPO, Vivo, Xiaomi |
| R4-2312321 | [NR_6GHz-Core] CR to 38.101-1: applicability note for n104 | Huawei, HiSilicon, CMCC, China Telecom, China Unicom, CBN, CATT, OPPO, Vivo, Xiaomi |
| R4-2313224 | [NR_CADC_R17_2BDL_xBUL-Core] CR on TS38.101-1 Modification on MSD due to UL harmonic interference for CA _n48A-n66A | Huawei,HiSilicon |
| R4-2313225 | [NR_CADC_R17_2BDL_xBUL-Core] CR on TS38.101-1 Modification on MSD due to UL harmonic interference for CA _n48A-n66A | Huawei,HiSilicon |
| R4-2313226 | [NR_CADC_R17_2BDL_xBUL-Core] CR on R17 TS38.101-1 Remove the PC2 CA configuration | Huawei,HiSilicon |
| R4-2312176 | CR for missing MSD due to 4th order harmonic mixing requirements | MediaTek Inc. |
| R4-2312177 | [NR_CADC_R17_2BDL_xBUL-Core]CR for missing MSD due to 4th order harmonic mixing requirements | MediaTek Inc. |
| R4-2312275 | [NR_CADC_R17_2BDL_xBUL-Core ] CR on MSD for shared spectrum band | LG Electronics France |
| R4-2312276 | [NR_CADC_R17_2BDL_xBUL-Core ] CR on MSD for shared spectrum band | LG Electronics France |
| R4-2312983 | Rel17 Cat F CR for 38.101-3 Correct the UL configuration for some concerned FR1+FR2 combo | Samsung |
| R4-2312984 | [NR_CADC_R18_3BDL_xBUL]Rel-18 Cat A CR for 38.101-3 Correct the UL configuration for some concerned FR1+FR2 combo | Samsung |
| R4-2312453 | [DC_R17_1BLTE_1BNR_2DL2UL-Core] Corrections on Tx requirements for inter-band NEDC including FR2 | ZTE Corporation, CHTTL |
| R4-2312454 | [DC_R17_1BLTE_1BNR_2DL2UL-Core] Corrections on Tx requirements for inter-band NEDC including FR2 | ZTE Corporation, CHTTL |
| R4-2313585 | [NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core] CR to TS 38.104: corrections of RMR-specific BS requirements for band n100 and n101, Rel-17 | Huawei, HiSilicon |
| R4-2313586 | [NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core] CR to TS 38.104: corrections of RMR-specific BS requirements for band n100 and n101, Rel-18 | Huawei, HiSilicon |
| R4-2313587 | [NR_RAIL_EU_900MHz-Perf, NR_RAIL_EU_1900MHz_TDD-Perf] CR to TS 38.141-1: corrections of RMR-specific BS requirements for band n100 and n101, Rel-17 | Huawei, HiSilicon |
| R4-2313588 | [NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core] CR to TS 38.141-1: corrections of RMR-specific BS requirements for band n100 and n101, Rel-18 | Huawei, HiSilicon |
| R4-2313589 | [NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core] CR to TR 38.852: corrections of RMR-specific BS requirements derivation for band n101, Rel-17 | Huawei, HiSilicon |
| R4-2313590 | [NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core] CR to TR 38.852: corrections of RMR-specific BS requirements derivation for band n101, Rel-18 | Huawei, HiSilicon |
| R4-2313591 | [NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core] CR to TR 38.853: corrections of RMR-specific BS requirements derivation for band n100, Rel-17 | Huawei, HiSilicon |
| R4-2313592 | [NR_RAIL_EU_900MHz-Core, NR_RAIL_EU_1900MHz_TDD-Core] CR to TR 38.853: corrections of RMR-specific BS requirements derivation for band n100, Rel-18 | Huawei, HiSilicon |
| R4-2312985 | Rel17 Cat F CR for 38.101-3 Correct the note for HPUE indication in configuration table | Samsung |
| R4-2312986 | [HPUE_FR1_DC_LTE_NR_R18]Rel18 Cat A CR for 38.101-3 Correct the note for HPUE indication in configuration table | Samsung |
| R4-2311124 | CR to TS 38.101-1 Rel-18 Corrections to UE co-existence requirements | Skyworks Solutions Inc. |
| R4-2311125 | CR to TS 38.101-1 Rel-17 Corrections to UE co-existence requirements | Skyworks Solutions Inc. |
| R4-2311126 | CR to TS 38.101-1 Rel-18 Introduction of TDD uplink RMC for shorter transients | Skyworks Solutions, Inc. |
| R4-2311127 | CR to TS 38.101-1 Rel-17 Introduction of TDD uplink RMC for shorter transients | Skyworks Solutions, Inc. |
| R4-2311129 | CR to TS 38.101-1 Rel-18 SUL MSD alignment | Skyworks Solutions, Inc. |
| R4-2311130 | CR to TS 38.101-1 Rel-17 SUL MSD alignment | Skyworks Solutions, Inc. |
| R4-2311131 | CR to TS 38.101-1 Rel-18 Corrections to cross-band MSD and CA_n7-n38 fallback | Skyworks Solutions, Inc. |
| R4-2311132 | CR to TS 38.101-1 Rel-17 Corrections to cross-band MSD | Skyworks Solutions Inc. |
| R4-2311476 | [TEI18] CR 38.101-1: Various maintenance issues R18 | Nokia |
| R4-2311477 | [TEI17] CR 38.101-1: Various maintenance issues R17 | Nokia |
| R4-2312981 | Rel17 Cat F CR for 38.101-1 Correct ?PPowerClass,CA relevant requirement in clause 6.2A.1.3 for inter-band ULCA | Samsung, Huawei, KT corporation, CHTTL, SGS Wireless |
| R4-2312982 | NR_SAR_PC2_interB_SUL_2BUL]Rel18 Cat A CR for 38.101-1 Correct ?PPowerClass,CA relevant requirement in clause 6.2A.1.3 for inter-band ULCA | Samsung, Huawei, KT corporation,CHTTL, SGS Wireless |
| R4-2313582 | [NR_ext_to_71GHz-Perf] CR to TS 38.141-2: propagation conditions annex J reference corrections, Rel-17 | Huawei, HiSilicon |
| R4-2313583 | [NR_ext_to_71GHz-Perf] CR to TS 38.141-2: propagation conditions annex J reference corrections, Rel-18 | Huawei, HiSilicon |