3GPP TSG RAN WG 4 Meeting #108bis		R4-2315907

Xiamen, China, October 9 – October 14, 2023

Title:	LS on UE capability and network assistant signalling for advanced receivers

Response to:

Release:	Release 18

Work Item:	NR_demod_enh3

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:	TSG RAN WG1

Contact person:	Karsten Petersen

karsten.petersen@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

Within the Release 18 work item on further enhancement on NR demodulation performance (NR_perf_enh3_Demod), RAN4 has discussed UE capability and network assistant signalling for MU-MIMO advanced receiver(s) for the following scenario:

- Scenario 1: UE capable of utilizing advance receiver supporting cancellation of 1 or more co-scheduled UEs (via MU-MIMO).

As an outcome, RAN4 has agreed the following:

- From the RAN4 perspective, new Rel-18 NR UE capabilities needs to be defined for MU-MIMO capable UEs with granularity per UE, no FDD/TDD difference and for FR1 only:

  - Capability #1: NR UE capable of using advanced receiver for MU-MIMO.

  - Tentative: [Capability #2: NR UE capable of R-ML with modulation order blind detection]

  - Tentative: [Capability #3a: Maximum number of layers of co-UE the NR UE is capable of for joint detection]

  - Tentative: [Capability #3b: Total number of layers the NR UE is capable of for joint detection]

  - Tentative: [Capability #4: Maximum number of DMRS ports NR UE can blindly detect]

  - Tentative: [Capability #5: Maximum modulation orders of interfering DMRS ports supported by NR UE]

- To perform MU-MIMO using advanced receiver(s), RAN4 has agreed that the UE should have the following default network configuration assumptions for the co-scheduled UEs:

  - PRB bundling size for the co-scheduled UE and the Frequency domain resource allocation for the co-schedule UE within each PRG of the UE: For the target and any co-scheduled UEs in different CDM groups and with the same DMRS sequence, the target UE assumes the precoding and resource allocation of the co-scheduled UE are the same in the PRG-level grid configured to the target UE when PRG=2 or 4.

  - DMRS power boosting for the co-scheduled UE(s) is the same as the UE

  - Time domain resource allocation information of the co-scheduled UE(s) is the same as the target UE

  - Tentative: [UE assumes the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE(s)]

  - Tentative: [UE assume the same frequency domain resource allocation type for target and co-UE]

- In addition, RAN4 agreed the need to introduce new Rel-18 optional RRC based network assistant signalling to be created for to assist UEs supporting Capability #1.

- Regarding the content of the Rel-18 new RRC network assistance signalling, for each UE, RAN4 has agreed the need for the following:

  - PRB bundling size for the co-scheduled UE and the Frequency domain resource allocation for the co-schedule UE within each PRG of the UE

    - The network can signal if above default assumption is not valid.

  - DMRS power boosting

    - The network can signal if above default assumption is not valid.

  - Time domain resource allocation information of the co-scheduled UE(s)

    - The network can signal if above default assumption is not valid.

  - Tentative: [UE assumes the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE(s)

    - The network can signal if above default assumption is not valid.]

  - Tentative: [UE assume the same frequency domain resource allocation type for target and co-UE

    - The network can signal if above default assumption is not valid.]

  - The MCS table used by the co-scheduled UEs

    - The network can optionally signal if MCS table with 256 QAM or 1024 QAM is used or not.

# 2	Actions

To RAN WG2

ACTION: 	RAN4 respectfully request RAN2 to take the above information into account and design the corresponding network assistance signalling for advance receivers.

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN WG4 Meeting #109		13 Nov. – 17 Nov., 2023	Chicago, US

TSG-RAN WG4 Meeting #110		26 Feb. – 1 Mar., 2024	Athens, Greece