3GPP TSG-RAN WG4 Meeting #108-bis	 R4-2316091

Xiamen, China, October 9 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Proposal for power back off for aerial NR UEs

Agenda item:	5.35.2

Document for:	Discussion and Approval

Introduction

The UAV WI, as presented in [1], includes the following objectives with RAN4 impact highlighted:

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. 2. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2] Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. 3. Specify the support for UAV identification broadcast (BRID) in NR PC5. Support of DAA using the same framework as BRID without DAA specific enhancements can be considered [RAN2]. Note:. UAV use of NR PC5 is to be used only in designated bands as defined in regulation for BRID/DAA use. 4. Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling [RAN1, RAN2]: - FR1 with directional antenna at UE side 5. Based on the technical conditions defined for aerial UE usage in ECC Decision (22)07, study and specify the necessary UE types and additional OOBE requirements for aerial UEs in 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz. [RAN4]. Note: In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE Note2: Applicability of power classes for aerial UE may need to be addressed in RAN4 Note3: RAN4 to identify the supported bands for aerial UE impacted by above information.

In this contribution we present further considerations for support of UAVs by the RAN4 specification.

Discussion

Background

During RAN99, an objective with RAN4 impact was added to the UAV WID [1]. RAN4 is requested to specify necessary UE types for additional Out Of Band Emissions (OOBE) requirements for aerial UE operating the frequency ranges; 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz according to ECC Decision (22)07.

The LS sent by ECC includes 4 requests to ETSI[2]:

Based on the ECC Decision (22)07, ECC invites ETSI to include the following requirements in future ETSI harmonised standard on aerial UE to ensure a coherence with this ECC framework: - Additional OOBE requirements applicable to aerial UEs in the following frequency bands 1710-1785 MHz, 2500-2570 MHz, 2570-2620 MHz, as defined by ECC Decision 22(07). In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE. - a mechanism/feature coherent with the above aerial UE definition in order to differentiate aerial UE, as defined by ECC Decision 22(07) from terrestrial UE operating under LTE/NR 5G networks - differentiation of aerial UE from other terrestrial UE shall not be changed by the end-user - d) the aerial UE shall not be capable to connect to LTE/NR 5G networks without aerial subscription

The objective added to the WID for addressing the LS from ECC clearly states that it is necessary “study and specify the necessary UE types and additional OOBE requirements for aerial UEs…”

The LS sent by ECC presents a list of requests to 3GPP, as previously mentioned in the discussion. The requests identified by b) and c) concern the discussion of the aerial UE types and the applicability of the respective further restrictions on emissions:

- Additional OOBE requirements applicable to aerial UEs in the following frequency bands 1710-1785 MHz, 2500-2570 MHz, 2570-2620 MHz, as defined by ECC Decision 22(07). In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE.

In this document we will address the OOBE requirements and the resulting A-MPR. The remaining aspects of the ECC LS is addressed in [3].

Affected NR bands

When considering the frequency ranges requested protected by ECC it can be identified that they correspond or overlap the NR bands as defined below from 38.101-1.

TS 38.101-1 - Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n3810 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD5 |
| NOTE 5: Unless otherwise stated, the applicability of requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the requirements for Band n41. A UE supporting Band n90 shall also support band n41. NOTE 10: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. |  |  |  |

According to the ECC decision:

There is a need for OOBE limits for aerial UE operating in the following frequency bands: 1710-1785 MHz: Protection of MetSat operating in the 1675-1710 MHz frequency band Out-of-band limit: -40 dBm/MHz in the frequency range 1675-1710 MHz for aerial UE operating in 1710-1785 MHz. 2500-2570 MHz / 2570-2620 MHz: Protection of RAS operating in 2690 – 2700 MHz and radars operating in 2700-2900 MHz Out-of-band limit: -50 dBm/MHz in the frequency range 2690-2900 MHz for aerial UE operating in 2500-2570 MHz or 2570-2620 MHz. In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE.

From the list of potential affected bands only UL of band 3, 7 and 38 may be affected since these to our understanding are the only ones operated in Europe and this additional restriction comes from ECC with jurisdiction only here.

Affected bands of the additional ECC OOBE limits are UL of both LTE and NR bands 3, 7 and 38.

Due to this, it was agreed at RAN4#107 to only discuss MPR/A-MPR for band n3/3, n7/7 and n38/38 as captured in the WF [5]. At RAN4#108 it was further agreed to only consider band B3, n3, B38 and n38 [6]. The reasoning for this was that it was found that there was no impact to band n7/B7.

It was agreed at RAN4#108 only to consider A-MPR for LTE and NR bands 3 and 38.

As a result of this the study for needed MPR/A-MPR is restricted to these two bands.

Existing NSs for n3 and n38

For NR the following NSs (besides NS_01) is listed in 38.101-1 (Table 6.2.3.1-1A) for the three bands:

- n3 – NS_100

- n38 – NS_44

From above it is clear that for at least for band n3 and n38 there are existing NSs which needs to be checked if they can be reused.

There are existing NSs containing A-MPR defined for band n3 and n38.

For n3, NS_100 is providing UTRA protection meaning it can’t be directly reused for aerial UEs. For n38 NS_44 could be a candidate, especially since the applicable region is also ECC governed.

However, these NSs are applicable for “normal” NR UEs while the additional requirement is only applicable to aerial UEs. Further discussion on the NS definition applicable only for aerial UEs is include in [9] which is also submitted for this meeting.

It seems no existing NSs for the three bands can be directly reused.

Simulation alignment

During RAN4#106bis a WF [4] were agreed which outlines the initial assumptions for the back off study (MPR/A-MPR). The agreed points have been included below:

<Way forward>: - P1 - For n3, RAN4 will perform back off analysis against MetSat protection on 1675-1710 MHz focusing on edge channels and specify the frequency distance when no back off is needed - P2 - For n7 and n38 protection of RAS 2690 – 2700 MHz and radars operating in 2700-2900 MHz analysis is done separately for n7 and n38 - Next meeting focuses on n38 - Concerned companies are encouraged to bring studies for n7 if seen necessary - P3 - For calibration, the current NR MPR will be used for submitted results for NR, meaning 0 MPR is as defined currently for n3, n7 and n38.

Additionally, in RAN4#108 a WF [6] were agreed which clarifies:

<Way forward>: - RAN4 will work on the definition of A-MPR for band B3 and n3 based on the provided simulations and measurements. - RAN4 will work on the definition of A-MPR for band B38 and n38 based on the provided simulations and measurements.

Simulation results, based on the alignment have been included in Annex A and Annex B and were also discussed during previous meetings, as presented in [7] and [8].

A-MPR Study and Proposals for n3

Additional spurious emission limits for UE co-existence are considered according to 38.101-1, as shown in Table 6.5.3.2-1 from 38.101-1. This Table list UE to UE spurious emission requirements. The additional requirement from ECC is however a specific requirement why it shall not be added here but under a NS.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence (n3)

| Frequency range (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 1884.5 MHz – 1915.7 MHz | -41 | 0.3 MHz | NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |

The additional requirement from ECC for n3 shall not be included to Table 6.5.3.2-1 of 38.101-1 but be placed under NS specific for aerial UEs.

A NS applicable for Aerial UE, as further discussed in [9], is to be defined which means this restriction also shall be included in clause 6.5.3.3 (Additional spurious emissions) of 38.101-1 as shown in TP on the following page.

***** Begin TP with Needed Changes to TS ****

6.5.3.3.x 	Minimum requirement (network signalled value “NS_xx”)

When “NS_xx” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.x-x. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.x-x: Additional requirements for “NS_xx”

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20, 30, 35, 40, 45, 50 |  |  |
| 1675 ≤ f ≤ 1710 | -40 | 1 MHz |  |

***** End TP ****

RAN4 to add the additional requirements for n3 as shown in the TP above to the specification.

Simulation Results

As presented in and Annex A, and also discussed during previous meetings, as presented in [7] and [8], the needed A-MPR due to the ECC mask in the 1675-1710 MHz range is very large when the channel is placed at the lower edge of the band. In Figure 1we present examples.

- Figure 1: Examples of the A-MPR when the channel is placed at the lower edge of the band n3.

Very large values of A-MPR, even above 20 dB, are required if the channel is placed at the lower edge of the band n3.

It can be noted that there are some minor differences here as compared the LTE study for band B3 as discussed in [10]. The differences to B3 are likely due to the use of baseband filtering in LTE instead of WOLA in NR.

Complete sets of results for channel BWs from 30 to 50 MHz at the upper edge of the band can be found in Annex A; below we present examples for 50 MHz.

Figure 2: Examples of the A-MPR with the 50 MHz channel BW when the channel is at the upper edge of the band n3.

Smaller A-MPR is required for channel BWs from 30 to 50 MHz if the channel is at the upper edge of the band n3.

The very large A-MPR needed, is a result of the additional requirement from ECC just below band n3. As a result, placing the aerial UE channel with an offset to the n3 lower band edge have been investigated.

Figure 3: Examples of the A-MPR when the channel is placed with 9 MHz and 33 MHz offset from the lower edge of the band n3 respectively for the 5 MHz and 20 MHz channel bandwidth.

The additional A-MPR simulations concludes that if the channel is placed either at the upper edge of the band or with an offset to the lower edge it is possible to avoid A-MPR completely for some channel placements.

A-MPR can be avoided for some channel placements of specific channel bandwidths.

Frequency Offset and/or Guardband

It can be observed from the simulation presented that it is possible to avoid A-MPR for some channel bandwidths by restricting the channel placement. This as the impact of the ECC restriction have the greatest impact at the lower edge of band n3. Hence, by offsetting the channel placement for the aerial UE channel from the lower edge of band n3 the A-MPR is not needed. An illustration of this offset is shown in Figure 4.

Figure 4: Frequency offset of the UAW channel from the lower edge of the band n3.

The minimum required frequency offset from the lower edge of band n3 (Foffset,low) for different channel bandwidths if A-MPR is to be avoided is shown below in Table 2.

Table 2: Frequency offset from the lower edge of band n3 to avoid A-MPR. An empty cell means the A-MPR can’t be avoided even if the channel is at the upper edge of the band.

| Channel Bandwidth [MHz] | Foffset,low [MHz] |
| --- | --- |
| 5 | 8.27 |
| 10 | 16.53 |
| 15 | 24.53 |
| 20 | 32.42 |
| 25 | 40.96 |
| 30 |  |
| 35 |  |
| 40 |  |
| 45 |  |
| 50 |  |

At RAN4#108 the offset values as captured in Table 3 were agreed [11].

Table 3: Tentatively agreed Frequency offset from the lower edge of band n3 to avoid A-MPR.

| Channel Bandwidth [MHz] | Min. Foffset,low [MHz] |
| --- | --- |
| 5 | 10 |
| 10 | 20 |
| 15 | 30 |
| 20 | 40 |
| 25 | 50 |
| 30 | N/A |
| 35 | N/A |
| 40 | N/A |
| 45 | N/A |
| 50 | N/A |

RAN4 shall consider no A-MPR for CBWs 5, 10, 15, 20 and 25 MHz if placed with a minimum frequency offset (Foffset,low) from the lower edge of band n3 as shown in Table 3.

Additionally, at RAN4#108 it was discussed whether a guardband (or frequency restriction) were to be added at the lower end of band B3. In specific there were a suggestion for 540kHz [12]. Due to this we have re-run the simulations with this guardband, and the results are shown in Figure 5 and captured in Annex A.

Figure 5: Examples of the A-MPR when the channel is placed with a guardband of 540kHz to the lower edge of the band n3.

Given the added guardband on this case only will lower the needed A-MPR and not remove the need for it the added specification work needed to specify this/these frequency restrictions it can be argued that it is simpler just to specify the A-MPR, even very high, for also the first RBs which otherwise would fall in the guardband.

Additional specification work is needed to specify guardbands/frequency restrictions.

A-MPR proposal

Given the A-MPR study results, and the suggested differentiate of channel placed with a specific offset to the lower band edge, it suggested to consider the following for A-MPR definition for band n3 under a new NS applicable for Aerial UEs, as further discussed in [9]. This new NS shall be added to 38.101-1 in clause 6.2.3 (UE additional maximum output power reduction) as shown in TP below (Note inclusion to Table 6.2.3.1-1 and Table 6.2.3.1-1A have been omitted).

***** Begin TP with Needed Changes to TS ****

Table 6.2.3.x-1: A-MPR regions for NS_xx (Power Class 3)

| Channel | Carrier Center | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
| Bandwidth, [MHz] | Frequency, Fc, [MHz] | RBstart*12*SCS [MHz] | LCRB*12*SCS [MHz] |  |
| 5 MHz | FC ≤ 1722.5 | ≤ 0.18 | ≤ 2.34 | A1 |
|  |  |  | > 2.34 | A3 |
|  |  | > 0.18, ≤ 0.36 | ≤ 0.36 | A2 |
|  |  |  | > 0.36 | A3 |
|  |  | > 0.36, ≤ 0.72 | < 1.8 | A4 |
|  |  |  | ≥ 1.8 | A3 |
|  |  | > 0.72 | > 1.5*RBstart*12*SCS - 2.5*12*SCS | A4 |
|  |  |  | ≤ 1.5*RBstart*12*SCS - 2.5*12*SCS | A5 |
| 10 MHz | FC ≤ 1735 | ≤ 0.72 | ≤ 1.26 | A1 |
|  |  |  | > 1.26 | A3 |
|  |  | > 0.72, ≤ 3.78 | > 1.5*RBstart*12*SCS - 2.5*12*SCS | A4 |
|  |  |  | ≤ 1.5*RBstart*12*SCS - 2.5*12*SCS | A5 |
|  |  | > 3.78, ≤ 5.04 | > 2.7 | A5 |
|  |  |  | ≤ 2.7 | A6 |
|  |  | > 5.04 | > 0 | A5 |
| 15 MHz | FC ≤ 1747.5 | ≤ 0.72 | ≤ 1.26 | A2 |
|  |  |  | > 1.26 | A3 |
|  |  | > 0.72, ≤ 5.58 | > 1.5*RBstart*12*SCS - 2.5*12*SCS | A4 |
|  |  |  | ≤ 1.5*RBstart*12*SCS - 2.5*12*SCS | A5 |
|  |  | > 5.58, ≤ 9.72 | > 3.78 | A5 |
|  |  |  | ≤ 3.78 | A6 |
|  |  | > 9.72 | > 0 | A5 |
| 20 MHz | FC ≤ 1760 | ≤ 0.72 | ≤ 8.1 | A2 |
|  |  |  | > 8.1 | A4 |
|  |  | > 0.72, ≤ 7.2 | > 1.5*RBstart*12*SCS - 2.5*12*SCS | A4 |
|  |  |  | ≤ 1.5*RBstart*12*SCS - 2.5*12*SCS | A5 |
|  |  | > 7.2, ≤ 12.96 | > 5.58 | A5 |
|  |  |  | ≤ 5.58 | A6 |
|  |  | > 12.96 | > 0 | A5 |
| 25 MHz | FC ≤ 1772.5 | < 0.72 | > 0 | A3 |
|  |  | > 0.72, ≤ 8.64 | > 1.5*RBstart*12*SCS - 2.5*12*SCS | A4 |
|  |  |  | ≤ 1.5*RBstart*12*SCS - 2.5*12*SCS | A5 |
|  |  | > 8.64 | > 6.12 | A5 |
|  |  |  | ≤ 6.12, > 0.72 | A6 |
|  |  |  | ≤ 0.72 | A5 |
| 30 MHz | FC ≤ 1750 | ≤ 24.3 | > 0 | A3 |
|  |  | > 24.3 | > 0 | A5 |
|  | 1750 < FC < 1765 | ≤ 24.3 | > 0 | A5 |
|  |  | > 24.3 | > 0 | A6 |
|  | FC ≥ 1765 | ≥ 0 | > 0 | A5 |
| 35 MHz | FC ≤ 1750 | ≤ 29.16 | > 0 | A3 |
|  |  | > 29.16 | > 0 | A5 |
|  | 1750 < FC < 1765 | ≤ 29.16 | > 0 | A5 |
|  |  | > 29.16 | > 0 | A6 |
|  | FC ≥ 1765 | ≥ 0 | > 0 | A6 |
| 40 MHz | FC ≤ 1740 | ≤ 32.4 | > 0 | A3 |
|  |  | > 32.4 | > 0 | A5 |
|  | FC > 1740 | ≤ 32.4 | > 0 | A5 |
|  |  | > 32.4 | > 0 | A6 |
| 45 MHz | FC ≤ 1740 | ≤ 35.64 | > 0 | A3 |
|  |  | > 35.64 | > 0 | A5 |
|  | FC > 1740 | ≤ 35.64 | > 0 | A5 |
|  |  | > 35.64 | > 0 | A6 |
| 50 MHz | FC ≤ 1740 | ≤ 38.88 | > 0 | A2 |
|  |  | > 38.88 | > 0 | A5 |
|  | FC > 1740 | ≤ 38.88 | > 0 | A5 |
|  |  | > 38.88 | > 0 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.x-2. NOTE 2: For any undefined region, MPR applies. |  |  |  |  |

Table 6.2.3.x-2: A-MPR for NS_xx (Power Class 3)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 30 | 25 | 20 | 15 | 10 | 5 |
|  | QPSK | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 16 QAM | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 64 QAM | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 256 QAM | 30 | 25 | 20 | 15 | 10 |  |
| CP-OFDM | QPSK | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 16 QAM | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 64 QAM | 30 | 25 | 20 | 15 | 10 | 5 |
|  | 256 QAM | 30 | 25 | 20 | 15 | 10 |  |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 |  |  |  |  |  |  |  |

***** End TP ****

RAN4 consider defining A-MPR for n3 due to the ECC requirements via NS_[xx] as shown in the TP above.

The A-MPR regions and values could be further optimized however as evident from the TP above the NS definition is already quite comprehensive. Therefor some simplification can be considered. Currently, the lower channel bandwidths have been split to regions consistent of those defined for LTE band B3 while the larger channel bandwidths have been split to regions based on upper and lower RB allocations.

RAN4 could consider a simplified version of the A-MPR tables for n3 due to the ECC requirements.

A-MPR Study and Proposals for n38

Additional spurious emission limits for UE co-existence are considered according to 38.101-1, as shown in Table 6.5.3.2-1 from 38.101-1. This Table list UE to UE spurious emission requirements. The additional requirement from ECC is however a specific requirement why it shall not be added here but under a NS.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence (n38)

| Frequency range (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 2620 MHz – 2645 MHz | -15.5 | 5 MHz | NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths up to 20 MHz. For channel bandwidth within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. . For carriers overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |
| 2645 MHz – 2690 MHz | -40 | 1 MHz | NOTE 15: See above NOTE 22: See above |

The additional requirement from ECC for n3 shall not be included to Table 6.5.3.2-1 of 38.101-1 but be placed under NS specific for aerial UEs.

A NS applicable for Aerial UE, as further discussed in [9], is to be defined which means this restriction also shall be included in clause 6.5.3.3 (Additional spurious emissions) of 38.101-1 as shown in TP below.

***** Begin TP with Needed Changes to TS ****

6.5.3.3.x 	Minimum requirement (network signalled value “NS_yy”)

When “NS_yy” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.x-x. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.x-x: Additional requirements for “NS_yy”

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20, 25, 30, 40 MHz |  |  |
| 2690 ≤ f ≤ 2900 | -40 | 1 MHz |  |

***** End TP ****

RAN4 shall add the additional requirements for n38 as shown in the TP above to the specification.

Simulation Results

As also presented during previous meetings, as presented in [7] and [8] and now in this meeting we simulated the A-MPR due to the ECC mask in the 2690-2900 MHz range. A-MPR is needed for 40 MHz channel BW, but not for 30 MHz. As the emissions generally reach further with larger channel BWs, we conclude that the channel BWs smaller than 30 MHz will not require A-MPR.

Only the 40 MHz channel BW requires A-MPR, due to the ECC requirements, in the case of n38.

A complete set of results for the 40 MHz channel BW at the upper edge of the band can be found in the Annex B; below we present examples.

Figure 3:  Examples of A-MPR when the center frequency is 2595 MHz. This position is the highest allowed position when network signaling NS_44 is taken into account.

We note that the A-MPR due to the ECC requirement is smaller than the A-MPR granted for NS_44.

The needed A-MPR due to the ECC requirement are smaller than what is already captured under NS_44.

A-MPR proposal

During RAN4#107 it was, based on provided simulation results, agreed that A-MPR already defined for n38 and B38 can be reused as a starting point as it may imposes enough reduction to meet the ECC limits [5]. At RAN4#108 it was agreed to include the requirements of NS_44 when defining A-MPR for arial UEs operating in band n38 [11].

Looking at the already defined NS_44 the A-MPR defined for the 40 MHz channels is sufficient to cover the additional requirements from ECC as shown in the simulation results.

The A-MPR already defined due to the restrictions of NS_44 is sufficient to cover the additional requirements from ECC.

Additionally, NS_44 is also defined due to ECC requirements why it is also applicable for the region where NS_yy for arial UE are to be deployed. Therefore, it is proposed that the A-MPR already defined for NS_44 is made applicable to also NS_yy defined for the arial UEs as shown in the TP below.

***** Begin TP with Needed Changes to TS ****

Table 6.2.3.20-1: A-MPR regions for NS_44 and NS_yy

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2582.5≤ FC ≤ 2602.5 | <18.0 | >max(0, 12*SCS* RBend - 3.6) | A3 |
|  |  | ≥18.0 | <7.2 | A3 |
|  |  | ≥18.0 | ≥7.2 | A6 |
| 30 MHz | 2585 ≤ FC ≤ 2600 | <21.6 | >max(0, 12*SCS* RBend - 3.6) | A3 |
|  |  | ≥21.6 | <12.6 | A3 |
|  |  | ≥21.6 | ≥12.6 | A6 |
| 40 MHz | 2590 ≤ FC ≤ 2595 | ≥0, <2.88 | >0 | A1 |
|  |  | ≥2.88, <14.4 | >max (0, 12*SCS*RBend - 3.6) | A2 |
|  |  | ≥14.4, <23.4 | >10.8 | A3 |
|  |  | ≥23.4, <32.4 | >16.2 | A4 |
|  |  | ≥32.4 | >0 | A5 |

Table 6.2.3.20-2: A-MPR for NS_44 and NS_yy

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | QPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 16 QAM | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 64 QAM | 5 |  | 3 | 7 | 12 | 4 |
|  | 256 QAM | 5 |  |  | 7 | 12 |  |
| CP-OFDM | QPSK | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 16 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 64 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 256 QAM |  |  |  | 8 | 12 |  |

***** End TP ****

RAN4 shall add applicability of the A-MPR defined for NS_44 to also the NS_yy for arial UEs as shown in the TP above to the specification.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: Affected bands of the additional ECC OOBE limits are UL of both LTE and NR bands 3, 7 and 38.

Observation 2: It was agreed at RAN4#108 only to consider A-MPR for LTE and NR bands 3 and 38.

Observation 3: There are existing NSs containing A-MPR defined for band n3 and n38.

Observation 4: It seems no existing NSs for the three bands can be directly reused.

Observation 5: The additional requirement from ECC for n3 shall not be included to Table 6.5.3.2-1 of 38.101-1 but be placed under NS specific for aerial UEs.

Proposal 1: RAN4 to add the additional requirements for n3 as shown in the TP above to the specification.

Figure 1: Examples of the A-MPR when the channel is placed at the lower edge of the band n3.

Observation 6: Very large values of A-MPR, even above 20 dB, are required if the channel is placed at the lower edge of the band n3.

Observation 7: Smaller A-MPR is required for channel BWs from 30 to 50 MHz if the channel is at the upper edge of the band n3.

Observation 8: A-MPR can be avoided for some channel placements of specific channel bandwidths.

Proposal 2: RAN4 shall consider no A-MPR for CBWs 5, 10, 15, 20 and 25 MHz if placed with a minimum frequency offset (Foffset,low) from the lower edge of band n3 as shown in Table 3.

Observation 9: Additional specification work is needed to specify guardbands/frequency restrictions.

Proposal 3: RAN4 consider defining A-MPR for n3 due to the ECC requirements via NS_[xx] as shown in the TP above.

Observation 10: RAN4 could consider a simplified version of the A-MPR tables for n3 due to the ECC requirements.

Observation 11: The additional requirement from ECC for n3 shall not be included to Table 6.5.3.2-1 of 38.101-1 but be placed under NS specific for aerial UEs.

Proposal 4: RAN4 shall add the additional requirements for n38 as shown in the TP above to the specification.

Observation 12: Only the 40 MHz channel BW requires A-MPR, due to the ECC requirements, in the case of n38.

Observation 13: The needed A-MPR due to the ECC requirement are smaller than what is already captured under NS_44.

Observation 14: The A-MPR already defined due to the restrictions of NS_44 is sufficient to cover the additional requirements from ECC.

Proposal 5: RAN4 shall add applicability of the A-MPR defined for NS_44 to also the NS_yy for arial UEs as shown in the TP above to the specification.

- References

- RP-230782, Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)

- RP-230025/R4-2304033, Reply to LS to 3GPP on ECC request for standardisation support related to ECC Decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands”, ETSI TC MSG/TFES

- R4-2308349, Discussion on identification of aerial UEs, Nokia, Nokia Shanghai Bell

- R4-2306642, WF on back off study (MPR/A-MPR) for UAV, Qualcomm Incorporated

- R4-2310487, WF on NR_LTE_UAV, Nokia

- R4-2314699, WF on [108][152] NR_LTE_UAV, Nokia

- R4-2308104, Discussion on power back off for aerial UEs, Nokia, Nokia Shanghai Bell

- R4-2313028, Discussion on power back off for aerial NR UEs, Nokia, Nokia Shanghai Bell

- R4-2316251, On the applicability of NS signalling for aerial Ues, Nokia, Nokia Shanghai Bell

- R4-2316092, Proposal for power back off for aerial LTE UEs, Nokia, Nokia Shanghai Bell

- R4-2314898, ad-hoc minutes for [108][152] NR_LTE_UAV_v3, Nokia, Nokia Shanghai Bell

- R4-2311295, NR UAV OOBE AMPR, Murata Manufacturing Co. Ltd

- Annex A

Note that the results have also been added to the zip-file as .mat files according to the agreed recommendation at RAN4#108 [11].

- Annex B

Note that the results have also been added to the zip-file as .mat files according to the agreed recommendation at RAN4#108 [11].