3GPP TSG-RAN WG4 Meeting # 108-bis	R4-2316099

Xiamen Meeting, October 9th – October 13th, 2023

Title:	TP to TR 37.718-11-11 Addition of DC_14A_n41A

Source:	Nokia, FirstNet

Agenda item:	4.3.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 37.718-11-11 to add DC_14A_n41A

************************************* Start of TP*****************************************

## 6.1.x		DC_14A_n41A

### 6.1.x.1 Configuration for DC

Table 6.1.x.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_14A_n41A | DC_14A_n41A | No |

### 6.1.x.2 Maximum output power for DC

Table 6.1.x.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_14A_n41A | 23 | +2/-3 |

### 6.1.x.3	Spurious emission band UE co-existence for DC

Table 6.1.x.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_14_n41 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.1.x.4	MSD analysis for DC

Table 6.1.x.4-1 lists Band 14 +Band n41 2UL DC up to 5th order harmonics and up to 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.x.4-1: Band 14 and Band n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 788 | 798 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1576 | 1596 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2364 | 2394 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3152 | 3192 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3940 | 3990 | 12480 | 13450 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1698 | 1902 | 3284 | 3488 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1114 | 900 | 4194 | 4592 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4072 | 4286 | 5780 | 6178 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 326 | 102 | 6690 | 7282 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3804 | 3396 | 6568 | 6976 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4860 | 5084 | 8276 | 8868 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9972 | 9186 | 696 | 462 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6494 | 5892 | 2598 | 3016 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10772 | 11558 | 5648 | 5882 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9064 | 9666 | 7356 | 7774 |

Table 6.1.x.4-2 lists up to 5th order harmonics mixing products for dual connectivity between band 14 and band n41,

Table 6.1.x.4-2: Band 14 and Band n41 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 14 | 788 | 798 | 758 | 768 | 1516 | 1536 | 2274 | 2304 | 3032 | 3072 | 3790 | 3840 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

Based on Table 6.1.x.4-1, 6.1.x.4-2,

- There is no UL Harmonic interference.

- There are no intermodulation products that interfere the receive bands.

- There is no Harmonic mixing interference.

### 6.1.x.5	∆TIB and ∆RIB values

For DC_14A_n41A, the TIB,c and RIB value is reused from DC_12A_n41A as in the tables below

Table 6.1.x.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_14_n41 | 0.3 | 0.3 |

Table 6.1.x.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_14_n41 | - | - |

************************************* End of TP*****************************************