# 3GPP TSG-RAN WG4 Meeting # 108bis	                               R4-2316333

# Xiamen, China, Oct. 9 – Oct. 13, 2023

Source:	ZTE Corporation

Title:	TP for TR38.718-03-01_3DL_2UL CA_n34A-n39A-n40A

Agenda Item:	4.11.2

Document for:	Approval

- Introduction

Both BCS0 and BCS 4 and 5 were requested for the configuration of n34A-n39A-n40A, so in this contribution, a TP to TR38.718-03-01is provided to introduce both BCS0 and BCS 4 and 5 for CA_n34A-n39A-n40A. Noted that the fallbacks of 2UL/2DL CA_n34-n39, n39-n40 and CA_n34-n40 have already been included in the spec.

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2),v0.7.0

- Text Proposal

----- Start of TP -----

  - 5.x			n34A-n39A-n40A

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| n34A-n39A-n40A | n34, n39, n40 |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| - n34A-n39A-n40A | CA_n34A-n39A CA_n34A-n40A CA_n39A-n40A | n34 | 5, 10, 15 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
| - n34A-n39A-n40A | CA_n34A-n39A CA_n34A-n40A CA_n39A-n40A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |

      - 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n34-n39-n40, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n34-n39-n40 | - | - | - |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n34-n39-n40 | 0.3 | 0.3 | 0.3 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	UE co-existence studies

For the 2UL/3DL CA_n34A-n39A-n40A in this TP,  simultaneous Rx/Tx operation is not supported for the constitute band, in other words, bands n34, n39 and n40 are synchronized for 2UL/3DL CA_n34A-n39A-n40A configuration. Therefore, No IMD issue for the TDD-TDD 2UL/3DL CA_n34A-n39A-n40.

      - 5.x.2.2	REFSENS requirements

Based on the co-existence there are no IMD issues, so there is no REFSEN exception requirements needed to be defined.

----- End of TP -----