3GPP RAN WG4 Meeting #108bis		R4-2316790

Xiamen, China, October 9-13, 2023

Agenda item:	5.26.1.3

Source:	Apple

Title:	On DMRS bundling with Doppler pre-compensation for NTN

Work Item:	NR_NTN_enh-Core

Document for:	Discussion

# 1	Introduction

The effort to specify NR requirements for non-terrestrial networks continues in Rel-18, with the following objective specifically dedicated to the applicability of the coverage enhancement feature [1]:

The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The following sentence will be revisited in RAN#99 as part of the DL enhancements discussion: “The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density.” No work on this topic will take place in RAN WGs before the discussion on DL enhancements in RAN#99. The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

RAN1 had been discussing how this feature might impact the physical layer design of NTN and reached a working assumpition, which has been shared with RAN4 in an LS for this meeting [2]:

RAN1 has discussed timing error limit (Table 7.1C.2-1 in 38.133), frequency error limit (Section 6.4.1 in 38.101-5), and phase difference limit (Table 6.4.2.5-1 in 38.101-1) for DMRS bundling in NTN case and agreed the following observations and working assumption. Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit Observation For NTN-specific PUSCH DMRS bundling, - In LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed. - Note: this does not imply that UE shall be scheduled within 6 PRBs from the DC carrier. Working assumption For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side. - UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. - FFS: how to determine the actual TDW - FFS: specification impact - Send an LS to RAN4.

This information has also been related to RAN4 via LS in [2].

Additionally, RAN #101 has tasked RAN4 with the following [3]:

RAN tasks RAN4 to discuss and conclude in Oct meeting on - The condition for testing the frequency error requirements to verify NGSO UE pre-compensation functionality for Doppler shift, which will be implemented in the RAN5 NTN specifications. - The condition for RRM uplink timing test cases to verify NGSO UE delay pre-compensation functionality, which can be implemented in RAN5 NTN specifications.

This contribution reviews the RAN1 working assumption, reflects upon the relevance of the recently added RAN task, and suggests next steps for RAN4 to make progress on the topic of DMRS bundling for Rel-18 NTN UE RF requirements.

# 2	Discussion

For terrestrial networks, the UE RF requirements on DMRS bundling are captured in Clause 6.4.2.5 of TS38.101-1 as follows:

#### 6.4.2.5 Phase continuity requirements for DMRS bundling For bands that UE indicates the support of DMRS bundling, when the UE is configured with DMRS bundling, the maximum allowable difference between the measured phase value in any slot p-1 and slot p, or slot 0 and any slot p for each antenna connector shall satisfy the requirements as listed in Table 6.4.2.5-1 for the measurement conditions defined in Table 6.4.2.5-2, within a measurement time window limited by the UE capability of maximum duration for DMRS bundling [maxDurationDMRS-Bundling-r17], and defined for each frequency band separately. The phase value for each slot is measured as shown in Annex F.9. These requirements apply to PUCCH and PUSCH transmissions with DFT-s-OFDM and CP-OFDM waveforms. Table 6.4.2.5-1: Maximum allowable phase difference for DMRS bundling UL channel; Modulation order; Phase difference between any slot p-1 and slot p (NOTE 2); Phase difference between slot 0 and any slot p (NOTE 3). PUSCH; Pi/2 BPSK, QPSK; [25] degrees; [30] degrees. PUCCH; Pi/2 BPSK, BPSK, QPSK. NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for FDD and TDD bands, for supported DMRS bundling configurations ≤ 8 slots. NOTE 3: This requirement applies only for FDD bands, for supported DMRS bundling configurations of 16 slots.. The above requirements are applicable when all the following conditions are met within the measurement time window: - RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated. - Modulation order does not change. - No network commanded TA takes effect. - The TPMI precoder does not change. - There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE. - UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions. - For TDD, no downlink slot(s) or downlink symbol(s) or flexible symbol(s) with/without DL monitoring occasion configured in-between the PUSCH or PUCCH transmissions. Table 6.4.2.5-2: Measurement conditions for the maximum allowable phase difference Parameter; Unit; Level. UE Output Power; dBm; PCMAX,f,c in clause 6.2.4, P-MPR = 0. UE downlink received power; Not change. Operating conditions; Normal conditions. Transmission bandwidth; Confined within FUL_low + [4] MHz and FUL_high – [4] MHz. DL signal frequency; Not change before and during the measurement window. DL signal timing; Maintained constant before and during the measurement window. UL slots for testing; Tested on consecutive UL slots. PUSCH waveform for testing; DFT-s-OFDM.

The side conditions on the terrestrial requirement on DMRS bundling, as currently captured in 38.101-1, specify that no network commanded timing advance takes place.  However, the NTN network requires the UE to apply timing pre-compensation when operating in NTN networks.  RAN4 should discuss how to replace this side condition with the appropriate one for NTN operation.

Observation 1:	RAN4 should update the TA side condition of the maximum allowable phase difference for DMRS bundling requirement to align with the timing pre-compensation procedure used in NTN networks.

As seen in the incoming LS, RAN1 has assumed “pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side” in order for the UE to meet the maximum allowable phase difference requirements for DMRS bundling.  In order to assess the RAN4 specification impact from this decision, RAN4 should further discuss the following:

-	Whether the requirement on maximum allowable phase difference for DMRS bundling with TA pre-compensation has any additional restrictions in terms of max DMRS bundling window length (e.g. fewer than 16 or 8 slots)

-	Whether the requirement on maximum allowable phase difference for DMRS bundling with TA pre-compensation has any additional restrictions in terms of RB allocations (e.g. maximum number of RBs from the DC carrier)

Observation 2:	RAN4 should determine whether additional side conditions on the maximum DMRS bundling length and/or RB allocations are needed for the requirement on the maximum allowable phase difference for DMRS bundling in NTN operation.

Finally, our understanding is that the RAN1 design of the coverage enhancement feature for NTN is based on the same performance metrics as the terrestrial coverage enhancement analysis from Rel-17.  Thus, there is no room to relax or change the requirements on the maximum allowable phase difference, and the values captured in Table 6.4.2.5-1 of TS38.101-1 will remain applicable for NTN.

Observation 3:	It is expected that the values of the maximum allowable phase difference for DMRS bundling captured in Table 6.4.2.5-1 of TS38.101-1 will remain applicable for NTN.

We note that the potential resolution of the RAN task on side conditions for UE RF requirements for Rel-17 NTN is anticipated to include non-zero Doppler and non-zero delay side conditions for the frequency error requirement [4].  Furthermore, if these conditions vary with time, then the UE is expected to adapt the compensation algorithm accordingly.  Thus, generally, some limit on the maximum DMRS bundling length is needed in light of variable Doppler conditions.

Observation 4:	generally, some limit on the maximum DMRS bundling length is needed in light of variable Doppler conditions.  RAN4 should discuss how the Rel-17 decision on Doppler side conditions impacts the NTN DMRS bundling requirement definition and the related capabilities.

Proposal 1:	RAN4 should further discuss the applicability of the DMRS bundling feature to Rel-18 NTN and determine whether additional NTN-specific side conditions are needed.

# 3	Conclusions

This contribution reviews the RAN1 working assumption on DMRS bundling in NTN networks and suggests next steps for RAN4 to make progress on this topic.  The following observations and proposals are made:

Proposal 1:	RAN4 should further discuss the applicability of the DMRS bundling feature to Rel-18 NTN and determine whether additional NTN-specific side conditions are needed.

Observation 1:	RAN4 should update the TA side condition of the maximum allowable phase difference for DMRS bundling requirement to align with the timing pre-compensation procedure used in NTN networks.

Observation 2:	RAN4 should determine whether additional side conditions on the maximum DMRS bundling length and/or RB allocations are needed for the requirement on the maximum allowable phase difference for DMRS bundling in NTN operation.

Observation 3:	It is expected that the values of the maximum allowable phase difference for DMRS bundling captured in Table 6.4.2.5-1 of TS38.101-1 will remain applicable for NTN.

Observation 4:	generally, some limit on the maximum DMRS bundling length is needed in light of variable Doppler conditions.  RAN4 should discuss how the Rel-17 decision on Doppler side conditions impacts the NTN DMRS bundling requirement definition and the related capabilities.

# 4	References

- RP-223534, “NR NTN (Non-Terrestrial Networks) enhancements,” Thales, 3GPP RAN #98, December 2022

- R1-2304094, “LS on PUSCH DMRS bundling for NR NTN coverage enhancement,” 3GPP RAN1 #112bis, April 2023

- RP-232682 “Summary of offline discussions for NTN testing”, RAN WG4 Chair, RAN WG4 Chair, 3GPP TSG-RAN Meeting #101, Bengaluru, India, September 11th – 15th, 2023

- R4-2315395, “Views on NTN testing work for NGSO deployments,” Apple, 3GPP RAN4 #108bis, October 2023