3GPP TSG-RAN WG4 Meeting RAN4#108bis      				R4-2316796 Xiamen , CN

October 9 – 13, 2023

Agenda Item:	5.22.2.6

Source:	Lenovo, Motorola Mobility

Title:	Carrier Frequency Offset Correction for CPP

Document for:  Discussion

- Introduction

The effect of carrier frequency offset on carrier phase measurements was discussed previously in contributions [1,2].  In [2], it was shown that if the frequency offset of the TRP relative to the PRU is different than the frequency offset of the TRP relative to the UE, then the carrier phase difference measurements will have an error which depends on the difference of these carrier frequency offsets and the number of symbols separating the carrier phase measurement.

In [2], it was shown that the mean error in the carrier phase difference measurement due to carrier frequency offsets is given by

n Δφm,UETX- Δφm,PRUTX

where Δφm,UETX and Δφm,UETX denote the phase rotation over one symbol of the carrier of the m-th TRP relative to the UE and the PRU, respectively, and n is the difference in the symbol indices of the two DL-PRS symbols.  Here it is assumed that positioning reference symbol received from the m-th TRP is received n symbols later than the positioning reference symbol received from the l-th TRP as in Figure 1.

Figure 1: TRP m DL-PRS located n = 4 symbols after TRP l DL-PRS

To allow the LMF to correct for this error, it was proposed in [2] that the carrier frequency offsets should be measured by both the UE and the PRU and that these carrier frequency offsets should be reported to the LMF.  Additionally, it was proposed that the UE and the PRU report the number of symbols separating the PRS symbols on which the DL-PRS phase or phase difference measurements were taken.

In this contribution, an alternative approach is proposed for removing the carrier phase offsets due to carrier frequency offsets without the need to report the carrier frequency offsets to the LMF.  With this approach, a common reference time is defined for the phase measurements and the carrier phase offsets due to the carrier frequency offsets in the time interval between the reference time and the DL-PRS measurement are removed from the carrier phase measurements.

- Carrier Frequency Offset

Carrier phase positioning assumes the existence of initial phase offsets among the DL-PRS transmitters, i.e., gNBs, and between the DL-PRS transmitters and the UE and PRU receivers which are static.  In general, however, there will be frequency offsets among the TRP transmitters which will cause the carrier phase offsets to change relative to each other over time.  Similarly, there will be carrier frequency offsets between the TRP transmitters and the UE and PRU receivers which will cause these phase offsets to change over time.

The carrier frequency tolerance for the base station is defined in TS 38.104 and is given in the following two tables:

Table 6.5.1.2-1: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

Table 9.6.1.3-1: OTA frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

The carrier frequency tolerance for the UE is defined in TS 38.101-1 for FR1.  In section 6.4.1 of TS 38.101-1, we have the requirement:

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B.

In [2], it was shown that the mean error in the carrier phase difference measurement between two TRP’s, l and m (in the same subcarrier), due to carrier frequency offsets is given by

n Δφm,UETX- Δφm,PRUTX

where Δφm,UETX and Δφm,UETX denote the phase rotation over one symbol of the carrier of the m-th TRP relative to the UE and the PRU, respectively, and n is the difference in the symbol indices of the two DL-PRS symbols. Here it is assumed that positioning reference symbol received from the m-th TRP is received n symbols later than the positioning reference symbol received from the l-th TRP.

The maximum carrier phase difference measurement as a function of carrier frequency and the number of symbols separating the PRS from the two different TRP’s is shown in Figure 2 below for a subcarrier spacing of 15 kHz.  From Figure 2, it can be observed that the maximum error increases with the carrier frequency and the time between the DL-PRS used to compute the carrier phase difference and that this maximum error can be very large.

An example of the placement of the DL-PRS symbols for three TRP’s is shown in Figure 3 below based on comb-6 configuration. For this example, there are two DL-PRS symbols from each of the three TRP’s in each row (subcarrier).  The distance between the DL-PRS for a pair of TRP’s depends both on the subcarrier selected and the DL-PRS selected within the subcarrier.

Figure 2:  Maximum phase error vs. time between DL-PRS symbol from different TRP’s (in symbols)

Figure 3: Example of a PRB with a comb- 6 positioning reference configuration for three TRP’s

- Correction of Errors Due to Carrier Frequency Offsets

In [2], it was proposed that the carrier phase difference measurement errors due to carrier frequency offsets be corrected at the LMF.  To allow the LMF to correct for these errors, it was proposed in [2] that the carrier frequency offsets should be measured by both the UE and the PRU and that these carrier frequency offsets should be reported to the LMF.  Additionally, it was proposed that the UE and the PRU report the number of symbols separating the PRS symbols on which the DL-PRS phase or phase difference measurements were taken.

A drawback with this approach is the bandwidth required to signal the carrier frequency offsets back to the LMF.  Furthermore, since the carrier frequency offsets will change over time, it may be necessary to report the carrier frequency offsets each time that carrier phase or carrier phase difference measurements are sent to the LMF.  Finally, if carrier phase is reported, then the symbol index must also be reported, and alternatively if carrier phase difference is reported, then the difference in the symbol indices of the positioning reference symbols used to generate the difference must be reported.

  - Phase Measurements Referred to a Common Time Reference

An alternative approach can be considered in which carrier phase measurements are referred to a common reference time as shown in Figure 4 below.  The steps are the following:

- A common reference time for measurements is defined.  In Figure 4 below, the common reference time is the first symbol of the slot.

- The carrier frequency offset is estimated by the UE and the PRU for each TRP for which DL-PRS phase measurements are taken.  Let Δωl,UETX and Δωl,PRSTX denote the carrier frequency offset of the l-th TRP relative to the UE and the PRU, respectively.  The symbol-to-symbol phase rotations are then given by

Δφl,UETX= Δωl,UETX Ts

and

Δφl,PRUTX= Δωl,PRUTX Ts

for the UE and the PRU, respectively.

- The DL-PRS phase measurement is referred to the common reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval since the common reference time.

Let θi,k,l denote the UE’s carrier phase measurement for the DL-PRS received from the l-th TRP in subcarrier i with symbol index k.  The referred carrier phase measurement for the UE is then given by

θi,k,l=modθi,k,l-k Δφl,UETX, 2π  .

Similarly, let Ωi,k,l denote the PRU’s carrier phase measurement for the DL-PRS received from the l-th TRP in subcarrier i with symbol index k.  The referred carrier phase measurement for the PRU is then given by

Ωi,k,l=modΩi,k,l-k Δφl,PRUTX, 2π  .

- The carrier phase difference is computed as the difference between the referred carrier phase measurements.  For the UE, the carrier phase difference between the DL-PRS received from TRP l on subcarrier i with symbol index k and the DL-PRS received from TRP m on subcarrier i with symbol index k+n is defined as

modθi,k,l-θi,k+n,m, 2π .

For the PRU and the same pair of DL-PRS, the carrier phase difference is defined as

modΩi,k,l-Ωi,k+n,m, 2π  .

- Either the referred carrier phase measurements or the differences between these referred carrier phase measurements are reported to the LMF.

In the Appendix, it is shown that by referring the carrier phase measurements to a common reference time, the effect of the carrier frequency offsets are removed so long as the estimates of the carrier frequency offsets are accurate.

An additional advantage of referring the carrier phase measurements to a common reference time is that referred carrier phase measurements for DL-PRS received from the same TRP on the same subcarrier can be averaged. Due to the symbol-to-symbol phase rotations of Δφl,UETX at the UE, carrier phase measurements on DL-PRS which are n symbols apart will have a mean phase offset of n Δφl,UETX and thus should not be averaged without first referring to a common reference time.

In Figure 4 below, an example is provided illustrating how carrier phase measurements can be referred to a common reference time.  In this example, the common reference time for carrier phase measurements is the first symbol of the slot.  The measurement of the DL-PRS in subcarrier i with symbol index 2 from TRP l is referred to the first symbol of the slot by subtracting twice the symbol-to-symbol phase rotation due to its carrier frequency offset, so that

θi,2,l=modθi,2,l-2 Δφl,UETX, 2π

Similarly, the measurement of the DL-PRS in subcarrier i with symbol index 6 from TRP m is referred to the first symbol of the slot by subtracting six times the symbol-to-symbol phase rotation due to its carrier frequency offset and is given by

θi,6,m=modθi,6,m-6 Δφm,UETX, 2π

Figure 4: Example of referring carrier phase measurements to a common reference time

In summary, to correct for carrier phase difference measurement errors due to carrier frequency offsets without the need to signal these carrier frequency offsets and the indices of the measured symbols to the LMU, we have the following proposals:

Proposal 1:	Define a common reference time and refer the DL-PRS carrier phase measurements to this reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval between the DL-PRS and the reference time from the carrier phase measurement.

Proposal 2:	Define the referred carrier phase difference as the difference between the referred carrier phase measurements.

Proposal 3:	Define the same common reference time for the UE and the PRU.

- Summary

To correct for carrier phase difference measurement errors due to carrier frequency offsets without the need to signal these carrier frequency offsets and the indices of the measured symbols to the LMU, we have the following proposals:

Proposal 1:	Define a common reference time and refer the DL-PRS carrier phase measurements to this reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval between the DL-PRS and the reference time from the carrier phase measurement.

Proposal 2:	Define the referred carrier phase difference as the difference between the referred carrier phase measurements.

Proposal 3:	Define the same common reference time for the UE and the PRU.

# References

- R1-2204807, “Design Aspects of Carrier Phase Measurements for NR Positioning”, Intel, #109-e.

- R4-2311991,” Effect of Carrier Frequency Offset on Carrier Phase Positioning,” Lenovo, Motorola Mobility, RAN4#108.

- 3GPP TS 38.104 NR; Base Station (BS) radio transmission and reception

- 3GPP TS 38.101-1 User Equipment radio transmission and reception; Part 1: Range1 Standalone

Appendix 1:  Referred Carrier Phase Measurements

With a carrier frequency offset between the TRP and the PRU, the mean of the measured phase for the TRP will vary with the time index of the PRS within the slot or subframe.  From [2] we have the following background.  Let yi,k,l denote the demodulated DL-PRS symbol received from TRP l on the i-th subcarrier and k-th symbol which is given by

yi,k,l=ri,k,l ej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0+ni,k ,

where ri,k,l denotes the known reference symbol, ωc denotes the carrier frequency (in radians), Δω denotes the subcarrier spacing, Δωl,UETX denotes the frequency offset of the carrier of the l-th TRP relative to that of the UE receiver, τl=dlc denotes the propagation delay corresponding to the distance dl between the l-th TRP and the UE, φUERX0 denotes the initial phase of the UE carrier, φlTX0 denotes the initial phase (for n=0) of the transmitter carrier for the l-th TRP, and ni,k is the additive noise at the output of the demodulator for the k-th symbol of the i-th subcarrier.

The phase increment Δφl,UETX denotes the change in the phase of the carrier of the l-th TRP relative to that of the UE carrier over the period of one symbol and is given by

Δφl,UETX= Δωl,UETX Ts,

where Ts is the symbol period.  The phase term Δωl,UETXπ Δω is due to the frequency offset of the subcarrier with the correlator demodulator and is computed as

argΔω2π02πΔωej Δωl,UETX t dt .

Multiplying yi,k,l by the conjugate ri,k,l* of the known positioning reference symbol for the l-th TRP (assumed to have unit amplitude) yields

wi,k,l=yi,k,l ri,k,l*=ej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0+ri,k,l* ni,k.

The phase of wi,k,l is given by

argwi,k,l=argej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0+ri,k,l* ni,k

=mod-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0+arg1+e-j-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0ri,k,l* ni,k,2π

=mod-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0+ψi,k,l,2π ,

where

ψi,k,l=arg1+e-j-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0ri,k,l* ni,k .

The expected value of argwi,k,l is given by

Eargwi,k,l=mod-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0, 2π

where we have used the fact that the expected value Eψi,k,l=0. The “signal” portion of Eargwi,k,l is given by the term

-ωc+i Δω+Δωl,UETX τl .

It can be noted that the phase terms of Eargwi,k,l are independent of the symbol index k except for the term k Δφl,UETX.  In general, there will be more than one PRS in the slot from the l-th TRP. To coherently combine the phase measurements for the symbols from the l-th TRP, the carrier phase term dependent on the symbol index must be removed from the phase measurement.

Let θi,k,l denote the phase of wi,k,l so that, as above

θi,k,l=argwi,k,l=mod-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0+ψi,k,l,2π ,

where ψi,k,l has zero mean. To remove the dependence of the phase measurement on the symbol index, we define a referred carrier phase measurement as

θi,k,l=modθi,k,l-k Δφl,UETX, 2π  ,

and note that

Eθi,k,l=mod-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0-φUERX0, 2π .

Because θi,k,l is no longer dependent on the symbol index, phase measurements taken on DL-PRS transmitted from the same TRP and in the same subcarrier can be averaged.  For the l0-th TRP, let Ki0, l0 denote the set of symbol indices k for which the PRS are transmitted in subcarrier i0.  With this notation, the phase measurements for TRP l0 and subcarrier i0 can be combined to form an average referred carrier phase measurement for the slot and the subcarrier as

θi0,l0=kεKi0, l0θi0,k,l0Ki0, l0

for which the mean value is the same as before averaging, and thus is also given by

Eθi0,l0=mod-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0-φUERX0, 2π .

Referred Carrier Phase Difference at the UE

For two TRP’s with indices l0 and m0, the UE can compute the carrier phase difference for a subcarrier i0 using either the referred carrier phase measurements θi0,k,l0 and θi0,k,m0, or the average referred carrier phase measurements for the slot given by θi0,l0 and θi0,m0.  In the first case using the referred carrier phase measurements, the carrier phase difference is given as

modθi0,k,l0-θi0,k,m0,2π =modmod-ωc+i0 Δω+Δωl0,UETX τl0+Δωl0,UETXπ Δω+φl0TX0-φUERX0+ψi0,k,l0,2π-mod-ωc+i0 Δω+Δωm0,UETX τm0+Δωm0,UETXπ Δω+φm0TX0-φUERX0+ψi0,k,m0,2π,2π

=modωc+i0 Δωτm0-τl0+Δωl0,UETX τl0-Δωm0,UETX τm0+Δωl0,m0TXπ Δω+φl0TX0-φm0TX0+ψi0,k,l0-ψi0,k,m0,2π ,

where Δωl0,m0TX=Δωl0,UETX-Δωm0,UETX.  The expected value is given by

Emodθi0,k,l0-θi0,k,m0,2π =modωc+i0 Δωτm0-τl0+Δωl0,UETX τl0-Δωm0,UETX τm0+Δωl0,m0TXπ Δω+φl0TX0-φm0TX0,2π

Average Referred Carrier Phase Difference at the UE

In the case of the average phase measurements, the difference is given by

modθi0,l0-θi0,l0,2π =modkεKi0, l0θi0,k,l0Ki0, l0-kεKi0, m0θi0,k,l0Ki0, m0,2π

=mod1Ki0, l0kεKi0, l0mod-ωc+i0 Δω+Δωl0,UETX τl0+Δωl0,UETXπ Δω+φl0TX0-φUERX0+ψi0,k,l0,2π-1Ki0, m0kεKi0, m0mod-ωc+i0 Δω+Δωm0,UETX τm0+Δωm0,UETXπ Δω+φm0TX0-φUERX0+ψi0,k,m0,2π,2π

=modωc+i0 Δωτm0-τl0+Δωl0,UETX τl0-Δωm0,UETX τm0+Δωl0,m0TXπ Δω+φl0TX0-φm0TX0+1Ki0, l0kεKi0, l0ψi0,k,l0-1Ki0, m0kεKi0, m0ψi0,k,m0,2π

The expected value for the average carrier phase difference is the same as for the carrier phase difference and is given by

Emodθi0,l0-θi0,l0,2π ==modωc+i0 Δωτm0-τl0+Δωl0,UETX τl0-Δωm0,UETX τm0+Δωl0,m0TXπ Δω+φl0TX0-φm0TX0,2π

The signal-to-noise ratio of the average referred carrier phase difference is improved relative to that of the referred carrier phase difference due to the averaging of more noise terms. The increase in the signal-to-noise ratio of the average referred carrier phase difference measurement is given approximately by the ratio

Ki0, l0+Ki0, m02 .

Carrier Phase Measurements at the PRU

In the same manner as at the UE, carrier phase measurements are taken at the PRU.  For the PRU, let yi,k,l denote the demodulated DL-PRS symbol received from TRP l on the i-th subcarrier and k-th symbol which is given by

yi,k,l=ri,k,l ej-ωc+i Δω+Δωl,PRUTX τl'+Δωl,PRUTXπ Δω+φlTX0+k Δφl,PRUTX-φPRURX0+ni,k ,

where ri,k,l denotes the known reference symbol, ωc denotes the carrier frequency (in radians), Δω denotes the subcarrier spacing, Δωl,PRUTX denotes the frequency offset of the carrier of the l-th TRP relative to that of the PRU receiver, τl'=dl'c denotes the propagation delay corresponding to the distance dl' between the l-th TRP and the PRU, φPRURX0 denotes the initial phase of the PRU carrier, φlTX0 denotes the initial phase (for n=0) of the transmitter carrier for the l-th TRP, and ni,k is the additive noise at the output of the demodulator for the k-th symbol of the i-th subcarrier.

The phase increment Δφl,PRUTX denotes the change in the phase of the carrier of the l-th TRP relative to that of the UE carrier over the period of one symbol given by

Δφl,PRUTX= Δωl,PRUTX Ts,

where Ts is the symbol period.  The phase term Δωl,PRUTXπ Δω is due to the frequency offset of the subcarrier with the correlator demodulator and is computed as

argΔω2π02πΔωej Δωl,PRUTX t dt .

Multiplying yi,k,l by the conjugate ri,k,l* of the known positioning reference symbol for the l-th TRP (assumed to have unit amplitude) yields

ui,k,l=yi,k,l ri,k,l*=ej-ωc+i Δω+Δωl,PRUTX τl'+Δωl,PRUTXπ Δω+φlTX0+k Δφl,PRUTX-φPRURX0+ri,k,l* ni,k.

Let the phase of ui,k,l be noted by

Ωi,k,l=argui,k,l=mod-ωc+i Δω+Δωl,PRUTX τl'+Δωl,UETXπ Δω+φlTX0+k Δφl,PRUTX-φPRURX0+χi,k,l,2π

where

χi,k,l=arg1+e-j-ωc+i Δω+Δωl,PRUTX τl'+Δωl,UETXπ Δω+φlTX0+k Δφl,PRUTX-φPRURX0ri,k,l* ni,k

and χi,k,l has zero mean.

In the same manner as for the UE, the carrier frequency offset Δωl,PRUTX can be estimated so that symbol dependent phase term k Δφl,PRUTX can be removed.  As a result, we define the referred carrier phase measurement as

Ωi,k,l=modΩi,k,l-k Δφl,PRUTX, 2π  ,

and note that

EΩi,k,l=mod-ωc+i Δω+Δωl,PRUTX τl'+Δωl,UETXπ Δω+φlTX0-φPRURX0, 2π .

In the same manner as for the UE, the phase measurements for TRP l0 and subcarrier i0 can be combined to form an average referred carrier phase measurement for the slot and the subcarrier as

Ωi0,l0=kεKi0, l0Ωi0,k,l0Ki0, l0

for which the mean value is the same as before averaging, and thus is also given by

EΩi0,l0=mod-ωc+i Δω+Δωl,PRSTX τl'+Δωl,PRUTXπ Δω+φlTX0-φPRURX0, 2π .

Referred Carrier Phase Difference at the PRU

For two TRP’s with indices l0 and m0, the PRU can compute the carrier phase difference for a subcarrier i0 using either the referred carrier phase measurements Ωi0,k,l0 and Ωi0,k,m0, or the average referred carrier phase measurements for the slot given by Ωi0,l0 and Ωi0,m0.  In the first case using the carrier phase measurements, the carrier phase difference is given as

modΩi0,k,l0-Ωi0,k,m0,2π =modmod-ωc+i0 Δω+Δωl0,PRUTX τl0'+Δωl0,PRUTXπ Δω+φl0TX0-φPRURX0+χi0,k,l0,2π-mod-ωc+i0 Δω+Δωm0,PRUTX τm0'+Δωm0,PRUTXπ Δω+φm0TX0-φPRURX0+χi0,k,m0,2π,2π

=modωc+i0 Δωτm0'-τl0'+Δωl0,PRUTX τl0'-Δωm0,PRUTX τm0'+Δωl0,m0TXπ Δω+φl0TX0-φm0TX0+χi0,k,l0-χi0,k,m0,2π ,

where Δωl0,m0TX=Δωl0,PRUTX-Δωm0,PRUTX.  The expected value is given by

EmodΩi0,k,l0-Ωi0,k,m0,2π =modωc+i0 Δωτm0'-τl0'+Δωl0,PRUTX τl0'-Δωm0,PRUTX τm0'+Δωl0,m0TXπ Δω+φl0TX0-φm0TX0,2π

Average Referred Carrier Phase Difference at the PRU

In the case of the average phase measurements, the difference is given by

modΩi0,l0-Ωi0,l0,2π =modkεKi0, l0Ωi0,k,l0Ki0, l0-kεKi0, m0Ωi0,k,l0Ki0, m0,2π

=mod1Ki0, l0kεKi0, l0mod-ωc+i0 Δω+Δωl0,PRUTX τl0'+Δωl0,PRUTXπ Δω+φl0TX0-φPRURX0+χi0,k,l0,2π-1Ki0, m0kεKi0, m0mod-ωc+i0 Δω+Δωm0,PRUTX τm0'+Δωm0,PRUTXπ Δω+φm0TX0-φPRURX0+χi0,k,m0,2π,2π

=modωc+i0 Δωτm0'-τl0'+Δωl0,PRUTX τl0'-Δωm0,PRUTX τm0'+Δωl0,m0TXπ Δω+φl0TX0-φm0TX0+1Ki0, l0kεKi0, l0χi0,k,l0-1Ki0, m0kεKi0, m0χi0,k,m0,2π

The expected value for the average carrier phase difference is the same as for the carrier phase difference and is given by

EmodΩi0,l0-Ωi0,l0,2π ==modωc+i0 Δωτm0'-τl0'+Δωl0,UETX τl0'-Δωm0,UETX τm0'+Δωl0,m0TXπ Δω+φl0TX0-φm0TX0,2π .

Appendix 2: Estimating Differential Distance Using Referred Carrier Phase Measurements

Using Referred Carrier Phase Difference Measurements

From above, we have that the expected value of the carrier phase difference at the PRU is given by

EmodΩi0,k,l0-Ωi0,k,m0,2π =modωc+i0 Δωτm0'-τl0'+Δωl0,PRUTX τl0'-Δωm0,PRUTX τm0'+Δωl0,m0TXπ Δω+φl0TX0-φm0TX0,2π .

The delays τm0' and τl0' between the TRP’s and the PRU are known to the LMF, so the phase term ωc+i0 Δωτm0'-τl0' is known.  Using the PRU’s estimate of the frequency offsets Δωl0,PRUTX and Δωm0,PRUTX, the term Δωl0,PRUTX τl0'-Δωm0,PRUTX τm0' can also be estimated by the PRU (Note that the term Δωl0,m0TXπ Δω can be explicitly cancelled separately at the UE and the PRU from the frequency offset estimates Δωl0,UETX and Δωl0,PRUTX. If not, this term will cancel in combination with the transmitter offset term as below).  With this, note that

modEmodΩi0,k,l0-Ωi0,k,m0,2π -ωc+i0 Δωτm0'-τl0'-Δωl0,PRUTX τl0'+Δωm0,PRUTX τm0',2π=modφl0TX0-φm0TX0+Δωl0,m0TXπ Δω,2π

so that the transmitter offset can be estimated as

modφl0TX0-φm0TX0+Δωl0,m0TXπ Δω,2π=modΩi0,k,l0-Ωi0,k,m0-ωc+i0 Δωτm0'-τl0'-Δωl0,PRUTX τl0'+Δωm0,PRUTX τm0',2π.

With this estimate of the difference in the transmitter phase offsets, the differential distance between the two TRP’s and the UE can be estimated.

From the UE phase difference measurement, we have

Emodθi0,k,l0-θi0,k,m0,2π =modωc+i0 Δωτm0-τl0+Δωl0,UETX τl0-Δωm0,UETX τm0+Δωl0,m0TXπ Δω+φl0TX0-φm0TX0,2π .

From this, the term

modωc+i0 Δωτm0-τl0+Δωl0,UETX τl0-Δωm0,UETX τm0,2π

can be estimated as

modmodθi0,k,l0-θi0,k,m0,2π -modΩi0,k,l0-Ωi0,k,m0-ωc+i0 Δωτm0'-τl0'-Δωl0,PRUTX τl0'+Δωm0,PRUTX τm0',2π,2π

for which the expected value is given by

Emodmodθi0,k,l0-θi0,k,m0,2π -modΩi0,k,l0-Ωi0,k,m0-ωc+i0 Δωτm0'-τl0'-Δωl0,PRUTX τl0'+Δωm0,PRUTX τm0',2π,2π

=modωc+i0 Δωτm0-τl0+Δωl0,UETX τl0-Δωm0,UETX τm0,2π

As shown in [2,3], the term Δωl0,UETX τl0-Δωm0,UETX τm0 is very small and can be ignored. Specifically, for a carrier frequency of 5 GHz with a frequency error of 0.1ppm and with the assumption of a maximum range of 200 meters, this term can be bounded by

Δωl0,UETX τl0-Δωm0,UETX τm0≤Δωl0,UETX τl0+Δωm0,UETX τm0

≤Δωl0,UETX τl0+Δωm0,UETX τm0

≤2 π ∙ 5∙109∙10-7∙200+2003∙108=0.004 radians=0.24 degrees ,

and as a result, this term can be ignored.  With this observation, we have

Emodmodθi0,k,l0-θi0,k,m0,2π -modΩi0,k,l0-Ωi0,k,m0-ωc+i0 Δωτm0'-τl0'-Δωl0,PRUTX τl0'+Δωm0,PRUTX τm0',2π,2π

≈modωc+i0 Δωτm0-τl0,2π

and this can be used to estimate τm0-τl0 if the number of integer cycles between the TRP’s and the UE are known.

Using Average Referred Carrier Phase Measurements

The differential distance τm0-τl0 can be estimated using the average carrier phase difference measurements in the same manner as is used with the carrier phase difference measurements.  Specifically, the term

modωc+i0 Δωτm0-τl0+Δωl0,UETX τl0-Δωm0,UETX τm0,2π≈modωc+i0 Δωτm0-τl0,2π

can be estimated as

modmodθi0,l0-θi0,m0,2π -modΩi0,l0-Ωi0,m0-ωc+i0 Δωτm0'-τl0'-Δωl0,PRUTX τl0'+Δωm0,PRUTX τm0',2π,2π .