3GPP TSG-RAN WG4 Meeting #108-bis                                                                                     R4-2316908

Xiamen, CN, Oct. 9 – Oct. 13, 2023

Source: 	CATT

Title: 	WF for NCR RF conformance test

Agenda item:	5.28.7

Document for:	Approval

- Introduction

This contribution captures the WF for NCR RF conformance test.

- Agreements

Issue 1-1: Mixed type introduction

- Agreement:

  - The Mixed type of NCR will not be introduced in Rel-18.

Issue 3-1: Test configuration

- For the joint UL test configuration following is consider:

  - Option 1:

    - Place an NCR-MT carrier at the lower end of each passband Generate an NR carrier using test equipment at the upper edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband.

    - Place an NCR-MT carrier at the upper end of each passband. Generate an NR carrier using test equipment at the lower edge of each passband, and a second NR carrier adjacent to the NCR-MT carrier within each passband. For each passband, if there is insufficient space for the NR carriers then remove firstly the NR carrier adjacent to the NCR-MT carrier and then if needed the NR carrier at the upper end of the passband.

  - Other Options are not precluded.

- References

[1] R4-2315599, Discussion on introduction of mix type for NCR-Fwd, NTT DOCOMO, INC

[2] R4-2315807,	Discussion of NCR conformance testing, Nokia, Nokia Shanghai Bell

[3] R4-2315093,	Discussion on RF conformance testing for NCR, CATT

[4] R4-2315603,	NCR conformance considerations, Ericsson

[5] R4-2316543,	Discussion on conformance testing requirement for NCR, ZTE Corporation