3GPP TSG-RAN WG4 Meeting#108bis                              R4-2316996

Xiamen, China, Oct 09 – Oct 13, 2023

Agenda item:	5.28.7

Source: 	ZTE Corporation

Title: 	WF on NCR RF requirement

Document for:	Approval

Agreement reached online for NCR-Fwd type 1-H

Issue 2-1-1 Repeater output power

DL direction:

- For wide area and ,medium range NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells)

- For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

UL direction:

- For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8) ;

- For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4) ;

- For uplink a note to indicate LA MT cannot exceed highest power class for that band

- NCR-MT can also use the scaling factors proposed here.

Issue 2-1-2 Adjacent Channel Leakage Power Ratio

DL direction:

- For wide area and ,medium range NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells)

- For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

UL direction:

- For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8) ;

- For local area NCR-fwd link, without scaling factor ;

Issue 2-1-3 Operating band unwanted emissions

DL direction:

- For wide area and ,medium range NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells)

- For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

UL direction:

- For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8) ;

- For local area NCR-fwd link, without scaling factor ;

Issue 2-1-4-1  Transmitter spurious emission requirement [Scaling factor]

DL direction:

- For wide area and ,medium range NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells)

- For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

UL direction:

- For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8) ;

- For local area NCR-fwd link, without scaling factor ;

Issue 2-1-4-1a Transmitter spurious emission [for the simultaneous transmission of NCR-Fwd and NCR-MT]

- for the simultaneous transmission of NCR-Fwd and NCR-MT part in FR2, propose to reuse the UE transmitter spurious emission requirement;

- Issue 2-1-4-2 Transmitter UEM requirement [for the simultaneous transmission of NCR-Fwd and NCR-MT]

- For the joint transmission between NCR-MT and NCR-Fwd, the UE requirement should be applicable for emission and spurious requirement.

Issue 2-1-5 Receiver spurious emissions

For DL direction (receiver spurious emission requirement) ;

- NTXU,counted = min(NTXU,active , 8*Ncells) for Wide area and Medium range NCR-Fwd

- NTXU,counted = min(NTXU,active , 4*Ncells) for local area  NCR-Fwd

For UL part (receiver spurious emission requirement):

- For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8) for emission requirement only;

- For local area NCR-fwd link, without scaling factor;

Agreement reached online for NCR-Fwd type 1-O

For scaling factor for NCR-Fwd type 1-O:

- For the scaling factors for NCR Fwd link type 1-O, follow same principle as NCR Fwd link type 1-H

Issue 2-2-6 Scaling factor for OTA EVM and OTA input intermodulation

- The EVM minimum input level is (conducted level) – (Sub-array gain based on input RoAoA)

Recommended Agreement for NCR-MT

For scaling factor for NCR-MT type 1-H and 1-O:

- For Wide area NCR-MT type 1-H and 1-O, the scaling factors applies and follow the NCR-Fwd type 1-H and 1-O principle;

- For Local area NCR-MT type 1-H and 1-O, don’t apply the scaling factors on top of basic emission requirements;

Simultaneous reception

- Propose to test individual reception and not simultaneous

- [Separate requirement for NCR-MT and NCR-Fwd]

- Consider the simultaneous reception with input intermodulation requirement only.

Issue 3-2-1    REFSENS requirement

- Proposals

  - for Local area NCR-MT REFSENS ,propose to use the legacy UE requirement;

Issue 2-2-6 Scaling factor for OTA input intermodulation

To follow with previous agreement with whole antenna array.

Open for further discussions

Issue 2-3-1 relationship mapping between input connectors and output connectors

  - Option 1: to declare the relationship between input connectors and output connector for NCR-Fwd type 1-H

  - Option 2: inject the signals on all input connectors and conduct the measurement at all output connectors.

Agreement:

Fix the the terminology for Rel-17 repeater.

Baseline agreement for NCR-MT feature list

- NR Rel-15 UE feature

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional | NCR-MT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. System parameter | 1-1 | 60kHz of subcarrier spacing for FR1 | 60kHz subcarrier spacing for data channel in FR1 |  | scs-60kHz | Phy-ParametersFR1 | No | Applicable only to FR1 |  | Optional with capability signalling | Optional with capability signalling |
|  | 1-2 | 64QAM modulation for FR2 PDSCH | 64QAM modulation for FR2 PDSCH |  | n/a | n/a | No | Applicable only to FR2 | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | Mandatory without capability signalling | Optional with capability signalling |
|  | 1-3 | 64QAM for PUSCH | 64QAM for PUSCH |  | n/a | n/a | No | No | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | Mandatory without capability signalling | [mandatory without capability signalling] |
|  | 1-4 | 256QAM for PDSCH | 256QAM for PDSCH |  | pdsch-256QAM-FR1 | Phy-ParametersFR1 | No | Yes | For FR1, it can be revisited in the future whether the 256QAM is mandated in all UE types or categories | Mandatory with capability signalling for FR1 | Optional with capability signalling for FR1 |
|  |  |  |  |  | pdsch-256QAM-FR2 | BandNR |  |  | For FR2, RAN4 agreed that no BS and UE requirements will be introduced in Rel.15. | Optional with capability signalling for FR2 | Optional with capability signalling for FR2 |
|  | 1-5 | 256QAM for PUSCH | 256QAM for PUSCH |  | pusch-256QAM | BandNR | No | Yes | For FR1, RAN4 can further discuss to mandate 256QAM for PUSCH for FR1 in future release. For FR2, RAN4 agreed that no BS and UE requirements will be introduced in Rel.15. | Optional with capability signalling (for both FR1 and FR2) | Optional with capability signalling (for both FR1 and FR2) |
|  | 1-6 | pi/2-BPSK for PUSCH | pi/2-BPSK for PUSCH |  | pusch-HalfPi-BPSK | Phy-ParametersFRX-Diff | No | Yes | RAN4 will define the same minimum requirements for pulse-shaped pi/2 BPSK and non-pulse shaped pi/2 BPSK for FR2. | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 1-7 | pi/2-BPSK for PUCCH format 3/4 | pi/2-BPSK for PUCCH format 3/4 |  | pucch-F3-4-HalfPi-BPSK | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 1-8 | Active BWP switching delay | Support of active BWP switching delay specified in TS38.133, candidate values set: {type1, type2} |  | bwp-SwitchingDelay | Phy-ParametersCommon | No | No | For this feature, RAN4 also sent another LS (R4-1803283). Network cannot configure the shorter delay for certain UE type. | Mandatory to support either type 1 or type 2 with capability signalling | Optional with capability signalling |
|  | 1-9 | Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective | 1) LTE and NR UL Transmission in the shared carrier via TDM only 2) LTE and NR UL Transmission in the shared carrier via FDM only 3) LTE and NR UL transmission in the shared carrier via FDM or TDM |  | ul-SharingEUTRA-NR | MRDC-Parameters | No | Applicable only to FR1 |  | Optional with capability signalling | not applicable |
|  | 1-10 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Support of switching type between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective. Type 1: <0.5us Type 2: <20us | 1-9 | ul-SwitchingTimeEUTRA-NR | MRDC-Parameters | No | Applicable only to FR1 | This feature is the switching time between LTE UL and NR UL in the same carrier Per band combination signalling UE Capability signalling elements. 1: <0.5us switching type. 2: <20us switching type. | Mandatory to support either type 1 or type 2 with capability signalling if UE reports its capability in 1-10 as 1) LTE and NR UL Transmission in the shared carrier via TDM only, or 3) LTE and NR UL transmission in the shared carrier via FDM or TDM | not applicable |
|  | 1-11 | 7.5kHz UL raster shift | 7.5kHz UL raster shift |  | n/a | n/a | No | No |  | Mandatory in the SUL bands with uplink sharing either from UE perspective or from network perspective 7.5KHz raster shift as mandatory without capability signalling. 7.5kHz UL raster shift is mandatory for the bands described in clause 5.4.2.1 of Release 15 TS 38.101-1. RAN4 can revisit the above bands in the future release. 7.5KHz raster shift is not mandatory for other LTE refarming band except the bands which were agreed to support 7.5kHz UL raster shift as mandatory | not applicable |
| 2. UE RF | 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC | 1) FR1 channel bandwidths in TS38.101-1 Table 5.3.5-1 2) FR2 channel bandwidths in TS38.101-2 Table 5.3.5-1 |  | channelBWs-DL channelBWs-UL | BandNR | No | No | UE capability signalling shall follow RP-172832 (Per-band capability signalling, separately for DL and UL and for each SCS) Whether a bandwidth newly introduced in future is mandatory for UE shall be discussed case by case. | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC. The bandwidths listed in the slide #3 of R4-1805985 are mandatory with a single CC. 90MHz is optional for n41, n77, n78. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. | All BW should be optional instead of mandatory for largest channel bandwidth per-band |
|  |  |  |  |  | supportedBandwidthDL channelBW-90mhz | FeatureSetDownlinkPerCC |  |  |  |  |  |
|  |  |  |  |  | supportedBandwidthUL channelBW-90mhz | FeatureSetUplinkPerCC |  |  |  |  |  |
|  | 2-2 | Simultaneous reception or transmission with same or different numerologies in CA | Support of simultaneous reception or transmission with same or different numerologies in CA |  | supportedSubcarrierSpacingDL | FeatureSetDownlinkPerCC | No | No | From RAN4 perspective UE shall be able to signal the supported SCS per CC for each band combination Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory support for Rel15 The capability of supporting SCS within the single carrier in the CA configuration will be signalled separately, i.e., there is no need to mandatory UE to support mixed numerologies in CA case If a UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s), the UE shall support two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL with capability signalling. | Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory with capability in both FR1 and FR2. Two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL are mandatory with capability if UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s). Optional for other cases. | Not applicable |
|  |  |  |  |  | supportedSubcarrierSpacingUL | FeatureSetUplinkPerCC |  |  |  |  |  |
|  | 2-3 | Non-contiguous intra-band CA frequency separation class for FR2 | 1) Support of frequency separation classes to handle the total frequency span for DL for intra-band non-contiguous CA 2) Support of frequency separation classes to handle the total frequency span for UL for intra-band non-contiguous CA |  | intraBandFreqSeparationDL | FeatureSetDownlink | No | Applicable only to FR2 | UE signals the supported Frequency separation classes with per band granularity (Type 1) based on R4-1803363 Separate Frequency separation classes can be signalled for DL and UL | Mandatory to support a frequency separation class within {I, II, III} specified in TS38.101-2 with capability if UE supports non-contiguous CA in FR2 | Not applicable |
|  |  |  |  |  | intraBandFreqSeparationUL | FeatureSetUplink |  |  |  |  |  |
|  | 2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) |  | simultaneousRxTxInterBandENDC | MRDC-Parameters | No | No | For TDD-FDD and TDD-TDD band combinations for which simultaneous RxTx capability is agreed to be supported, corresponding capability indication must be set to "supported". Band combinations for which simultaneous RxTx capability is mandatory EN-DC combinations (Both FR1 LTE – FR1 NR and FR1 LTE- FR2 NR) are captured in TS 38.101-3. | Mandatory/Optional support depends on band combination and captured in TS 38.101-3 | Not applicable |
|  | 2-5 | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) |  | simultaneousRxTxInterBandCA | CA-ParametersNR | No | No | For TDD-FDD and TDD-TDD band combinations for which simultaneous RxTx capability is agreed to be supported, corresponding capability indication must be set to "supported". Band combinations for which simultaneous RxTx capability is mandatory are captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3. | Mandatory/Optional support depends on band combination and captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3 | not applicable |
|  | 2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | Asynchronous FDD-FDD intra-band EN-DC |  | asyncIntraBandENDC | MRDC-Parameters | Applicable only to FDD | Applicable only to FR1 |  | Optional with capability signalling | Not applicable |
|  | 2-7 | Almost contiguous UL CP-OFDM | Support of almost contiguous UL CP-OFDM transmissions |  | almostContiguousCP-OFDM-UL | Phy-ParametersFRX-Diff | No | Yes | RAN4 had defined the requirements for "Almost contiguous UL CP-OFDM" in Rel-15. | Optional with capability signalling | Optional with capability signalling |
|  | 2-8 | UE power class | 1) Support of FR1 UE power class 2) Support of FR2 UE power class 3) Support of FR1 UE power class for EN-DC 4) Support of FR1 UE power class for NR-CA |  | ue-PowerClass | BandNR | No | No | Capability signalling - FR1 UE power class (per band) - FR2 UE power class (per band) - FR1 UE power class for EN-DC (per band combination) - FR1 UE power class for NR CA (per band combination) Default power class for each component is indicated in TS38.101-1/2/3. If the default power class is not indicated, UE shall report supported power class. The component 2) is also used as power class for intra-band NR-CA in FR2 | Mandatory to support at least one power class with capability. The capability signalling is absent if UE supports only default power class | not applicable |
|  |  |  |  |  | powerClass | BandCombination |  |  |  |  |  |
|  | 2-9 | Simultaneous reception and transmission for SA SUL band combinations | Simultaneous reception and transmission for SA SUL band combinations |  | simultaneousRxTxSUL | CA-ParametersNR | No | No |  | Mandatory/Optional support depends on band combination and captured in TS 38.101-1 | Not applicable |
|  | 2-10 | Multiple frequency band indication | Multiple frequency band indication |  | n/a | n/a | No | No | Per UE capability | Mandatory without capability signalling | The same as UE |
|  | 2-11 | Modified MPR behaviour | Modified MPR behaviour |  | modifiedMPR-Behaviour | BandNR | No | No | Per band capability | Optional with capability signalling | not applicable |
|  | 2-12 | Multiple NS/P-Max | Multiple NS/P-Max |  | n/a | n/a | No | No | Per UE capability | Mandatory without capability signalling | Optional |
|  | 2-13 | Maximum uplink duty cycle for FR1 power class 2 UE | Maximum percentage of uplink symbols can be scheduled within a certain evaluation period provided by regulatory bodies. The value range is {60%, 70%, 80%, 90%, 100%}. If the field is absent, 50% shall be applied. |  | maxUplinkDutyCycle-PC2-FR1 | BandNR | No | Applicable only to FR1 | Per band capability. If this capability is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%, or this capability is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than this capability, apply all requirements for the default power class. The evaluation period is up to UE implementation, no less than one radio frame. UE do not need to do UL duty cycle calculation when it's transmit power is below 23dBm and all the UL/DL configurations can be scheduled. | Optional with capability signalling. The capability signalling is absent if UE supports 50% | Not applicable |
|  | 2-14 | Power boosting for Pi/2 BPSK for power class 3 UE | Power boosting for Pi/2 BPSK for power class 3 UE in TDD bands n40, n77, n78 and n79 with duty cycle less than 40% | 1-6, 1-7 | powerBoosting-pi2BPSK | BandNR | Applicable only to TDD | Applicable only to FR1 | Per band capability | Optional with capability signalling | Optional with capability signalling |
|  | 2-15 | Maximum uplink duty cycle for FR2 | 1) Maximum percentage of uplink transmission time that can be scheduled within 1s time window in order to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. The value range is {15%, 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%}. |  | maxUplinkDutyCycle-FR2 | BandNR | No | Applicable only to FR2 | Per band capability. If the field of UE capability is present and the percentage of uplink symbols transmitted within any 1 s evaluation period is larger than this capability, the UE follows the uplink scheduling and can apply P-MPR as in TS38.101-2. If the field of UE capability is absent, the compliance to electromagnetic power density exposure requirements are ensured by means of scaling down the power density or by other means. This capability is applicable for all power classes in FR2 | Optional with capability signalling | Not applicable |
|  | 2-16 | PA architectures for intra-band EN-DC | Support of dual PA |  | dualPA-Architecture | MRDC-Parameters | No | No | Per band per band combination capability Single PA is default architecture The following requirements are involved by this capability - A-MPR/MPR and MSD values for dual uplink. Whether two sets of requirements will be introduced in RAN4 can be further discussed for each specific band combination - Switching time between LTE UL and NR UL in single switched UL operation mode for intra-band EN-DC | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band EN-DC configuration in uplink. The capability signalling is absent if UE supports single PA architecture. | Not applicable |
|  | 2-17 | PA architectures for intra-band UL CA | Support of dual PA |  | dualPA-Architecture | CA-ParametersNR-v1540 | No | No | Per band per band combination capability Single PA is default architecture The following requirements are involved by this capability - A-MPR/MPR and MSD values for dual uplink. Whether two sets of requirements will be introduced in RAN4 can be further discussed for each specific band combination | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band CA configuration in uplink. The capability signalling is absent if UE supports single PA architecture | Not applicable |
| 3. Baseband | 3-1 | Independent measurement gap configurations for FR1 and FR2 | Measurement gaps for FR1 and FR2 are configured independently. |  | independentGapConfig | MeasAndMobParametersMRDC-Common | No | No |  | Optional with capability signalling | Optional |
|  | 3-2 | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement |  | simultaneousRxDataSSB-DiffNumerology | MeasAndMobParametersFRX-Diff MeasAndMobParametersMRDC-FRX-Diff | No | Yes |  | Optional with capability signalling | Optional |
|  | 3-3 | Short measurement gap | Measurement gap patterns with short MGL (gap pattern#2, 3, 6, 7, 8, 10) are supported for E-UTRAN measurement. Gap patterns #6, 7, 8, 10 only apply to E-UTRAN measurement when MO includes both E-UTRAN and NR. |  | supportedGapPattern | MeasAndMobParametersCommon | No | No | Per UE capability This capability is signalled as a part of supportedGapPattern in TS38.306. | Optional with capability signalling | Optional |
|  | 3-4 | SU-MIMO Interference Mitigation advanced receiver | 1) R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas. 2) R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas. |  | n/a | n/a | No | No | UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 | Optional without capability signalling | Optional |

- NR Rel-16/17 UE feature

Baseline Agreement:

All of R16 and R17 features are optional for NCR-MT except for CA, DC related feature , unlicensed band and, HPUE Duty cycle,  MPR related feature which is not applicable for NCR-MT.