3GPP TSG-RAN WG4 Meeting #108bis	R4-2317337

Xiamen, China, 09th ‒ 13th October, 2023

Title: 	WF on support for bandwidth part operation without restriction

Agenda Item:	5.10.3

Source: 	vivo

Document for:	Approval

- Topic #1: General aspects

  - Sub-topic 1-1: Requirements for combined options

#### Issue 1-4: Requirements/UE behaviour for UE supporting both option B-1-1 and A

- FFS

  - Option 1:

    - There is no obvious gain if option A and option B-1-1 are supported simultaneously.

    - For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is NOT expected to perform RLM/BFD/BM based on CD-SSB outside active BWP.

  - Option 2:

    - For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

  - Option 3:

    - For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define requirements/ UE behaviour.

  - Option 4:

    - RAN4 does not need to define additional requirements or clarify UE behaviour for the UE supporting options B-1-1 and A

  - Option 5:

    - RAN4 not to define additional requirements or clarify UE behaviour for UE supporting multiple options.

#### Issue 1-5: Requirements/UE behaviour for UE supporting both option C and A

- FFS

  - Option 1:

    - For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, there is no need to define requirements/ UE behaviour.

  - Option 2:

    - For UE supporting both option C and A, both CSI-RS and NCD-SSB within the active BWP can be configured for L1 measurements.

  - Option 3:

    - RAN4 does not need to define additional requirements or clarify UE behaviour for the UE supporting options C and A.

  - Option 4:

    - For UE supporting both option B-1-1 and C, RAN4 shall discuss which SSB UE shall measure when active BWP does NOT contain neither CD-SSB nor NCD-SSB. Candidate solutions:

      - Alt 1: Leave it to network control.

      - Alt 2: UE always measures CD-SSB unless the active BWP contains NCD-SSB.

  - Option 5:

    - RAN4 not to define additional requirements for UE supporting multiple options.

#### Issue 1-6: Reporting behaviour for UE supporting both option B-1-1 and B-1-2

- FFS

  - Option 1:

    - A UE shall not indicate support of both option B-1-1 and option B-1-2 per BC.

#### Issue 1-7: LS to RAN2 on RAN4 conclusions

- Conclusions captured in issues 4-6 and 3-4.

- Topic #2: RRM Core requirements

  - Sub-topic 2-1: Requirements for Option A

#### Issue 2-2: Any clarification on existing timing requirements when CD-SSB is outside active BWP for option A

- It is RAN4 common understanding that timing requirements apply when the SSB is within the channel BW of the UE.

  - Sub-topic 2-2: Requirements for Option B-1-1

#### Issue 3-3: Clarification on existing timing requirements for Option B-1-1

- It is RAN4 common understanding that timing requirements apply when the SSB is within the channel BW of the UE.

#### Issue 3-4: Clarification on reporting of NeedForGap/NeedForGapNCSG for UE supporting option B-1-1

- For UE capable of supporting Option B-1-1 capability and additionally supporting NeedForGap or NeedForGapNCSG or NeedForInterruption”

  - UE shall report no gap and no interruption/no NCSG for intra-frequency measurement.

  - Sub-topic 2-3: Requirements for Option C

#### Issue 4-1: Principle for specifying handover requirements for Option C

- FFS

  - Option 1:

    - Existing handover requirements for RedCap UE with 2Rx are reused as baseline. Necessary changes can be made.

  - Option 2:

    - Existing handover requirements and handover scenarios of RedCap UE with 2Rx can be applied to option C.

  - Option 3:

    - The existing handover requirements defined in clause 6.1.1 (NR Handover) of TS 38.133 are also applicable for the HO when the NCD-SSB is in the active DL BWP of the source cell and the NCD-SSB is in the active DL BWP of the target cell.

#### Issue 4-2: Definition of intra-frequency handover and inter-frequency handover

- Definition of intra-frequency handover is introduced for non-RedCap and it is to reuse the definition for handover in RedCap.

#### Issue 4-3: Scenarios of handover for Option C

<Online agreement>

- Intra frequency handover from NCD-SSB to NCD-SSB

- Inter frequency handover form NCD-SSB to CD-SSB

- Inter frequency handover from CD-SSB to NCD-SSB

- Inter frequency handover from NCD-SSB to NCD-SSB

#### Issue 4-3-1: Requirements for applicable scenarios for handover for Option C

- For UE supporting option C, the handover requirements are applicable for the scenarios

  - Handover to the first active DL BWP of the target cell associated with CD-SSB

  - Handover to the first active DL BWP of the target cell associated with NCD-SSB.

#### Issue 4-4: Tsearch in handover requirements for known inter-frequency cell

- FFS

  - Option 1:

    - For UE supporting option C, the handover requirements for known inter-frequency cell are specified as follows.

      - if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0ms;

      - if the measured SSB and the target SSB for handover belong to the same NR target cell and fulfil the following conditions, Tsearch = Trs ms:

        - CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in first active DL BWP is the target SSB for handover

        - NCD-SSB in DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for handover

        - Both measured SSB and the target SSB for handover are NCD-SSB within different DL BWPs

  - Option 2:

    - For known inter-frequency target cell, three cases are included and Tsearch is N*Trs ms.

      - The measured SSB is a CD-SSB and the target SSB for HO is an NCD-SSB

      - The measured SSB is an NCD-SSB and the target SSB for HO is a CD-SSB

      - The measured SSB is an NCD-SSB and the target SSB for HO is another NCD-SSB

#### Issue 4-5: Tsearch in handover requirements for other cases than known inter-frequency cell

- FFS

  - Option 1:

    - Reuse the requirements for RedCap UE with 2Rx.

#### Issue 4-6: CA requirements for UE supporting option C

- The following scenario is supported from RAN4 requirement perspective:

  - NCD-SSB + CA, NCD-SSB only in PCell

- Inform RAN1/2 the above agreement

  - Sub-topic 2-4: Requirements for Option B-1-2

#### Issue 5-2a: Interruption length (in ms) for option B-1-2

- FFS

  - Option 1:

    - Interruption length is 0.5ms for FR1 and 0.25ms for FR2.

  - Option 2:

    - RAN4 shall define the interruption length requirements the same as these defined for NCSG in Rel-17 (i.e., VIL=1 ms in FR1 and VIL=0.75 ms in FR2).

#### Issue 5-2b: Interruption length requirements (in number of slots) for option B-1-2

- FFS

  - Option 1:

    - The maximum length of each interruption is based on RF tuning time of 0.5 ms for FR1 and 0.25 ms for FR2 and additional factors and is defined as follows:

Interruption length in NR CA:

Table 1: Interruption due to RLM and RLP measurements on other active serving cell in inter-band NR CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 2: Interruption due to RLM and RLP measurements on other active serving cell in intra-band NR CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + NSSB |
| 1 | 0.5 | 1 + NSSB |
| 2 | 0.25 | 2 + NSSB |
| 3 | 0.125 | 4 + NSSB |
| 5 | 0.03125 | 16 + NSSB |
| 6 | 0.015625 | 32 + NSSB |
| NOTE: NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR |  |  |

Interruption length in NR DC:

- For synchronous inter-band NR-DC, the interruption length is the same as in table 2.

- For asynchronous inter-band NR-DC, the interruption length is shown in table 3.

Table 3: Interruption due to RLM and RLM on other active serving cell in asynchronous inter-band NR-DC

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 4 |
| 3 | 0.125 | Aggressor cell is on FR2 | 5 |
|  |  | Aggressor cell is on FR1 | 6 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 18 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 34 |

Interruption length in EN-DC and NE-DC:

For inter-band EN-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band EN-DC or

- 2 subframes for asynchronous inter-band EN-DC.

For synchronous intra-band EN-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

For inter-band NE-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band NE-DC or

- 2 subframes for asynchronous inter-band NE-DC.

For synchronous intra-band NE-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

Where, NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR.

  - Option 2:

    - The maximum length of each interruption is based on RF tuning time of 1 ms for FR1 and 0.75 ms for FR2.

#### Issue 5-2c: Interruption ratio for option B-1-2

- FFS

  - Option 1:

    - For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%].

  - Option 1a:

    - The interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to 0.5%.

    - The maximum interruption probability on other active serving cells due to RLM/LRP measurements performed on the serving cell are defined:

      - The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD measurements on the serving cell shall not exceed 0.5 %.

      - The rate of ACK/NACK feedback loss on any other active serving cell resulting from CBD measurements on the serving cell shall not exceed 0.5 %.

      - The rate of ACK/NACK feedback loss on any other active serving cell resulting from L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 0.5 %.

  - Option 2:

    - For UE supporting option B-1-2, the probability of missed ACK/NACK is 1% for ALL RLM/BFM/BM(L1-RSRP/L1-SINR) measurements based on SSB outside active BWP.

  - Option 2a:

    - The probability of missed ACK/NACK for a UE supporting Option B-1-2, due to interruptions caused by UE performing BM/RLM/BFD measurements based on SSB outside the active BWP, shall not exceed [1.0]%.

  - Option 3:

    - X%=interruption length * 2 / L1-RS periodicity, where X% is the interruption ratio, interruption length is 0.5ms in FR1 and 0.25ms in FR2, and L1-RS periodicity is the periodicity of SSB configured for BM/RLM/BFD after taking scaling factor P into account.

  - Option 4:

    - RAN4 shall define interruption length for Option B-1-2 and RAN shall not define interruption ration for the same option.

#### Issue 5-4: How interruption requirements for supporting option B-1-2 are captured

- The existing clause 8.2 in 38.133 and clauses 7.32 and 7.36 in 36.133, can be reused for defining the interruption requirements as shown below in table 4 and table 5:

- Table 4: List of top sections on interruption requirements in TS 38.133 impacted for UE supporting option B-1-2:

| No. | Section number | Section title |
| --- | --- | --- |
| 1 | 8.2.2 | SA: Interruptions with Standalone NR Carrier Aggregation |
| 2 | 8.2.4 | NR-DC: Interruptions |

- Table 5: List of top sections on interruption requirements in TS 36.133 impacted for UE supporting option B-1-2:

| No. | Section number | Section title |
| --- | --- | --- |
| 1 | 7.32 | 7.32 Interruptions with EN-DC |
| 2 | 7.36 | 7.36 Interruptions with NE-DC |

----EoD---