3GPP TSG RAN WG#4 Meeting #108bis                                                        R4-2317646

Xiamen, China, Oct 9th – 13th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | draftCR | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running draftCR to TS 36.102 on introducing L+S FDD band for IoT NTN operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., ZTE, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_LS_band-Core |  |  |  |  |  | Date: |  |  | 2023-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A placeholder and running draft CR for consolidating all draft CRs endorsed in RAN4 on top of endorsed draft CR R4-2314713 and the agreements RAN4#108bis and merged CRs R4-2315679, R4-2316131, R4-2316658. Note that 6.5A.4.4.4 and 6.5B.4.4.4 for NS_03N (FCC) will be added later. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add NS_03N into the tables in 6.2A.3 and 6.2B.3 - Update out-of-band blocking for Cat. M1 and NB1/NB2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | - 5.2, 5.4A.2, 5.4A.3, 6.2A.1, 6.2A.3, 6.2B.1, 6.2B.3, 7.3, 7.3A, 7.3B, 7.6A, 7.6B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change >>

# 5	Operating bands and channel arrangement

## 5.2	Operating bands

E-UTRA satellite access is designed to operate in the operating bands defined in Table 5.2-1.

Table 5.2-1 E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

## 5.4A	Channel arrangement for category M1

### 5.4A.2	Channel raster, carrier frequency and EARFCN

The global frequency raster is defined for all frequencies. The granularity of the global frequency raster is 100 kHz, which means that the carrier centre frequency must be an integer multiple of 100 kHz. For each operating band, a subset of frequencies from the global frequency raster are applicable and forms a channel raster with a granularity ΔFRaster.

The carrier frequency in the uplink and downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143. The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in Table 5.4A.2-1 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in Table 5.4.2-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

The applicable channel raster and EARFCNs for each operating band are specified in Table 5.4A.2-1.

For operating bands with a channel raster of 100 kHz, every EARFCN within the operating band shall be applicable for the channel raster, and the step size for the channel raster in Table 5.4A.2-1 is given as <1>. The broadcast parameter earfcn-LSB defined in TS36.331 [6] may be used to assist the UE in synchronizing to the cell.

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 –<1>- 229375 | 1980 | 261844 | 261844 –<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 –<1>- 229075 | 1626.5 | 261504 | 261504 –<1>- 261843 |
| 254 | 100 | 2483.5 | 228571 | 228571 –<1>- 228735 | 1610 | 261339 | 261339 –<1>- 261503 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

### 5.4A.3	TX–RX frequency separation

The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.4A.3-1 for the TX and RX channel bandwidth defined in Table 5.3A-1.

Table 5.4A.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 256 | 190 MHz |
| 255 | -101.5 MHz |
| 254 | 873.5 MHz |

# 6	Transmitter characteristics

## 6.2A	Transmit power for category M1

### 6.2A.1	UE maximum output power for category M1

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.2A.1-1: UE Power Class

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | +/-2 | 20 | +/-2 |
| 255 |  |  | 23 | +/-2 | 20 | +/-2 |
| 254 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

### 6.2A.3	UE additional maximum output power reduction for category M1 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2A.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2A.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | N/A |
| NS_03N | 6.5A.4.4.4 | 254 | Table 5.3A-1 | N/A |
| NS_24 | 6.5A.4.4.3 | 256 | Table 5.3A-1 | ≤ 3.5 |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2A.3-2 for category M1 UE are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-2: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | N/A |
| NS_02N | 6.5A.4.4.2 | 255 | N/A |
| NS_03N | 6.5A.4.4.4 | 254 | N/A |
| NS_24 | 6.5A.4.4.3 | 256 | ≤ 3.5 |

## 6.2B	Transmit power for category NB1 and NB2

### 6.2B.1	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |
| 254 | 23 | +/-2 | 20 | +/-2 |

### 6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |
| NS_24 | 6.5B.4.4.3 | 256 | ≤ [3.5] |

## 6.5A	Output RF spectrum emissions for category M1

### 6.5A.4	Spurious emission for category M1

#### 6.5A.4.3	Spurious emission band UE co-existence

This clause specifies the requirements for E-UTRA satellite bands for UE coexistence with protected bands.

Table 6.5A.4.3-1: Requirements for spurious emissions for UE co-existence

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 254 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 31, 41, 48, 54, 66, 70, 71, 72, 85, 87, 88, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n54, n65, n67, n74, n75, n76, n77, n78, n90, n91, n92, n93, n94, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 255 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n65, n67, n74, n75, n76, n90, n91, n92, n93, n94 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 256 | E-UTRA Band 1, 3, 5,7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 33, 32, 35, 38, 40, 41, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n12, n13, n14, n24, n29, n30, n39, n48, n53, n66, n67, n71, n78, n79, n85, n90, n91, n92, n93, n94, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n2, n25, n70 | FDL_low | - | FDL_high | NA | NA | 3 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.4A.2-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5A.4.2-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: The co-existence between 256 and band 2, 25 and 70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

#### 6.5A.4.4	Additional spurious emissions

##### 6.5A.4.4.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	In addition to the requirements below, additional UE region-specific emissions requirements for European are expected to be added once more information becomes available.

##### 6.5A.4.4.2	Minimum requirement (network signalled value "NS_02N")

When "NS_02N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.2-1 and 6.5A.4.4.2-2. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth. Network signalling remark NS_02N applies integer-value 2. The NTN IoT band operation which applies NS_02N should avoid applying NS_02.

Table 6.5A.4.4.2-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4MHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Table 6.5A.4.4.2-2: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 – 0.7 | -2 for PC3 -5 for PC5 | 4 kHz |
| 0.7 – 2.8 | -12 for PC3 -15 for PC5 | 4 kHz |
| >2.8 | -13 for PC3 and PC5 | 4 kHz |

##### 6.5A.4.4.3	Minimum requirement (network signalled value "NS_24")

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3-1.

Table 6.5A.4.4.3-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4MHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |  |  |

##### 6.5A.4.4.4	Minimum requirement (network signalled value "NS_03N")

(To be added)

#### 6.5B.4.4	Additional spurious emissions

##### 6.5B.4.4.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	In addition to the requirements below, additional UE region-specific emissions requirements for European are expected to be added once more information becomes available.

##### 6.5B.4.4.2	Minimum requirement (network signalled value "NS_02N")

When "NS_02N" is indicated in the cell, the power of any UE spurious emission shall not exceed the levels specified in Table 6.5B.4.4.2-1 and 6.5B.4.4.2-2. This requirement also applies for the frequency ranges that are less than FOOB (MHz) specified in 6.5B.4.2 from the edge of the channel bandwidth. Network signalling remark NS_02N applies integer-value 2. The NTN IoT band operation which applies NS_02N should avoid applying NS_02.

Table 6.5B.4.4.2-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Table 6.5B.4.4.2-2: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  [0.09 – 0.28] | -2 for PC3 -5 for PC5 | 4 kHz |
|  [0.28 – 0.85] | -12 for PC3 -15 for PC5 | 4 kHz |
|  [>0.85] | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [10], of 0.38 MHz.

##### 6.5B.4.4.3	Minimum requirement (network signalled value "NS_24")

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.4.3-1.

Table 6.5B.4.4.3-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |  |  |

##### 6.5B.4.4.4	Minimum requirement (network signalled value "NS_03N")

(To be added)

## 7.3	Reference sensitivity power level

This clause is reserved.

## 7.3A	Reference sensitivity power level for UE category M1

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category M1, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes [A.2.2 and A.3.2] (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex [A.5.1.1]) with parameters specified in Table 7.3A-1 and Table 7.3A-2 for category M1.

Table 7.3A-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 254 | -102.2 | FDD |
| 255 | -102.7 | FDD |
| 256 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101 [7]. |  |  |

Table 7.3A-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 254 | -103.1 | HD-FDD |
| 255 | -103.5 | HD-FDD |
| 256 | -103 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3A-1/Table 7.3A-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3A-3.

NOTE:	Table 7.3A-3 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex [G] (informative).

Table 7.3A-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 254 | 61 | FDD and HD-FDD |
| 255 | 61 | FDD and HD-FDD |
| 256 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3A-1). |  |  |

## 7.3B	Reference sensitivity power level for UE category NB1 and NB2

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with received signal level as specified in Table 7.3B-1. Requirement in Table 7.3B-1 applies for any uplink configuration.

Table 7.3B-1: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

## 7.6A	Blocking characteristics for category M1

### 7.6A.2	In-band blocking requirements for category M1

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 [7] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 [7] Annex A.5.1.1) with parameters specified in Tables 7.6A.2-1 and 7.6A.2-2. For operating bands with an unpaired DL part (as noted in Table 5.2A-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6A.2-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| BWInterferer | MHz | 1.4 |
| FIoffset, case 1 | MHz | 2.1+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.6A.2-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255, 254 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

### 7.6A.3	Out-of-band blocking requirements for category M1

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band. For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5A and subclause 7.6A.2 shall be applied.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 [7] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 [7] Annex A.5.1.1) with parameters specified in Tables 7.6A.3-1 and 7.6A.3-2.

Table 7.6A.3-1: Out-of-band blocking parameters for category M1 UE

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 1.4 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [7] with PCMAX_L as defined in subclause 6.2.5. |  |  |

Table 7.6A.3-2: Out of-band blocking for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2542 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 255 | Finterferer (C`W) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 2: The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

For Table 7.6A.3-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configuration. For these exceptions the requirements of subclause 7.7A spurious response are applicable.

## 7.6B	Blocking characteristics for category NB1 and NB2

### 7.6B.3	Out-of-band blocking requirements for category NB1 and NB2

For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5B and subclause 7.6B.2 shall be applied.

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 [7] Annexes A.3.2 with parameters specified in Table 7.6B.3-1.

For Table 7.6B.3-1 in frequency range 1, 2 and 3, up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7B spurious response are applicable.

Table 7.6B.3-1: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 2545 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2562 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 in TS 36.101 [7] with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 3: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz.” NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 5: The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

## << End change >>