3GPP TSG-RAN WG4 Meeting # 108-bis											R4-2317586

Xiamen, China, October 09 – October 13, 2023

Title: 	WF on HPUE for FDD bands

Agenda Item:	4.22

Source: 	China Unicom

Document for:	Approval

<Topic 1 UTRA protection issues>

<Agreement>:

- Reuse the PC3 UTRA ACLR for PC2, i.e.,

Proposed UTRA ACLR requirement

|  | Power class 2, 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

- The PC2 A-MPR for NS_xxU (i.e. UTRA protection related NS) can be defined as max{A-MPR for NS_100, PC2 A-MPR for NS_xx}.

<Topic 2 Reference Sensitivity Degradation>

<Sub-topic 2-1 1Tx sensitivity degradation>

<Agreement>:

- Take the average values as agreement for n7 (based on 2Rx).

- The Four antenna port reference sensitivity allowance ΔRIB,4R is also applicable for PC2 for band n7

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n7 | Skyworks (R4-2311152) | 0.3 | 0.5 | 0.6 | 0.7 | 0.9 | 0.9 | 0.9 | 0.9 | - | 2.1 |
|  | Apple (R4-2311251) | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | N/A | 2.0 |
|  | Murata (R4-2311442) | 0.7 | 0.4 | 0.3 | 0.2 | 0.2 | 0.1 | 0.1 | 0.1 |  | 0.8 |
|  | Qualcomm (R4-2313553) | 0.6 | 0.4 | 0.4 | 0.4 | 0.3 | 0.3 | 0.3 | 0.3 |  | 1.8 |
|  | Huawei, HiSilicon (R4-2316472) based on 2Rx | 0.8 | 0.8 | 0.8 | 1.1 | 1.1 | 1.1 | 1.1 | 1.1 | - | 3.2 |
|  | Average (Moderator) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |  | 2.0 |

<Way forward>: Take the the following values in table below for n13 and n14 as starting point, companies are encouraged to further check the values for FDD PC2 by 1Tx architecture:

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n13 | Skyworks(R4-2300652) | 0.8 | 0.8 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 0.9 | 1.0 |  |  |  |  |  |  |  |  |
|  | Average (Moderator) | 0.8 | 0.9 |  |  |  |  |  |  |  |  |
| n14 | Skyworks (R4-2311152) | 0.4 | 0.8 |  |  |  |  |  |  |  |  |
|  | Apple (R4-2311251) | 0.3 | 0.6 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 0.9 | 1.0 |  |  |  |  |  |  |  |  |
|  | Average (Moderator) | 0.5 | 0.8 |  |  |  |  |  |  |  |  |

<Sub-topic 2-2 2Tx sensitivity degradation>

<Agreement>:

- Take the average values as agreement for n7 (based on 2Rx).

- The Four antenna port reference sensitivity allowance ΔRIB,4R is also applicable for PC2 for band n7

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n7 | Skyworks (R4-2311152) | 0.5 | 0.9 | 1.1 | 1.2 | 1.4 | 1.4 | 1.4 | 1.4 | - | 5.5 |
|  | Apple (R4-2311251) | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | N/A | 5.6 |
|  | Murata (R4-2311442) | 0.6 | 0.6 | 0.6 | 0.6 | 0.7 | 0.6 | 0.6 | 0.7 |  | 5.6 |
|  | Qualcomm (R4-2313553) | 1.2 | 1.2 | 1.3 | 1.3 | 1.2 | 1.2 | 1.2 | 1.4 |  | 3.8 |
|  | Huawei, HiSilicon (R4-2316472) based on 2Rx | 1.4 | 1.5 | 1.4 | 1.3 | 1.5 | 1.5 | 1.5 | 1.5 | - | 6.2 |
|  | Average (Moderator) | 0.9 | 1.0 | 1.0 | 1.0 | 1.1 | 1.1 | 1.1 | 1.1 |  | 5.3 |

<Way forward>: Take the the following values in table below for n13 and n14 as starting point, companies are encouraged to further check the values for FDD PC2 by 2Tx architecture:

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n13 | Skyworks(R4-2300652) | 1.7 | 1.7 |  |  |  |  |  |  |  |  |
|  | Apple(R4-2305364) | 0.5 | 0.5 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 2.0 | 2.0 |  |  |  |  |  |  |  |  |
|  | Average (Moderator) | 1.4 | 1.4 |  |  |  |  |  |  |  |  |
| n14 | Skyworks (R4-2311152) | 0.8 | 1.3 |  |  |  |  |  |  |  |  |
|  | Apple (R4-2311251) | 0.8 | 0.8 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 2.0 | 2.0 |  |  |  |  |  |  |  |  |
|  | Average (Moderator) | 1.2 | 1.4 |  |  |  |  |  |  |  |  |

- Comments

| Company | Comments |
| --- | --- |

<Topic 3 A-MPR>

<Sub-topic 3-1 NS_35 (A-MPR for n71 PC2)>

- Proposal 1 (R4-2315373): For PC2 A-MPR use the region below and set the allowed power back-off to 3dB DFT-s-OFDM and 4.5dB for and CP-OFDM.

(LCRB ≤0.20∙NRB and (RBstart ≤ 5 or RBstart +LCRB ≥NRB -5)) or ( LCRB = 1 and 5 ∙ ¦ RBstart + 0.5 – NRB / 2 ¦ ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

- Proposal 2 (R4-2316860):  The following yellow highlighted text is used for PC2 NS_35 in section 6.2.3.31, values are captured in brackets to confirm CBW > 20MHz and 2Tx case:

For power class 2 operation A-MPR = [3] dB for DFT-s-OFDM and A-MPR = [4.5] dB for CP-OFDM and for power class 1 operation A-MPR = 8.5 dB if

( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )

or

( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

- Recommended WF

  - Proposal 1

- Comments

| Company | Comments |
| --- | --- |

<Sub-topic 3-2 NS_46 (A-MPR for n7 PC2)>

- Proposal 1 (R4-2315375): Consider Table 1 and Table 2 when defining requirements for PC2. For simplicity and to avoid inconsistencies in case of power class fallback re-use the PC3 RB restrictions for PC2.

Table 1: A-MPR regions for NS_46 for PC2 (red colour indicates changes to original proposal from [2])

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2512.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.5 | >0 | A4 |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 16.2 | A10 |

Table 2: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 256 QAM | 6.5 |  |  | 6 | 10 | 6.5 |  |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 8 |  |

- Proposal 2 (R4- 2316473): Align the carrier centre frequency range for BW=25MHz with that for PC3 when defining the A-MPR requirements. Consider specifying A-MPR and removing RB restrictions for BW=15/20MHz for PC2 capable UEs. Take into account the simulation results in this contribution (R4- 2316473) when determining the PC2 A-MPR for NS_46, especially for the A-MPR values of A5/A6/A7/A8.

- Proposal 3 (R4-2316694): Adopt the A-MPR regions and A-MPR values in Table 1 and Table 2 for NS_46 for PC2 operation.

Table 1: A-MPR regions for NS_46 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 2505 ≤ FC ≤ 2565 | ≥ 7.92 | > 0 | A9 |
|  |  | ≥ 7.92 | < 5.4 | A10 |
| 15 MHz | 2507.5 ≤ FC ≤ 2562.5 | ≥ 11.16 | > 0 | A9 |
|  |  | ≥ 11.16 | < 5.4 | A10 |
| 20 MHz | 2510 ≤ FC ≤ 2560 | ≥12.6, < 15.3 | ≥ 10.8 | A7 |
|  |  | ≥ 15.3 | > 0 | A9 |
|  |  | ≥ 14.4 | < 5.4 | A10 |
| 25 MHz | 2512.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.5 | >0 | A4 |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 19.8 | A10 |

Table 2: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 256 QAM | 6.5 |  |  | 6 | 10 | 6.5 |  |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 8 |  |

- Proposal 4 (R4-2316694): Allow PC2 capable UEs to reduce their output power 23 dBm – max(MPR,4) dB in case PC2 needs to fall back to PC3 operation and transmit more than 54 RB. Apply this for both SC-FDMA and CP-OFDM and all modulation orders.

- Recommended WF

  - Use proposal 3 as starting point, further discuss the following issues:

    - The A-MPR regions and values for BW≥25MHz

    - Whether to specify A-MPR for BW≤20MHz? If yes, how to treat power class fallback?

    - The A-MPR regions and values for BW≤20MHz if needed

    - How to specify the carrier centre frequency range for BW≤25MHz (e.g. does it need to cover the whole range of band n7?)

- Comments

- The A-MPR regions and values for BW≥25MHz

| Company | Comments |
| --- | --- |

- Whether to specify A-MPR for BW≤20MHz? If yes, how to treat power class fallback?

| Company | Comments |
| --- | --- |

- The A-MPR regions and values for BW≤20MHz if needed

| Company | Comments |
| --- | --- |

- The carrier centre frequency range for BW≤25MHz (e.g. does it need to cover the whole range of band n7?)

| Company | Comments |
| --- | --- |

<Sub-topic 3-3 NS_06>

<Way forward>: Take the simulation results in R4-2316474 as starting point when deciding the PC2 A-MPR for NS_06 based on 1Tx. It is encouraged for more input and further discuss in the next meeting.

<Sub-topic 3-4 NS_07 (A-MPR for n13 PC2)>

<Way forward>: Take the simulation results in R4-2316475 as starting point when deciding the PC2 A-MPR for NS_07 based on 1Tx. It is encouraged for more input and further discuss in the next meeting.

<Sub-topic 3-5 NS_03>

<Agreement>: The following yellow highlighted text is used for PC2 NS_03 in Table 6.2.3.7-1. Values are captured in brackets to confirm the CBW > 20MHz and 2Tx case:

Table 6.2.3.7-1 A-MPR for NS_03

| Modulation/Waveform |  | Outer (dB) | Inner (dB) |
| --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 | N/A1 |
|  | QPSK | ≤ 2 |  |
|  | 16 QAM | ≤ 3 |  |
|  | 64 QAM | ≤ 3.5 |  |
|  | 256 QAM | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 4 |  |
|  | 16 QAM | ≤ 4 |  |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.5 |  |
| NOTE 1: Void for DFT-s-OFDM 1RB allocations, [0.5] dB A-MPR is required for PC2 operation. NOTE 2: Void |  |  |  |

| Company | Comments |
| --- | --- |

<Sub-topic 3-6 A-MPR for n26 PC2>

<Way forward>: Take the simulation results in R4-2315374 as starting point when deciding the PC2 A-MPR for NS_12, NS_13, NS_14, NS_15. It is encouraged for more input and further discuss in the next meeting.

Table 1: A-MPR regions for NS_12

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 5MHz | ≤1.8 | >0 | A1 |
|  | >1.8 | >1.08 | A2 |
| 10MHz | ≤3.6 | >0 | A1 |
|  | >3.6, ≤5.04 |  | A2 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 +2.5 | 3.0 |
| DFT-s-OFDM QPSK | ≤ 5 +2.5 | 3.0 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 +2 | 3.0 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 +2 | 3.0 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | 4.5 |
| CP-OFDM QPSK | ≤ 7 +1.5 | 4.5 |
| CP-OFDM 16 QAM | ≤ 7 +1.5 | 4.5 |
| CP-OFDM 64 QAM | ≤ 7 +1.5 | 4.5 |
| CP-OFDM 256 QAM | ≤ 9.5 | 6.5 |

Table 3: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
|  |  | >1.44 | ≥1.8 | A3 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
|  |  |  | <3.24, ≥2.52 | A4 |

Table 4: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 +1.0 | ≤ 4.5 +2 | ≤ 3 +1.0 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 +1.0 | ≤ 4.5 +2 | ≤ 3 +1.0 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 +1.0 | ≤ 5 +1.5 | ≤ 3 +1.0 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 +1.5 | ≤ 3 +1.0 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 +1 |  |  |
| CP-OFDM QPSK | ≤ 5 +1.0 | ≤ 6.5 +1.0 | ≤ 4.5 +1.0 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 +1.0 | ≤ 6.5 +1.0 | ≤ 4.5 +1.0 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 +1.0 | ≤ 4.5 +1.0 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

Table 5: A-MPR regions for NS_14

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10MHz | ≤0.18 | <1.08 | A1 |
|  | ≥0 | ≥9 | A2 |
|  | ≥0 | ≥8.64, <9 | A4 |
| 15MHz | ≤1.8 | <1.8 | A1 |
|  | ≥0 | ≥9 | A2 |
|  | ≤1.8 | ≥1.8, <3.6e6 | A4 |
|  | ≥0 | ≥8.64, <9 | A4 |
| 20MHz | ≤3.42 | <1.8 | A3 |
|  | ≥0 | ≥9 | A2 |
|  | ≤3.42 | ≥1.8, <3.6e6 | A4 |
|  | ≥0 | ≥8.64, <9 | A4 |

Table 6: A-MPR for NS_14

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3 +1.5 | ≤ 2 +1.5 | ≤ 3 +1.5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3 +1.5 | ≤ 2 +1.5 | ≤ 3 +1.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3 +1.5 | ≤ 2 +1.5 | ≤ 3 +1.5 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 3 +1.5 |  | ≤ 3 +1.5 | ≤ 3 |
| DFT-s-OFDM 256 QAM |  |  | ≤ 8 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 5 +1.5 | ≤ 4 +1.0 | ≤ 5 +1.5 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 5 +1.5 | ≤ 4 +1.0 | ≤ 5 +1.5 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 6 +0.5 | ≤ 5.0 | ≤ 6 +0.5 | ≤ 3.5 |
| CP-OFDM 256 QAM | ≤ 8 |  | ≤ 8 | ≤ 6.5 |

Table 7: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
|  |  | <2.53, ≥1.8 | ≥1.44 | A5 |
|  |  | <1.08 | >0.36, ≤0.72 | A5 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  |  | <4.14, ≥1.8 | ≥2.52 | A5 |
|  |  | <2.52 | >0.36, ≤0.72 | A5 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
|  |  | <7.2, ≥5.22 | <4.32, ≥3.24 | A5 |
|  |  | <1.44 | >0.36, ≤0.72 | A5 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  |  | <4.68, ≥1.8 | ≥2.52 | A5 |
|  |  | <3.96 | >0.36, ≤1.08 | A5 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A5 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  |  | <2.7 | >0.36, ≤0.72 | A5 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
|  |  | <1.08 | >0.36, ≤0.72 | A5 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  |  | < 8.46, ≥1.8 | ≥3.6 | A5 |
|  |  | <6.12 | >0.36, ≤1.08 | A5 |
|  |  | <12.24, ≥8.46 | < 5.4, ≥3.6 | A5 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |
|  |  | <4.68 | >0.36, ≤1.08 | A5 |

Table 8: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 +1.5 | ≤ 5 +1.5 | ≤ 4 +3 | ≤ 9 +1.5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 9 +1.5 | ≤ 5 +1.5 | ≤ 4 +3 | ≤ 9 +1.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 9 +1.5 | ≤ 5 +1.5 | ≤ 4 +3 | ≤ 9 +1.5 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 9 +1.5 | ≤ 5 +1.5 | ≤ 4 +3 | ≤ 9 +1.5 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 9 +1.5 | ≤ 5 +1.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 10.5 +2 | ≤ 6.5 +1.5 | ≤ 4 +3 | ≤ 10.5 +1.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 10.5 +2 | ≤ 6.5 +1.5 | ≤ 4 +3 | ≤ 10.5 +1.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 10.5 +2 | ≤ 6.5 +1.5 | ≤ 4 +3 | ≤ 10.5 +1.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 10.5 +2 | ≤ 6.5 +1.5 | ≤ 9 | ≤ 13.5 | ≤ 6.5 |

| Company | Comments |
| --- | --- |

<Sub-topic 3-7 A-MPR for n28 PC2>

<Agreement>:

- Analyze and specify A-MPR for NS_17 and NS_18 assuming full band duplexer and no filter attenuation towards the additional emission requirements.

- Analyze also A-MPR for 3 MHz channel bandwidth for n28 PC2.

- Comments

  - Proposal 1

| Company | Comments |
| --- | --- |

  - Proposal 2

| Company | Comments |
| --- | --- |

<Topic 4 Draft CR on Adding PC2 requirements for band n8 and n28. (R4-2316477)>

- Comments

| Company | Comments |
| --- | --- |