3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315495

# Xiamen, China, 9th – 13th October, 2023

Title:	LS on update for “asyncIntraBandENDC“

Response to:

Release:	Rel-16

Work Item:	TEI16

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Apple

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

During the discussion on MRTD requirement applicability in RAN4#108bis meeting, it is identified that the current description for asyncIntraBandENDC may need to be updated to well accommodate inter-band EN-DC with overlapping frequency in non-collocated scenario.

The signaling “asyncIntraBandENDC” was introduced for intra-band EN-DC, while it is further extended to cover inter-band EN-DC with overlapping frequency. While the signaling description only mentioned intra-band EN-DC in the first paragraph in the description part. For inter-band EN-DC with overlapping frequency, if asynchronous FDD-FDD intra-band (NG) EN-DC is not supported, the MRTD requirement for collocated scenario and non-collocated scenario are different.

- For collocated scenario, the synchronous requirement for intra-band collocated scenario in 7.6.3 apply.

- For non-collocated scenario, the synchronous requirement for inter-band EN-DC in 7.6.2.1 apply.

Further, the signaling “asyncIntraBandENDC” now only cover EN-DC UE reporting. RAN4 would like to ask RAN2 to update this signaling description to cover NE-DC reporting.

# 2	Actions

To RAN2:

ACTION: 	RAN4 respectfully asks RAN2 to consider the above information and consider updating the signaling description for asyncIntraBandENDC.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #109                13th – 17th Nov 2023			Chicago, US

TSG-RAN WG4 Meeting #110                26th Feb – 1st Mar 2024			Athens, Greece