3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315011

Xiamen, China, 09 – 13 October 2023

3GPP TSG RAN WG1 Meeting #114	R1-2308610

Toulouse, France, 21st – 25th August 2023

Title:	LS on reduced peak data rate for Rel-18 eRedCap UEs

Response to:

Release:	Rel-18

Work Item:	Enhanced support of reduced capability NR devices (NR_redcap_enh-Core)

Source:	RAN1

To:	RAN2

Cc:	RAN4

Contact person:	Johan Bergman, johan (dot) bergman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1 has agreed the following for the Rel-18 eRedCap WI objective on peak data rate reduction, which seems to have specification impacts at least on TS 36.306 clause 4.1.2 (‘Supported max data rate for DL/UL’).

- The UE signals peak data rate related parameters vLayers, Qm and f corresponding to 10 Mbps.

  - No new values for the above parameters will be introduced for Rel-18 eRedCap.

- For UE peak data rate reduction with UE BB bandwidth reduction (i.e., FG 48-1),

  - The 10-Mbps peak rate target corresponds to a vLayers·Qm·f of 3.2.

  - 25 PRBs for μ=0 and 12 PRBs for μ=1 for  is always assumed in the UE supported max data rate expression in TS 38.306 for any given band.

- For UE peak data rate reduction without UE BB bandwidth reduction (i.e., FG 48-2),

  - When vLayers = 1, the 10-Mbps peak rate target corresponds to a vLayers·Qm·f of 0.75.

  - When vLayers = 2, the peak rate target corresponds to a vLayers·Qm·f of 0.8.

  - 106 PRBs for μ=0 and 51 PRBs for μ=1 for  is always assumed in the UE supported max data rate expression in TS 38.306 for any given band.

- In all cases, the same value for vLayers·Qm·f is used for DL and UL.

RAN1 respectfully requests RAN2 to take the above into account in their future work.

# 2	Actions

To RAN2:

ACTION: 	RAN1 respectfully requests RAN2 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #114-bis	9th – 13th October 2023		Xiamen, CN

TSG-RAN WG1 Meeting #115	13th – 17th November 2023	Chicago, US