3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315016

Xiamen, China, 09 – 13 October 2023

3GPP TSG-RAN WG2 Meeting #123	R2-2309159

# Toulouse, France, 21 – 25 August, 2023

Title:	On frequencyInfo for NR SL RSRP measurements

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN2

To:	RAN1

Cc:	RAN4, RAN5

Contact person:	Tao Cai

Tao.Cai@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN2 received from RAN5 one LS R5-233768/R2-2307050 on frequencyInfo for NR SL RSRP measurements. RAN2 understands that, without field description in TS 38.331, the exact meaning of frequencyInfoSL-r16 in IE SL-MeasObject-r16 can be unclear, and this ambiguity can lead to unpredictable UE behaviour during testing.

RAN2 understands that the field frequencyInfoSL-r16 should be the indication of the carrier, on which SL-RSRP measurement is performed, RAN2 respectfully asks RAN1 to confirm that the below field description for frequencyInfoSL in IE SL-MeasObjectList is acceptable:

"It indicates the lowest usable subcarrier on this carrier, determined according to sl-AbsoluteFrequencyPointA in IE SL-FreqConfig/SL-FreqConfigCommon and offsetToCarrier in IE SCS-SpecificCarrier configured for sl-SCS-SpecificCarrierList in IE SL-FreqConfig/SL-FreqConfigCommon. See TS 38.211 [16], clause 4.4.2."

# 2	Actions

To RAN1

RAN2 respectfully asks RAN1 to feedback whether the above field description of frequencyInfoSL in IE SL-MeasObjectList in TS 38.331 is acceptable.

# 3	Dates of next TSG-RAN WG2 meetings

TSG RAN WG2 Meeting #123-bis			October 9 – 13, 2023		Xiamen, China

TSG RAN WG2 Meeting #124			November 13 – 17, 2023	Chicago, US