3GPP RAN WG4 Meeting #109		R4-2318433

Chicago, USA, November 13th – 17th, 2023

Agenda item:	7.26.2

Source:	Apple

Title:	n8 30MHz DL REFSENS and PC2 RSD

WI/SI:	NR_bands_R18_BWs-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

n8 30MHz DL channel bandwidth was recently proposed and approved in RAN #101 meeting [1]. As n8 is also being proposed to support PC2 [2], in addition to PC3 REFSENS with 20MHz/30MHz asymmetric UL/DL channel bandwidth, PC2 RSD relative to PC3 REFSENS also needs to be evaluated and specified. In this contribution, provide our n8 DL 30MHz PC3 REFSENS and PC2 RSD analysis results for consideration.

# Discussion

Our PC3 REFSENS and PC2 RSD for n8 DL 30MHz channel BW is based on numerical simulations and link analysis with UL/DL carrier configuration as shown in Figure 2-1.

Figure 2-1 n8 30MHz DL carrier configuration for REFSENS and PC2 RSD analysis

Since at this carrier configuration, the REFSENS degradation is expected to be dominated by Tx noise spillage into DL carrier, only Tx noise is considered in both PC3 REFSENS and PC2 RSD analysis. Table 2-1 summarizes the simulation assumptions for the analysis.

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx and PA model | Same as used for MPR/A-MPR simulations | CIM3, CIM5, IRR, etc. |
| Tx power | 23dBm (PC3) and 26dBm (PC2) | At antenna port |
| FE insertion loss | 4 dB |  |
| Channel BW | UL/DL: 20/30 | MHz |
| UL configuration | 20RB RBStart = 87 |  |

Table 2-1 Simulation assumptions for n8 30MHz DL REFSENS and PC2 RSD analysis

Figure 2-2 shows the simulated PC3 spectral profile at n8 PA output where the range highlighted in light green is the 30MHz DL carrier allocation.

Figure 2-2 Simulated PC3 spectral profile at n8 PA output

The simulated PC3 Tx noise power in DL carrier range was then incorporated in link analysis for PC3 REFSENS calculation where further assumed parameters are summarized in Table 2-2.

| Parameter | Value | Comment |
| --- | --- | --- |
| FE insertion loss | 4 dB |  |
| Duplexer isolation | 50 dB | Tx to Rx |
|  | 42 dB | Tx to ANT |
| Antenna isolation | 10 dB | Main to diversity |
| MRC |  | Uncorrelated |

Table 2-2 Assumed parameters for n8 REFSENS link analysis

For PC2 RSD with 1Tx implementation, the PC2 Tx noise in DL carrier range was also incorporated in the link analysis for PC2 REFSENS calculation first using the same assumed parameters as summarized in Table 2-2. The PC2 RSD was then derived by taking the PC2 and PC3 REFSENS difference in dB. For PC2 RSD with 2Tx implementation, the simulated PC3 Tx noise in both main path and diversity path are added to thermal noise as the total noise for both Tx path and diversity path, followed by MRC to calculate the equivalent total noise for 2Tx configuration. The MSD was then derived by taking the difference between the 2Tx PC2 equivalent total noise and the PC3 equivalent total noise. To account for the PA reverse IMD when 2PA are transmitting simultaneously, additional 1dB Tx noise contribution was assumed in our analysis.

Table 2-3 summarizes the PC3 REFSENS and PC2 RSD based on our analysis.

| n8 | PC3 REFSENS (dBm) | 1Tx | -81.2 |
| --- | --- | --- | --- |
|  | PC2 RSD (dB) | 1Tx | 2.3 |
|  |  | 2Tx | 5.1 |

Table 2-3 Simulated n8 30MHz DL PC3 REFSENS and PC2 RSD

Proposal: RAN4 to take the n8 DL 30MHz PC3 REFSENS and PC2 RSD values in Table 2-3 into consideration for the corresponding specifications development.

# 3	Conclusion

In this contribution, we provide our n8 DL 30MHz PC3 REFSENS and PC2 RSD analysis results for consideration.

Proposal: RAN4 to take the n8 DL 30MHz PC3 REFSENS and PC2 RSD values in the table below into consideration for the corresponding specifications development.

| n8 | PC3 REFSENS (dBm) | 1Tx | -81.2 |
| --- | --- | --- | --- |
|  | PC2 RSD (dB) | 1Tx | 2.3 |
|  |  | 2Tx | 5.1 |

# 4	References

RP-231706 “Basket WID on adding channel bandwidth support to existing NR bands”, Ericsson, 3GPP TSG-RAN Meeting #101, Bengaluru, India, September 11th – 15th, 2023

RP-231064 “Revised WID High power UE (power class 2) for NR FR1 FDD single band”, China Unicom, 3GPP TSG-RAN Meeting #100, Taipei, Taiwan, June 12th – 14th, 2023