3GPP TSG-RAN WG4 # 109	R4-2318557

Chicago, USA, November 13 – November 17, 2023

Agenda item:	8.11.4

Source:	MediaTek Inc.

Title:	Simulation results of UE demodulation requirements for intra-band non-collocated NR-CA deployment scenario

Document for:	Discussion

# Introduction

In RAN4#108-bis, the Way Forward document for demodulation requirements was approved [1]. In RAN4#107 it was agreed that RAN4 should discuss the need for demodulation requirements only for Type-2 UEs and preclude Type-3a/3b and Type-4a/4b UE from the UE demodulation performance requirements discussion.  Furthermore, for Type-2 UE requirements are agreed to be defined in NR-CA scenario only. We provide simulation results for the Rel-18 work in this contribution by repeating results we already submitted for RAN4#108-bis meeting.

# Simulation results

In this chapter we present simulation results.

We have simulated static channel 2x2 simulations with all fixed MCS options in MCS tables 1 and 2 with Rank 1 and Rank 2. This gives us 4 independent configuration options for PCell and SCell.

- Rank 1 with MCS table 1 (64-QAM table)

- Rank 1 with MCS table 2 (256-QAM table)

- Rank 2 with MCS table 1 (64-QAM table)

- Rank 2 with MCS table 2 (256-QAM table)

Table 1 lists required SNR to achieve 70% of maximum throughput for every MCS in all previously listed 4 configurations. For Type 2 UE testing it was agreed to assume 25dB power difference between carriers. Therefore, we need to find suitable MCS pairs to achieve approximately 25dB difference.

Table 2 lists fixed MCS mapping between Rank 1 MCS Table 1 to Rank 2 MCS Table 2 with corresponding SNR differences.

Furthermore, Table 3 lists fixed MCS mapping between Rank 1 MCS Table 2 to Rank 2 MCS Table 2 with corresponding SNR differences.

| MCS index | Rank 1 MCS Table 1 | Rank 1 MCS Table 2 | Rank 2 MCS Table 1 | Rank 2 MCS Table 2 |
| --- | --- | --- | --- | --- |
| MCS 0 | -6.5 dB | -6.5 dB | -3.8 dB | -3.8 dB |
| MCS 1 | -5.7 dB | -5.0 dB | -2.7 dB | -2.1 dB |
| MCS 2 | -5.0 dB | -3.0 dB | -2.1 dB | -0.1 dB |
| MCS 3 | -3.9 dB | -1.1 dB | -1.0 dB | 1.9 dB |
| MCS 4 | -3.0 dB | 0.7 dB | -0.1 dB | 3.7 dB |
| MCS 5 | -2.0 dB | 2.6 dB | 1.0 dB | 5.7 dB |
| MCS 6 | -1.1 dB | 3.6 dB | 1.9 dB | 6.5 dB |
| MCS 7 | -0.2 dB | 4.5 dB | 2.8 dB | 7.5 dB |
| MCS 8 | 0.7 dB | 5.5 dB | 3.7 dB | 8.5 dB |
| MCS 9 | 1.5 dB | 6.3 dB | 4.7 dB | 9.3 dB |
| MCS 10 | 2.1 dB | 7.0 dB | 5.0 dB | 9.9 dB |
| MCS 11 | 2.6 dB | 8.3 dB | 5.7 dB | 11.3 dB |
| MCS 12 | 3.6 dB | 9.3 dB | 6.5 dB | 12.3 dB |
| MCS 13 | 4.5 dB | 10.0 dB | 7.5 dB | 13.3 dB |
| MCS 14 | 5.5 dB | 10.9 dB | 8.5 dB | 13.9 dB |
| MCS 15 | 6.3 dB | 12.0 dB | 9.3 dB | 15.0 dB |
| MCS 16 | 7.0 dB | 12.9 dB | 9.9 dB | 15.9 dB |
| MCS 17 | 7.6 dB | 14.1 dB | 10.5 dB | 17.1 dB |
| MCS 18 | 8.3 dB | 14.8 dB | 11.3 dB | 17.9 dB |
| MCS 19 | 9.3 dB | 15.9 dB | 12.3 dB | 18.9 dB |
| MCS 20 | 10.0 dB | 17.1 dB | 13.3 dB | 20.1 dB |
| MCS 21 | 10.9 dB | 17.5 dB | 13.9 dB | 20.5 dB |
| MCS 22 | 12.0 dB | 18.4 dB | 15.0 dB | 21.5 dB |
| MCS 23 | 12.9 dB | 19.9 dB | 15.9 dB | 22.9 dB |
| MCS 24 | 14.1 dB | 20.7 dB | 17.1 dB | 23.7 dB |
| MCS 25 | 14.8 dB | 21.6 dB | 17.9 dB | 24.7 dB |
| MCS 26 | 15.9 dB | 22.6 dB | 18.9 dB | 25.7 dB |
| MCS 27 | 17.1 dB | 24.0 dB | 20.1 dB | 27.0 dB |
| MCS 28 | 17.8 dB |  | 20.8 dB |  |

Table 1: Simulation results of fixed MCS in Rank 1 and Rank 2 with MCS tables 1 and 2.

| MCS index | Rank 1 MCS Table 1 | MCS index | Rank 2 MCS Table 2 | SNR difference |
| --- | --- | --- | --- | --- |
| MCS 0 | -6.5dB | MCS 19 | 18.9dB | 25.4dB |
| MCS 1 | -5.7dB | MCS 19 | 18.9dB | 24.6dB |
| MCS 2 | -5.0dB | MCS 20 | 20.1dB | 25.1dB |
| MCS 3 | -3.9dB | MCS 22 | 21.5dB | 25.4dB |
| MCS 4 | -3.0dB | MCS 22 | 21.5dB | 24.5dB |
| MCS 5 | -2.0dB | MCS 23 | 22.9dB | 24.9dB |
| MCS 6 | -1.1dB | MCS 24 | 23.7dB | 24.8dB |
| MCS 7 | -0.2dB | MCS 25 | 24.7dB | 24.9dB |
| MCS 8 | 0.7dB | MCS 26 | 25.7dB | 25.0dB |
| MCS 9 | 1.5dB | MCS 27 | 27.0dB | 25.5dB |
| MCS 10 | 2.1dB | MCS 27 | 27.0dB | 24.9dB |

Table 2: Fixed MCS mapping between Rank 1 MCS Table 1 to Rank 2 MCS Table 2.

| MCS index | Rank 1 MCS Table 2 | MCS index | Rank 2 MCS Table 2 | SNR difference |
| --- | --- | --- | --- | --- |
| MCS 0 | -6.5dB | MCS 19 | 18.9dB | 25.4dB |
| MCS 1 | -5.0dB | MCS 20 | 20.1dB | 25.1dB |
| MCS 2 | -3.0dB | MCS 22 | 21.5dB | 24.5dB |
| MCS 3 | -1.1dB | MCS 24 | 23.7dB | 24.8dB |
| MCS 4 | 0.7dB | MCS 26 | 25.7dB | 25.0dB |
| MCS 5 | 2.6dB | MCS 27 | 27.0dB | 24.4dB |

Table 3: Fixed MCS mapping between Rank 1 MCS Table 2 to Rank 2 MCS Table 2.

# Conclusion

In this paper we provided simulation results to the UE demodulation requirements for intra-band non-collocated NR-CA deployment scenario for Type-2 UE.

# Reference

- R4-2316917, “WF for NonCol_intraB_ENDC_NR_CA_Demod”, Ericsson