3GPP TSG-RAN WG4 Meeting #109		R4-2318647

# Chicago, US, November 13 – 17, 2023

Agenda item:	8.8.2.2

Source:	Apple

Title:	On UE capability for FR2 SCell activation

Document for:	Discussion

# Introduction

In last RAN4#108bis meeting, the WF of eFeRRM has been agreed in [1]. In this paper, we further discuss the remaining issues for FR2 SCell activation enhancement.

# Discussion

UE capability of enhancement L3 report after SCell activation command

The options we discussed in last meeting were:

Issue 3-1-1: UE capability of enhancement L3 report after SCell activation command - Agreement: - this feature shall be defined in one single feature group (FR) of feature list and it’s optional with capability signaling Issue 3-1-1a: if option 1 is adopted in issue 3-1-1, discuss the following issues - FFS: - Proposal 1 (CMCC): - If it is agreed that the L3 measurement report after SCell activation command, and using SSB periodicity instead of SMTC periodicity are applicable for FR1 SCell activation delay reduction, it is proposed that the related UE capabilities are applied for both FR1 and FR2. - Proposal 2 (HW, Apple): - For L3 measurement reporting triggered by SCell activation, introduce per UE capability indication. - Proposal 3 (OPPO): - Two options to define UE capability for FR2 unknown SCell activation enhancement: - Option 1: introduce UE capability for support the feature of FR2 unknown SCell activation enhancement at least including new L3 report, enhanced SCell activation delay and faster beam sweeping as one whole package. - Option 2: include FR2 unknown SCell activation enhancement as part of R18 RRM enhancement in UE feature list. - Proposal 4 (ZTE): - such UE capability is common for the single SCell activation, single PUCCH SCell activation and multiple SCell activation.

As agreed in last meeting, the L3 report during unknown SCell activation can be used for both FR1 and FR2, and therefore the per-UE capability can be assumed for such feature. Since it’s a fundamental enhancement for SCell activation, we think it shall be applied for single SCell activation, single PUCCH SCell activation, and multiple SCell and PUCCH SCell activation.

Proposal 1: UE capability of enhancement L3 report after SCell activation command shall be:

- per-UE capability

- no difference between FR1 and FR2, and between TDD and FDD

- appliable for single SCell activation, single PUCCH SCell activation, and multiple SCell and PUCCH SCell activation

UE capability of beam sweeping factor reduction for L3 and L1 (X1 and X2)

The options we discussed in last meeting were:

Issue 3-1-2: capability of beam sweeping factor reduction for L3 and L1 (X1 and X2) - Agreement: - this feature (X1/X2) shall be defined in one single feature group (FR) of feature list - define UE capability of beam sweeping factor reduction for L3 and L1 (X1 and X2) - It’s Optional with capability signaling. Issue 3-1-2a: if option 1 is adopted in issue 3-1-2, discuss the following issues - FFS - Proposal 1: - Option 1(MTK, CMCC, CTC, ZTE, Apple): Two beam sweeping factor reduction capability for L3 and L1, respectively, to indicate beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement. - Option 2(Ericsson, vivo): Beam sweeping factor reduction to be a single capability and if UE supports this capability, UE should report X1 and X2. - Proposal 2 (HW, Apple): - For X1 and X2, introduce per band capability indication

As agreed in last meeting, this feature (X1/X2) shall be defined in one single feature group (FR) of feature list. And since RAN4 agreed that separated beam sweeping factors will be introduced, i.e., beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement, we will have two IEs in this single FR to reflect X1 and X2 respectively. Moreover, the beam performance could be different on different bands, e.g., higher frequency and low frequency may have a bit different beam implementation, but it only applies for FR2 band. Thus, we think per band capability indication shall be used for both X1 and X2.

Proposal 2: UE capability of beam sweeping factor reduction for L3 and L1 (X1 and X2) shall be:

- per-band capability

- only appliable for FR2, and TDD only

- Two beam sweeping factor reduction capability for L3 and L1, respectively, to indicate beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement.

UE capability of “using SSB periodicity instead of SMTC” and “perform L1-RSRP measurement in non-DRX mode even DRX is configured”

The options we discussed in last meeting were:

Issue 3-1-3: capability of “using SSB periodicity instead of SMTC” and “perform L1-RSRP measurement in non-DRX mode even DRX is configured” - Agreement: - “using SSB periodicity instead of SMTC” and “perform L1-RSRP measurement in non-DRX mode even DRX is configured” shall be defined in one single feature group (FR) of feature list - Define this feature as “optional with capability signaling”

Since this feature is a small enhancement and not differentiated between FR1 and FR2, we propose to have per-UE capability.

Proposal 3: UE capability of “using SSB periodicity instead of SMTC” and “perform L1-RSRP measurement in non-DRX mode even DRX is configured” shall be:

- per-UE capability

- no difference between FR1 and FR2, and between TDD and FDD

Based on the above analysis, the UE feature list for SCell activation enhancement is proposed in annex.

# Conclusion

In this paper, we further discuss the remaining issues for FR2 SCell activation enhancement.

Proposal 1: UE capability of enhancement L3 report after SCell activation command shall be:

- per-UE capability

- no difference between FR1 and FR2, and between TDD and FDD

- appliable for single SCell activation, single PUCCH SCell activation, and multiple SCell and PUCCH SCell activation

Proposal 2: UE capability of beam sweeping factor reduction for L3 and L1 (X1 and X2) shall be:

- per-band capability

- only appliable for FR2, and TDD only

- Two beam sweeping factor reduction capability for L3 and L1, respectively, to indicate beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement.

Proposal 3: UE capability of “using SSB periodicity instead of SMTC” and “perform L1-RSRP measurement in non-DRX mode even DRX is configured” shall be:

- per-UE capability

- no difference between FR1 and FR2, and between TDD and FDD

# References

[1] R4-2317350, WF on NR eFeRRM (part1), Apple, RAN4 #108bis

# Annex: NR_RRM_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_RRM_enh3 | 31-1 | L3 measurement report for unknown SCell activation | Support of reporting valid L3 measurement results for the target being-activated SCell after receiving the SCell activation command |  | Yes | N/A | UE does not support reporting valid L3 measurement results after receiving the SCell activation command | Per UE | No | No | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. | Optional with capability signaling |
| 31. NR_RRM_enh3 | 31-2 | Beam sweeping factor reduction for FR2 unknown SCell activation | Support of reducing beam sweeping factor for cell detection if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure Support of reducing beam sweeping factor for SSB based L1-RSRP measurement if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure |  | Yes | N/A | UE does not support beam sweeping factor reduction for cell detection during FR2-1 unknown SCell activation. UE does not support beam sweeping factor reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation. | Per Band | N/A | FR2 only | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. Candidate values for beam sweeping reduction for cell detection during FR2-1 unknown SCell activation are 1,2,4, or 6. [Agreed in WF R4-2310081] Candidate values for beam sweeping reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation are 0,1,2,3,4,5,6, or 7. [Agreed in WF R4-2310081] | Optional with capability signaling |
| 31. NR_RRM_enh3 | 31-3 | Other measurement enhancement for unknown SCell activation | Support of using SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in measurement object for enhanced unknown SCell activation requirement. Support of performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation |  | Yes | N/A | UE does not use SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in MO for enhanced unknown Scell activation requirement. UE does not support performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation | Per UE | No | No | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. | Optional with capability signaling |