3GPP TSG RAN WG#4 Meeting #109                                                        R4-2318711

Chicago, USA, Nov 13th – 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 38.101-1 on system parameters for supporting enhanced channel raster |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_channel_raster_enh-Core |  |  |  |  |  | Date: |  |  | 2023-10-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New channel raster (10kHz) agreed for bands below 3GHz and with 100kHz channel raster. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding 10kHz new channel raster to adjacent channel spacing - Adding 10kHz new channel raster to the description and table for channel raster entries - Adding 10kHz new channel raster to nominal channel spacing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The corresponding feature is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.1, 5.4.2.3, 5.4A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change 1/3 >>

## 5.4	Channel arrangement

### 5.4.1	Channel spacing

#### 5.4.1.1	Channel spacing for adjacent NR carriers

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent NR carriers is defined as following:

-	For NR operating bands with 10kHz or 100 kHz channel raster,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

-	For NR operating bands with 15 kHz channel raster,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-5 kHz, 0 kHz, 5 kHz} for ∆FRaster equals 15 kHz

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-10 kHz, 0 kHz, 10 kHz} for ∆FRaster equals 30 kHz

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NR carriers. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario.

## << End change 1/3>>

## << Start of change 2/3>>

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NR operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

For NR operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <20>.

For NR operating bands with 10 kHz channel raster, ΔFRaster = 2 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <2>.

For NR operating bands with 15 kHz channel raster below 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {3,6}. Every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as < I >.

For NR operating bands with 15 kHz channel raster above 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {1,2}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <I>.

In frequency bands with two or more ΔFRaster, the higher ΔFRaster: For 15 kHz and 30 kHz channel raster applies to channels using only the SCS that is equal to or larger than the higher ΔFRaster and SSB SCS is equal to the higher ∆FRaster.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20 or 25> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20 or 25> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20 or 25> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20 or 25> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20 or 25> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20 or 25> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20 or 25> – 143200 | 145800 – <20> – 149200 |
| n13 | 100 | 155400 – <20 or 25> – 157400 | 149200 – <20> – 151200 |
| n14 | 100 | 157600 – <20 or 25> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20 or 25> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20 or 25> – 172400 | 158200 – <20> – 164200 |
| n24 | 100 | 325300 – <20 or 25> – 332100 | 305000 – <20> – 311800 |
| n25 | 100 | 370000 – <20 or 25> – 383000 | 386000 – <20> – 399000 |
| n26 | 100 | 162800 – <20 or 25> – 169800 | 171800 – <20> – 178800 |
| n28 | 100 | 140600 – <20 or 25> – 149600 | 151600 – <20> – 160600 |
| n29 | 100 | N/A | 143400 – <20> – 145600 |
| n30 | 100 | 461000 – <20 or 25> – 463000 | 470000 – <20> – 472000 |
| n34 | 100 | 402000 – <20 or 25> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20 or 25> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20 or 25> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20 or 25> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n462 | 15 | 743334 – <1> – 795000 | 743334 – <1> – 795000 |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20 or 25> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20 or 25> – 286400 | 285400 – <20> – 286400 |
| n53 | 100 | 496700 – <20 or 25> – 499000 | 496700 – <20> – 499000 |
| n54 | 100 | 334000 – <20 or 25> – 335000 | 334000 – <20> – 335000 |
| n65 | 100 | 384000 – <20 or 25> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20 or 25> – 356000 | 422000 – <20> – 440000 |
| n67 | 100 | N/A | 147600 – <20> – 151600 |
| n70 | 100 | 339000 – <20 or 25> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20 or 25> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20 or 25> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20 or 25> – 357000 | N/A |
| n81 | 100 | 176000 – <20 or 25> – 183000 | N/A |
| n82 | 100 | 166400 – <20 or 25> – 172400 | N/A |
| n83 | 100 | 140600 – <20 or 25> –149600 | N/A |
| n84 | 100 | 384000 – <20 or 25> – 396000 | N/A |
| n85 | 100 | 139600 – <20 or 25> – 143200 | 145600 – <20> – 149200 |
| n86 | 100 | 342000 – <20 or 25> – 356000 | N/A |
| n89 | 100 | 164800 – <20 or 25> – 169800 | N/A |
| n90 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20 or 25> – 538000 | 499200 – <20> – 538000 |
| n91 | 100 | 166400 – <20 or 25> – 172400 | 285400 – <20> – 286400 |
| n92 | 100 | 166400 – <20 or 25> – 172400 | 286400 – <20> – 303400 |
| n93 | 100 | 176000 – <20 or 25> – 183000 | 285400 – <20> – 286400 |
| n94 | 100 | 176000 – <20 or 25> – 183000 | 286400 – <20> – 303400 |
| n95 | 100 | 402000 – <20 or 25> – 405000 | N/A |
| n963 | 15 | 795000 – <1> – 875000 | 795000 – <1> – 875000 |
| n97 | 100 | 460000 – <20 or 25> – 480000 | N/A |
| n98 | 100 | 376000 – <20 or 25> – 384000 | N/A |
| n99 | 100 | 325300 – <20 or 25> – 332100 | N/A |
| n100 | 100 | 174880 – <20 or 25> – 176000 | 183880 – <20> – 185000 |
| n101 | 100 | 380000 – <20 or 25> – 382000 | 380000 – <20> – 382000 |
| n1024 | 15 | 795000 – <1> – 828333 | 795000 – <1> – 828333 |
| n104 | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |
| n105 | 100 | 132600 – <20 or 25> – 140600 | 122400 – <20> – 130400 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. NOTE 2: The following NREF are allowed for operation in Band n46: see Table 5.4.2.3-2. NOTE 3: The following NREF are allowed for operation in Band n96: see Table 5.4.2.3-3. NOTE 4: The following NREF are allowed for operation in Band n102: see Table 5.4.2.3-4. NOTE 5: Only applicable for UE indicating a capability [enhancedChannelRaster-r18] to support enhanced channel raster for the corresponding band. |  |  |  |

## << End change 2/3>>

## << Start of change 3/3>>

## 5.4A	Channel arrangement for CA

### 5.4A.1	Channel spacing for CA

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent NR component carriers is defined as the following unless stated otherwise:

For NR operating bands with a 100 kHz channel raster:

For NR operating bands with a 10 kHz channel raster:

Nominal channel spacing=BWChannel(1)+BWChannel(2)-2GBChannel(1)-GBChannel(2)0.30.15 [MHz]

while for NR operating bands without a 10k or 100 kHz channel raster:

with

n = µ0

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NR component carriers according to Table 5.3.2-1 with values in MHz, μ0  is the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1 and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for the said μ value with μ as defined in TS 38.211. In case there is no common μ value for both of the channel bandwidths, μ0=1 is selected and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for μ=1 with μ as defined in TS 38.211.

The channel spacing for intra-band contiguous carrier aggregation can be adjusted to any multiple of least common multiple of channel raster and sub-carrier spacing less than the nominal channel spacing to optimize performance in a particular deployment scenario.

For intra-band non-contiguous carrier aggregation, the channel spacing between two NR component carriers in different sub-blocks shall be larger than the nominal channel spacing defined in this clause.

## << End change 3/3>>