# 3GPP TSG-RAN WG4 Meeting # 109	R4-2318916

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | Draft | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.106 to introduce conducted transmitter requirement for NCR-MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2023-11-03> |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | to introduce conducted transmitter requirement for NCR-MT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | to introduce conducted transmitter requirement for NCR-MT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | conducted transmitter requirement is missing for NCR-MT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2, 6.5.2, 6.5.3, 6.5.4, 6.11, 6.12, 6.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.115-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

6	Conducted characteristics

6.1	General

6.2	Repeater output power

6.2.1	General

6.2.2	Minimum requirement  for NR Repeater

6.2.3	Minimum requirement for NCR

6.2.3.1	Minimum requirement for NCR-Fwd

#### 6.2.3.2	Minimum requirement for NCR-MT

6.2.3.2.1	General

The NCR-MT conducted output power requirement is at antenna connector for 1-C, or at TAB connector for type 1-H.

The rated carrier output power of the NCR-MT type 1-C shall be as specified in table 6.2.3.2.1-1.

Table 6.2.3.2.1-1: NCR-MT type 1-C UL transmission classes rated output power limits

| BS class | Prated,c,AC |
| --- | --- |
| Wide Area NCR-MT | (Note) |
| Local Area NCR-MT | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,c,AC rated output power of the Wide Area NCR-MT. |  |

The rated carrier output power of the NCR-MT type 1-H shall be as specified in table 6.2.3.2.1-2.

Table 6.2.3.2.1-2: NCR-MT type 1-H UL transmission classes rated output power limits

| BS class | Prated,c,sys | Prated,c,TABC |
| --- | --- | --- |
| Wide Area NCR-MT | (Note 1) | (Note 1) |
| Local Area NCR-MT2 | ≤ 24 dBm +10log(NTXU,counted), where NTXU,counted = min(NTXU,active ,4) | ≤ 24 dBm |
| NOTE 1: There is no upper limit for the Prated,c,AC rated output power of the Wide Area NCR-MT. NOTE 2: LA MT cannot exceed highest power class for that band as specified in TS 38.101-1 |  |  |

6.2.3.2.2	Minimum requirement for NCR-MT type 1-C and NCR-MT type 1-H

In normal conditions, Pmax,c,AC shall remain within +2 dB and -2 dB of the rated carrier output power Prated,c,AC, declared by the manufacturer.

In extreme conditions, Pmax,c,AC shall remain within +2.5 dB and -2.5 dB of the rated carrier output power Prated,c,AC, declared by the manufacturer.

<<next Change>>

6.5.2	Adjacent Channel Leakage Power Ratio

6.5.2.1	General

6.5.2.2	Minimum requirements for NR Repeater

6.5.2.3     Minimum requirement for NCR

6.5.2.3.1     Minimum requirement for NCR-Fwd

6.5.2.3.2    Minimum requirement for NCR-MT type 1-C and type 1-H

For WA NCR-MT, the BS ACLR requirements specified in clause 6.6.3 and 6.6.4 in TS 38.104 apply. For Absolute ACLR requirements, following scaling factor should be added on on top of 1-C requirements.

10log(NTXU,counted), where NTXU,counted = min(NTXU,active ,8)

For LA NCR-MT, the UE ACLR requirements specified in clause 6.5.2.5 in TS 38.101-1 apply

6.5.3	Operating band unwanted emissions

6.5.3.1	General

6.5.3.2	Minimum requirements for NR Repeater

6.5.3.3	Minimum requirement for NCR

6.5.3.3.1     Minimum requirement for NCR-Fwd

6.5.3.3.2    Minimum requirement for NCR-MT

For 1-C and 1-H  LA NCR-MT, whether FWD is transmiting simultaneously or not, the UE spectrum emission mask requirements and additional spectrum emission mask requirements specified in clause 6.5.2 in TS 38.101-1 applies  .

For 1-C WA NCR-MT, when NCR-MT and NCR-fwd are not transmting simultaneously, the BS requirements specified in clause in TS 38.104 apply. And for 1-H WA, following scaling factor should be added on top of 1-C requirements.

10log(NTXU,counted), where NTXU,counted = min(NTXU,active ,8)

6.5.4	Transmitter spurious emissions

6.5.4.1	General

6.5.4.2	Minimum requirements for NR Repeater

6.5.4.3	Minimum requirements for NCR

6.5.4.3.1     Minimum requirement for NCR-Fwd

6.5.4.3.2    Minimum requirement for NCR-MT

When NCR-MT and NCR-fwd are not transmting simultaneously, the requirements in clause 6.5.4.2 applies for WA 1-C NCR-MT and the requirements in clause 6.5.3 in TS 38.101-1 applies for LA 1-C and 1-H NCR-MT. For 1-H WA NCR-MT, following scaling factor should be added on top of 1-C requirements

10log(NTXU,counted), where NTXU,counted = min(NTXU,active ,8)

When NCR-MT and NCR-fwd are transmting simultaneously, the requirements in clause 6.5.3 in TS 38.101-1 applies for LA 1-C and 1-H NCR-MT.

<<next Change>>

## 6.11	Output power dynamics

6.11.1 General

Output power dynamics is specified in terms of: OFF power, ON/OFF time mask and power control requirements.

6.11.2	Transmit OFF power for NCR-MT

For WA 1-C and 1-H NCR-MT, the BS requirements specified in clause 6.4.1.2 in TS 38.104  applies.

For LA 1-C and 1-H NCR-MT,  the UE requirement specified in clause 6.3.2  in TS 38.101-1 applies.

- 6.11.3	Transmit ON/OFF time mask for NCR-MT

For WA 1-C and 1-H NCR-MT, The BS requirements specified in clause 6.4.2.2 in TS 38.104 applies .

For LA 1-C and 1-H NCR-MT, The UE requirements specified in clause 6.3.3 in TS 38.101-1 applies.

- 6.11.4	Power control for NCR-MT

For WA NCR-MT, The IAB-MT requirements specified in clause 6.3.2 output dynamic range requirement, 6.3.3.1 relative power tolerance and 6.3.3.2 aggregated power tolerance requirements in TS 38.174 applies.

For LA NCR-MT, The UE requirements specified in clause 6.3.4.3 of relative power tolerance and clause 6.3.4.4 of aggregate  power tolerance in TS 38.101-1 applies.

6.12	Transmit signal quality

6.12.1	General

Transmit signal quality is specified in terms of: frequency error and transmit modulation quality requirements.

6.12.2	Frequency error requirements for NCR-MT

The IAB-MT requirements specified in clause 6.5.1.2 in TS 38.174 apply to NCR-MT.

6.12.3	Transmit modulation quality

The IAB-MT requirements specified in clause 6.5.2.2 in TS 38.174 apply to NCR-MT.

6.13	Transmit intermodulation

6.13.1	General

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

NCR-MT transmit intermodulation is defined by the ratio of the mean power of the wanted signal to the mean power of the intermodulation product when an interfering signal is added at a level below the wanted signal at each transmitter antenna port with the other antenna port(s) if any terminated.

6.13.2	minimum requirements for NCR-MT

The Tx IMD requirement for IAB-MT specified in clause 6.7 of Rel-16 TS 38.174 apply for WA and LA NCR-MT. The IM interference level is based on NCR-Fwd link.

<<End of Change>>