3GPP TSG RAN WG4 Meeting #109		R4-2319109

Chicago, USA, November 13 – 17th 2023

Agenda item:	8.16.3

Source:	Apple

Title:	On FR1 MIMO OTA noise evaluation

WI/SI:	NR_MIMO_OTA_enh-Core

Release:	Rel-18

Document for:	Discussion

# 1	Introduction

At RAN4 #108 Apple’s lab  fulfilled the requirements to be consider a volunteer lab for the second phase (low frequency) on FR1 MIMO OTA Lab alignment [1].

In previous meeting (RAN4 #106) it was agreed that FR1 MIMO OTA volunteer labs should exclude the impact of measurement noise prior to perform MIMO measurements [2]

# 3	Measurement results

Following similar test procedure as defined on [3 and 4], however with 10dB attenuation only. The following results were achieved at band n28:

RS EPRE  between FR1 MIMO OTA system with and without 10dB attenuators @ 50% throughput = 0.59 dB

RS EPRE  between FR1 MIMO OTA system with and without 10dB attenuators @ 70% throughput = 0.66 dB

RS EPRE  between FR1 MIMO OTA system with and without 10dB attenuators @ 90% throughput = 0.67 dB

# 4	Proposal

Proposal 1		RAN4 to accept the measurement results presented in this contribution as an evidence of Apple’s FR1 MIMO OTA lab compliance on low frequency noise mitigation evaluation.

# 5	References

[1] 		R4-2313900, WF for Rel-18 MIMO OTA, CAICT

[2]		R4-2302975, WF for Rel-18 MIMO OTA, CAICT

[3]		R4-2301049, On noise impact and FR1 lab alignment, CAICT

[4]		R4-2316227, Noise impact of FR1 MIMO OTA for low band, OPPO