3GPP TSG-RAN WG4 Meeting # 109							R4-2319924

Chicago, US, November 13 – 17, 2023

Agenda Item:	8.30.2.1

Source: 	OPPO

Title: 	on the SL-U MRP simulation results

Document for:	Approval

- Introduction

The sidelink unlicensed MPR A-MPR simulation has been fully discussed in the last RAN4 meeting and the single carrier MPR for SL-U PSSCH/PSCCH has been agreed. Still the MPR for PSFCH and S-SSB, also the MPR for wide-band operation for SL-U PSSCH/PSCCH has still not been agreed yet. In this paper, we try to finalize these issues.

- Discussion

The SL-U MPR for PSSCH/PSCCH has been agreed as:

Agreement:

- Table: SL-U MPR requirement

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) |

| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 |
| --- | --- | --- | --- |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | TBD | TBD |
| - NOTE 1: The average method will be used to derive the MPR values - NOTE 2: The wide band operation MPR will be further discussed next meeting. |  |  |  |

Firstly, it has been agreed that the averaged method will be used to derive the MPR values. In such case, we have proposed in the last meeting with 7.5dB as the final number. However, still some companies would like to further contribute on the MPR simulation and also try to align more on the numbers so we will not propose the number at this stage but after the tdoc submission we try to provide an average number in the thread summary for the whole group to check if the number is agreeable.

For the 2nd remaining issue as the wide band operation MPR, we would like to again show our simulation results as below.

Table 1 simulation cases

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 40 | 105RB0 | 30 |
|  | 5 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 6 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation Single CC | 8 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 9 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 10 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 11 | CP-OFDM | 40 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 13 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 14 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation contiguous RB allocation | 15 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 16 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 17 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 18 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 21 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 22 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 23 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 29 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 30 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 31 | CP-OFDM | 100 | Bitmap 00100 | 30 |
| Wide band operation Interlaced RB allocation | 32 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 33 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 34 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 35 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 36 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 37 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 38 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 39 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 40 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 41 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 42 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 43 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 44 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 45 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 46 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 47 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 48 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Table 2 MPR for Wideband operation

|  | Bitmap | 10 | 100 | 1000 | 10000 | 010 | 0100 | 01000 | 00100 | 110 | 1100 | 11000 | 0110 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Figure 1 MPR for wide band operation

If we look into figure 1 above, clearly it has shown that for interlaced RB allocation starting from case 32 to 48, the MPR for all the modulation are lower than the corresponding bitmap but with full RB allocation. Also for full RB allocation case 18, 21, 23, 27, 30 and 31, the MPR for 256QAM are about 1dB higher than other cases for 256QAM full RB allocation.

Observation 1: For interlaced RB allocation starting from case 32 to 48, the MPR for all the modulation are lower than the corresponding bitmap but with full RB allocation

Observation 2: For full RB allocation case 18, 21, 23, 27, 30 and 31, the MPR for 256QAM are about 1dB higher than other cases for 256QAM full RB allocation.

From the observation 1 and 2 above, we try to differentiate the MPR for full RB allocation and interlaced RB allocation as NR-U has done. Also for the specific cases 18, 21, 23, 27, 30 and 31, we try to introduce the exception bitmap as also NR-U has done.

Proposal 1: Differentiate the MPR for full RB allocation and interlaced RB allocation as NR-U has done.

Proposal 2: For the specific cases 18, 21, 23, 27, 30 and 31, introduce the exception bitmap as NR-U has done.

During the last meeting discussion, there is company propose to further differentiate the bitmap for inner allocation and outer allocation, here we list the results from different companies for comparison to these “inner and outer bitmaps”.

Table 3 Simulation result comparison for inner bitmaps

|  | LGE full | LGE Interlaced | OPPO Full | OPPO Interlaced | Qualcomm Full | Qualcomm Interlaced |
| --- | --- | --- | --- | --- | --- | --- |
| 010 | 2.13 | 0.00 | 2.8 | 2.6 | NA | NA |
|  | 2.13 | 0.36 | 3.6 | 3.0 | NA | NA |
|  | 3.16 | 2.45 | 4.7 | 4.2 | NA | NA |
|  | 5.88 | 5.46 | 7.3 | 6.8 | NA | NA |
| 0110 | 2.13 | 0.00 | 2.8 | 2.7 | 1.3 | 1.6 |
|  | 2.13 | 0.63 | 3.5 | 3.0 | 1.3 | 1.6 |
|  | 3.16 | 2.46 | 4.8 | 4.1 | 3.5 | 4.0 |
|  | 5.88 | 5.06 | 7.6 | 6.7 | 5.9 | 6.5 |
| 0100/0010 | 2.46 | 0.00 | 2.8 | 2.6 | 1.3 | 1.4 |
|  | 2.46 | 0.10 | 3.5 | 2.9 | 1.3 | 1.4 |
|  | 3.15 | 2.12 | 5.1 | 4.1 | 3.6 | 3.8 |
|  | 5.46 | 5.05 | 7.4 | 6.6 | 6.0 | 6.1 |
| 01110 | 2.46 | 0.13 | 2.8 | 2.8 | 1.5 | 1.6 |
|  | 2.47 | 0.64 | 3.7 | 3.0 | 1.5 | 1.6 |
|  | 3.16 | 2.47 | 5.0 | 4.1 | 3.8 | 4.0 |
|  | 5.47 | 5.06 | 8.5 | 6.7 | 6.5 | 6.6 |
| 01100/00110 | 2.46 | 0.00 | 2.8 | 2.7 | 1.4 | 1.6 |
|  | 2.46 | 0.63 | 3.6 | 2.8 | 1.4 | 1.6 |
|  | 3.16 | 2.46 | 5.0 | 4.0 | 3.5 | 4.0 |
|  | 5.47 | 5.06 | 8.0 | 6.6 | 5.9 | 6.5 |
| 01000/00010 | 2.13 | 0.00 | 2.8 | 2.6 | NA | 1.4 |
|  | 2.13 | 0.35 | 3.6 | 3.0 | NA | 1.4 |
|  | 3.16 | 2.13 | 4.9 | 4.1 | NA | 3.8 |
|  | 5.47 | 5.06 | 8.0 | 6.7 | NA | 6.1 |
| 00100 | 2.13 | 0.00 | 2.8 | 2.6 | NA | NA |
|  | 2.13 | 0.10 | 3.5 | 2.9 | NA | NA |
|  | 3.15 | 2.12 | 5.1 | 4.1 | NA | NA |
|  | 5.46 | 5.06 | 7.5 | 6.6 | NA | NA |

From the table 3 above, we marked yellow for some of the MPR to be specifically as low as 0 dB for interlaced RB allocation in the 2nd column while compare to the regular MPR numbers for full RB allocation. Despite the NA from the last 2 columns, it can be found from both OPPO result as well as Qualcomm result, the difference between full RB allocation and interlaced RB allocation is within 0.6dB.

Observation 3: For some companies proposed inner bitmap, the difference between full RB allocation and partial RB allocation are as large as 2.46dB while for other companies results, the difference are within 0.6dB.

Based on the comparison above and the observation 3, it is clearly shown that the differentiation between inner and outer bitmap should not be applied.

Proposal 3: Not to apply the differentiation of inner and outer bitmap.

With the above proposal 1,2 and 3 we can try to propose the MPR for PSSCH and PSCCH with similar format as NR-U such as:

Table 9: NR-U MPR requirement

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |

| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 |
| --- | --- | --- | --- |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | TBD | TBD |

With Bitmap exception

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 40 | 10, 01 |
| 60 | 110, 011 |
| 80 | 1000, 1100, 0100, 0010, 0011, 0001 |
| 100 | 10000, 11000, 11100, 01000, 01100, 00110, 00010, 00111, 00011, 00001 |

How to apply the exception and further detail on the exception table can be further discussed based on the simulation results as submitted this meeting.

3	Conclusions

Observation 1: For interlaced RB allocation starting from case 32 to 48, the MPR for all the modulation are lower than the corresponding bitmap but with full RB allocation

Observation 2: For full RB allocation case 18, 21, 23, 27, 30 and 31, the MPR for 256QAM are about 1dB higher than other cases for 256QAM full RB allocation.

Observation 3: For some companies proposed inner bitmap, the difference between full RB allocation and partial RB allocation are as large as 2.46dB while for other companies results, the difference are within 0.6dB.

Proposal 1: Differentiate the MPR for full RB allocation and interlaced RB allocation as NR-U has done.

Proposal 2: For the specific cases 18, 21, 23, 27, 30 and 31, introduce the exception bitmap as NR-U has done.

Proposal 3: Not to apply the differentiation of inner and outer bitmap.

4 References

[1] TR 38.786