3GPP TSG-RAN WG4 Meeting # 109															R4-2319928

# Chicago, US, November 13 – 17, 2023

Source: 	OPPO

Title: 	TP on SL-U MPR and A-MPR

Agenda Item:	8.30.2.2

Document for:	Approval

Introduction

In this meeting, it is proposed to settle down the LTE and NR co-existence, In this TP, we capture the discussion and agreement on LTE and NR co-existence to the TP 38.786.

# References

# Text proposal

## <<Start of Change for TR 38.786>>

- 8	Co-channel coexistence between LTE Sidelink and NR Sidelink

- 8.1.Configured transmitted power for V2X UE supporting co-channel coexistence with LTE SL

For the measured configured maximum output power PUMAX,c for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions, the same requirement as in clause 6.2.4 shall be applied. When NR V2X UE is configured to co-channel coexistence operation with LTE V2X and NR SCS is configured to 30kHz, the evaluation period for PUMAX,c for NR V2X sidelink is the first slot of NR SL slots overlapping with an LTE SL subframe.

- 8.2.	Relative slot power tolerance for V2X UE supporting co-channel coexistence with LTE SL

Relative slot power tolerance is defined for NR V2X supporting co-channel coexistence to ensure the RX performance of the LTE V2X UE operating within the same channel. This requirement applies only to transmissions with 30kHz SCS.

The relative slot power tolerance for V2X UE supporting co-channel coexistence with LTE SL is the ability of the NR V2X UE operating with 30kHz SCS to control the output power of transmitted slots during PSCCH/PSSCH transmission consisting of two slots overlapping with an LTE SL subframe (500us). The reference slot is the 1st slot overlapping with LTE SL subframe and target slot is the subsequent NR SL slot overlapping with the LTE SL subframe. The measurement period is one NR SL slot with guard symbol omitted.

Currently in RAN WG1 specification TS 38.213, it has been clearly stated that “For sidelink co-channel coexistence between E-UTRA and NR, and for NR PSCCH/PSSCH transmissions with SCS configuration μ=1 in slots that overlap with an E-UTRA subframe on the sidelink, the UE transmits NR PSCCH/PSSCH in the earlier overlapping slot with a power that is larger than or equal to the power in the later overlapping slot.” In such case, there is no need to define additional RAN4 UE RF requirement for this.

## <<End of Change>>