3GPP TSG-RAN WG4 Meeting # 109	R4-2320060
Chicago, US, November 13 – 17, 2023

Agenda item:	8.16.1

Source:	CAICT

Title:	TP to TR 38.761 on General Aspects and Measurement Setup

Document for:	Discussion

- Introduction

At RAN#101 plenary, the revised WID RP-232679 is approved and an internal TR38.761 is allocated to capture measurement of MIMO OTA performance for UEs.

In this contribution, we provide a TP on general aspects and measurement setup.

2	References

- RP-232679, Revised WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs, CAICT, 3GPP RAN#101

- 3GPP TR 38.761 v0.1.0

3	Text Proposal to TR 38.761

--------------Start of text proposal 1-------------

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.151: “NR; Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements for NR UEs”.

[3]	3GPP TR 38.827: “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[4]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[5]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[6]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[7]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements"

[8]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

[9]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment"

[10]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

[11]	F. Zhang, L. Hentilä, P. Kyösti and W. Fan, "Millimeter-wave New Radio Test Zone Validation for MIMO Over-the-air Testing," in IEEE Transactions on Antennas and Propagation, doi: 10.1109/TAP.2021.3111326.

--------------End of text proposal 1 -------------

--------------Start of text proposal 2-------------

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

FS: UE used in a free space configuration.

Handheld UE: A UE intended to be used in hand held scenario.

MIMO Average Spherical Coverage: An averaged sensitivity of best 18 FR2 MIMO OTA sensitivity values within the 3D sphere with constant-density points for PC3 device.

Primary mechanical mode: The mode that is most often used for a specific user scenario. Every terminal has at least one primary mechanical mode, if multiple modes are supported, different primary mechanical modes may be applicable for different user scenarios, e.g., different primary mechanical modes for Free Space and Hand phantom usage for the same UE.

PSP (PAS Similarity Percentage):  The similarity of the PAS produced by the OTA system and the reference PAS, which is presented by the Total Variation Distance (TVD) of power angular spectrum (PAS). PSP is defined as (1-TVD)*100%. PSP=100% denotes full similarity and PSP=0% denotes full dissimilarity.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AOA	Azimuth angle Of Arrival

AOD	Azimuth angle Of Departure

BS	Base Station

CDL	Clustered Delay Line

CW	Continuous Wave

DML	Data Mode Landscape

DMP	Data Mode Portrait

DMSU	Data Mode Screen Up

DUT	Device Under Test

EUT	Equipment Under Test

FR1	Frequency Range 1

FR2	Frequency Range 2

FS	Free Space

MASC	MIMO Average Spherical Coverage

MIMO	Multiple Input Multiple Output

MPAC	Multi-Probe Anechoic Chamber

NR	New Radio

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

OTA	Over The Air

PAS	Power Angular Spectrum

PDP	Power Delay Profile

PSP	PAS Similarity Percentage

RS-EPRE	Reference Signal-Energy Per Resource Element

SS 	System Simulator

SSS	Secondary Synchronization Signal

TRMS	Total Radiated Multi-antenna Sensitivity

UE	User Equipment

UMa	Urban Macro

UMi	Urban Micro

XPR	Cross-Polarization Ratio

ZOA	Zenith angle Of Arrival

ZOD	Zenith angle Of Departure

ZSA	Zenith angle Spread of Arrival

ZSD	Zenith angle Spread of Departure

--------------End of text proposal 2 -------------

--------------Start of text proposal 3-------------

4	General

## 4.1	Device types

The present technical report covers the smartphone device type.

## 4.2	Testing configuration

The present technical report covers the free space (FS) testing configuration.

## 4.3	Testing Bands

The present technical report covers both FR1 and FR2 operating bands. FR1 operating bands are defined in TS 38.101-1 [4] and FR2 operating bands are defined in TS 38.101-2 [5]. NSA band combinations are defined in TS 38.101-3 [6]. E-UTRA is designed to operate in operating bands defined in TS 36.101 [8].

--------------End of text proposal 3 -------------

--------------Start of text proposal 4-------------

# 5	Measurement setup

## 5.1	General

The measurements in the present technical report are based on the test methodology defined in 3GPP TS 38.151 [2] and 3GPP TR 38.827 [3].

## 5.2	Measurement setup for FR1 MIMO OTA

The multi-probe anechoic chamber (MPAC) test method is the reference methodology for FR NR MIMO OTA testing, the measurement setup and test procedure are defined in TS 38.151 [2].

## 5.3	Measurement setup for FR2 MIMO OTA

The 3D MPAC test method is the reference methodology for FR2 NR MIMO OTA testing, the measurement setup and test procedure are defined in TS 38.151 [2].

--------------End of text proposal 4 -------------