3GPP TSG-RAN4 Meeting #109		R4-2320686

# Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR |  | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on conditions for mIAB-MT in Annex H of TS 38.174 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Perf |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To define side conditions for mIAB-MT RRM requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | mIAB-MT side conditions in terms of SSB Es/Iot and min SSB_RP; and CSI-RS Es/Iot and minimum CSI-RS_RP are defined for following requirements for mIAB-MT class and all IAB types (1-H, 1-O and 2-O): - mIAB-MT RRC connection re-establishment requirements to intra-frequency cell. - mIAB-MT measurements on intra-frequency cells. - SSB based L1-RSRP measurement reporting by mIAB-MT. - CSI-RS based L1-RSRP measurement reporting by mIAB-MT. The conditions for the mIAB-MT are the same as defined for the local area (LA) IAB-MT. This is in accordance with the RF session agreement in the WF (R4-2316911) that the LA IAB-MT requirements also apply to mIAB-MT: - Add a new section in TS 38.174 to state that requirements for mIAB-MT, unless otherwise stated, follows Local area IAB-MT requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | mIAB-MT will not be able to meet core requirements for RRC re-establishment, intra-frequency measurements and L1-RSRP reporting due to the missing side conditons. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.1.1, H.1.1.1, H.1.1.4, H.1.1.5, H.1.1.5.1, H.1.1.5.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

## H.1	Conditions for RRC_CONNECTED state mobility for IAB-MT

### H.1.1	Introduction

In Annex H.1, the following conditions are specified:

-	IAB-MT conditions which shall apply for IAB-MT RRC Connection Re-establishment requirements for NR intra-frequency cells in clause 12.1.1.1.

-	IAB-MT conditions which shall apply for IAB-MT RRC Connection Re-establishment requirements for NR inter-frequency cells in clause 12.1.1.1.

-	IAB-MT conditions which shall apply for IAB-MT RRC Connection Release with Redirection requirements for NR cells in clause 12.1.1.3.

- mIAB-MT conditions which shall apply for mIAB-MT RRC Connection Re-establishment requirements for NR intra-frequency cells in clause 12.1.1.1A.

- mIAB-MT conditions which shall apply for mIAB-MT measurement requirements for NR intra-frequency cells in clause 12.4A.2 and

- mIAB-MT conditions which shall apply for mIAB-MT measurement requirements for L1-RSRP reporting in clause 12.4A.3.

#### H.1.1.1	Conditions for Measurements on NR Intra-frequency Cells for RRC Connection Re-establishment

This clause defines the following conditions in terms of SSB_RP and SSB Ês/Iot for measurements on NR intra-frequency cells for RRC connection re-establishment:

-	The conditions are defined in Table H.1.1.1-1 for FR1 NR cells for Wide Area IAB-MT and IAB Type 1-H.

-	The conditions are defined in Table H.1.1.1-2 for FR1 NR cells for Local Area IAB-MT or mIAB-MT and IAB Type 1-H.

-	The conditions are defined in Table H.1.1.1-3 for FR1 NR cells for Wide Area IAB-MT and IAB Type 1-O.

-	The conditions are defined in Table H.1.1.1-4 for FR1 NR cells for Local Area IAB-MT or mIAB-MT and IAB Type 1-O.

-	The conditions are defined in Table H.1.1.1-5 for FR2 NR cells for Local Area IAB-MT, Wide Area IAB-MT or mIAB-MT and IAB Type 2-O.

Table H.1.1.1-1: Conditions for RRC connection re-establishment for intra-frequency cell for Wide Area IAB-MT and IAB Type 1-H

| IAB-MT channel bandwidth (MHz) | SSB sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | SSB Ês/Iot (dB) | Minimum SSB_RP (dBm) |
| 10, 15 | 30 | -6 | -107 - 10*Log10(NPRB *12) |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | -6 | -101.4- 10*Log10(NPRB *12) |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. |  |  |  |

Table H.1.1.1-2: Conditions for RRC connection re-establishment for intra-frequency cell for Local Area IAB-MT or mIAB-MT and IAB Type 1-H

| IAB-MT channel bandwidth (MHz) | SSB sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | SSB Ês/Iot (dB) | Minimum SSB_RP (dBm) |
| 10, 15 | 30 | -6 | -99 - 10*Log10(NPRB *12) |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | -6 | -92.5- 10*Log10(NPRB *12) |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. |  |  |  |

Table H.1.1.1-3: Conditions for RRC connection re-establishment for intra-frequency cell for Wide Area IAB-MT and IAB Type 1-O

| IAB-MT channel bandwidth (MHz) | SSB sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | SSB Ês/Iot (dB) | Minimum SSB_RP (dBm) |
| 10, 15 | 30 | -6 | -107 - 10*Log10(NPRB *12) - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | -6 | -101.4- 10*Log10(NPRB *12) - ΔOTAREFSENS |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. |  |  |  |

Table H.1.1.1-4: Conditions for RRC connection re-establishment for intra-frequency cell for Local Area IAB-MT or mIAB-MT and IAB Type 1-O

| IAB-MT channel bandwidth (MHz) | SSB sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | SSB Ês/Iot (dB) | Minimum SSB_RP (dBm) |
| 10, 15 | 30 | -6 | -99 - 10*Log10(NPRB *12) - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | -6 | -92.5- 10*Log10(NPRB *12) - ΔOTAREFSENS |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. |  |  |  |

Table H.1.1.1-5: Conditions for RRC connection re-establishment for intra-frequency cell for Local Area IAB-MT, Wide Area IAB-MT or mIAB-MT and IAB Type 2-O

| IAB-MT channel bandwidth (MHz) | SSB sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | SSB Ês/Iot (dB) | Minimum SSB_RP (dBm) |
| 50 | 120 | -6 | EISREFSENS_50M - 10*Log10(NPRB *12) + ΔFR2_REFSENS -5 |
| 100, 200, 400 | 120 | -6 | EISREFSENS_50M - 10*Log10(NPRB *12) -2+ΔFR2_REFSENS |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. NOTE 2: EISREFSENS_50M for wide area IAB-MT, local area IAB-MT and mIAB-MT is defined in section 10.3.3.3. |  |  |  |

----------------------NEXT CHANGE----------------------------

#### H.1.1.4	Conditions for NR intra-frequency measurements

This clause defines the following conditions in terms of SSB_RP and SSB Ês/Iot for measurements on NR intra-frequency cells in clause 12.4A.2:

-	The conditions defined in Table H.1.1.1-2 shall also apply for measurements on NR intra-frequency cells in FR1 for mIAB-MT and IAB Type 1-H.

-	The conditions defined in Table H.1.1.1-4 shall also apply for measurements on NR intra-frequency cells in FR1 for mIAB-MT and IAB Type 1-O.

-	The conditions defined in Table H.1.1.1-5 shall also apply for measurements on NR intra-frequency cells in FR2 for mIAB-MT and IAB Type 2-O.

#### H.1.1.5	Conditions for L1-RSRP measurement reporting

#### H.1.1.5.1	Conditions for SSB based L1-RSRP measurement reporting

This clause defines the following conditions in terms of SSB_RP and SSB Ês/Iot for SSB based L1-RSRP measurement reporting in clause 12.4A.3:

-	The conditions are defined in Table H.1.1.5.1-1 for SSB based L1-RSRP measurement reporting in FR1 for mIAB-MT and IAB Type 1-H.

-	The conditions are defined in Table H.1.1.5.1-2 for SSB based L1-RSRP measurement reporting in FR1 for mIAB-MT and IAB Type 1-O.

-	The conditions are defined in Table H.1.1.5.1-3 for SSB based L1-RSRP measurement reporting in FR2 for mIAB-MT and IAB Type 2-O.

Table H.1.1.5.1-1: Conditions for SSB based L1-RSRP measurements for mIAB-MT and IAB Type 1-H

| IAB-MT channel bandwidth (MHz) | SSB sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | SSB Ês/Iot (dB) | Minimum SSB_RP (dBm) |
| 10, 15 | 30 | -3 | -96 - 10*Log10(NPRB *12) |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | -3 | -89.5- 10*Log10(NPRB *12) |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. |  |  |  |

Table H.1.1.5.1-2: Conditions for SSB based L1-RSRP measurements for mIAB-MT and IAB Type 1-O

| IAB-MT channel bandwidth (MHz) | SSB sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | SSB Ês/Iot (dB) | Minimum SSB_RP (dBm) |
| 10, 15 | 30 | -3 | -96 - 10*Log10(NPRB *12) - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | -3 | -89.5- 10*Log10(NPRB *12) - ΔOTAREFSENS |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. |  |  |  |

Table H.1.1.5.1-3: Conditions for SSB based L1-RSRP measurements for mIAB-MT and IAB Type 2-O

| IAB-MT channel bandwidth (MHz) | SSB sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | SSB Ês/Iot (dB) | Minimum SSB_RP (dBm) |
| 50 | 120 | -3 | EISREFSENS_50M - 10*Log10(NPRB *12) + ΔFR2_REFSENS -2 |
| 100, 200, 400 | 120 | -3 | EISREFSENS_50M - 10*Log10(NPRB *12) + ΔFR2_REFSENS +1 |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. NOTE 2: EISREFSENS_50M for mIAB-MT is defined in section 10.3.3.3. |  |  |  |

#### H.1.1.5.2	Conditions for CSI-RS based L1-RSRP measurement reporting

This clause defines the following conditions in terms of CSI-RS_RP and SSB Ês/Iot for SSB based L1-RSRP measurement reporting in clause 12.4A.3:

-	The conditions are defined in Table H.1.1.5.2-1 for CSI-RS based L1-RSRP measurement reporting in FR1 for mIAB-MT and IAB Type 1-H.

-	The conditions are defined in Table H.1.1.5.2-2 for CSI-RS based L1-RSRP measurement reporting in FR1 for mIAB-MT and IAB Type 1-O.

-	The conditions are defined in Table H.1.1.5.2-3 for CSI-RS based L1-RSRP measurement reporting in FR2 for mIAB-MT and IAB Type 2-O.

Table H.1.1.5.2-1: Conditions for CSI-RS based L1-RSRP measurements for mIAB-MT and IAB Type 1-H

| IAB-MT channel bandwidth (MHz) | CSI-RS sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | CSI-RS Ês/Iot (dB) | Minimum CSI-RS_RP (dBm) |
| 10, 15 | 30 | -3 | -96 - 10*Log10(NPRB *12) |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | -3 | -89.5- 10*Log10(NPRB *12) |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. |  |  |  |

Table H.1.1.5.2-2: Conditions for CSI-RS based L1-RSRP measurements for mIAB-MT and IAB Type 1-O

| IAB-MT channel bandwidth (MHz) | CSI-RS sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | CSI-RS Ês/Iot (dB) | Minimum CSI-RS_RP (dBm) |
| 10, 15 | 30 | -3 | -96 - 10*Log10(NPRB *12) - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | -3 | -89.5- 10*Log10(NPRB *12) - ΔOTAREFSENS |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. |  |  |  |

Table H.1.1.5.2-3: Conditions for CSI-RS based L1-RSRP measurements for mIAB-MT and IAB Type 2-O

| IAB-MT channel bandwidth (MHz) | CSI-RS sub-carrier spacing (kHz) | Side conditions |  |
| --- | --- | --- | --- |
|  |  | CSI-RS Ês/Iot (dB) | Minimum CSI-RS_RP (dBm) |
| 50 | 120 | -3 | EISREFSENS_50M - 10*Log10(NPRB *12) + ΔFR2_REFSENS -2 |
| 100, 200, 400 | 120 | -3 | EISREFSENS_50M - 10*Log10(NPRB *12) + ΔFR2_REFSENS +1 |
| NOTE 1: NPRB is the number of PRBs within the IAB-IMT channel bandwidth defined in section 5.3.2. NOTE 2: EISREFSENS_50M for mIAB-MT is defined in section 10.3.3.3. |  |  |  |

----------------------END OF CHANGE----------------------------