3GPP TSG-RAN WG4 Meeting # 109	R4-2320780

Chicago, USA, 13-17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TDCP requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2023-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TDCP is introduced in Rel-18 by RAN1 and RAN4 needs to define the requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TDCP requirements are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TDCP requirements will be missing for UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 9.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==== Start of changes ====

## 9.X	TDCP measurements for Reporting

### 9.X.1	Introduction

When configured by the network with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'tdcp', the UE shall be able to perform TDCP amplitude correlation measurements on the configured CSI-RS resources for TDCP.

The UE shall be able to measure all the resources configured within CSI Resource Setting (given by higher layer parameter resourcesForChannelMeasurement). The CSI Resource Setting may be periodic, with KTRS∈{1, 2, 3} CSI-RS Resource Sets configured with higher layer parameter trs-Info or aperiodic, with a single CSI-RS Resource Set configured with higher layer parameter trs-Info.

The UE shall report the measurement quantity (reportQuantity set to ‘tdcp’) and send aperiodic reports, according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP.

### 9.X.2	Requirements applicability

The requirements in clause 9.X apply, provided:

-	The CSI-RS resources configured for TDCP measurements are measurable.

A CSI-RS resource configured for TDCP report shall be considered measurable when for each relevant CSI-RS the following conditions are met:

-	TDCP related side conditions given in clauses 10.x.x.x,

-	CSI-RS Ês/Iot is more than [X]dB.

TDCP requirements are defined for periodic, and aperiodic measurement resources and aperiodic reporting resources.

### 9.X.3	Measurement Reporting Requirements

The UE shall send TDCP amplitude correlation reports only for report configurations configured for the active BWP.

The UE shall report the quantised TDCP value as a 4-bit value ranging from 0 to 15 with the absolute amplitude correlation is in the range [0.996094, 0.292893] with step size as described in clause 5.2.1.4.5 of TS 38.214.

#### 9.X.3.1	Aperiodic Reporting

Reported TDCP amplitude measurements contained in aperiodic triggered, but measured over periodic resources or aperiodic resources shall meet the accuracy requirements specified in clause 10.y.y.y.

The UE shall only send aperiodic TDCP measurement reports, if a DCI trigger has been received.

After the UE receives CSI request in DCI with (reportQuantity set to ‘tdcp’), the UE shall transmit the aperiodic TDCP reporting on PUSCH over the air interface at the time specified according to clause [x.y.z] in TS 38.214 [26].

### 9.X.4	TDCP measurement requirements

The UE shall be capable of performing TDCP measurements based on the configured CSI-RS resource for TDCP computation. The reported TDCP measurement may be an average of TDCP measured over last [Y] TDCP measurement resource ocssions. The value of Y is upto UE implementation as long as the accuracy requirement in clause 10.y.y.y is met.

===== End of changes =====