3GPP TSG-RAN WG4 Meeting #109	R4-2320811

# Chicago, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | XXXX | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38.133 Transmission and reception configurations for PRS/SRS BW aggregation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 03/11/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Transmission configuration for minimum transmission timing requirements for SRS bandwidth aggregation is missing in TS 38.133. - Transmission and reception configurations for RRM requirements for PRS bandwidth aggregation are missing in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Transmission configuration for minimum transmission timing requirements for SRS bandwidth aggregation is added. - Transmission and reception configurations for RRM requirements for PRS bandwidth aggregation are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing transmission and reception configurations for PRS/SRS bandwidth aggregation. Feature is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.X (new), 9.9.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

### 7.5.X	Minimum Requirements for SRS bandwidth aggregation

When UE supports [SRS bandwidth aggregation capability] and is configured with [SRS bandwidth aggregation configuration], relative transmission timing difference between slot timing of all pairs of configured intra-band contiguous carriers to be aggregated are based on common numerology across all intra-band contiguous carriers, whereby

-	SRS resources to be aggregated from different carriers may have equal or different number of SRS RBs.

-	SRS resources to be aggregated from different carriers are transmitted in the same slot and in the same symbols.

-	SRS resources to be aggregated from different carriers are transmitted by single RF chain and associated with a common Antenna Reference Point (ARP), as specified in TS 38.101-1 for FR1 and TS 38.101-2 for FR2.

[The minimum requirements are FFS.]

--- End of change 1 ---

--- Start of change 2 ---

#### 9.9.1.1	General Aspects of Gap-based Measurement

For gap-based RSTD, PRS-RSRP, UE Rx-Tx time difference, and PRS-RSRPP measurements, the requirements in clauses 9.9.2.5, 9.9.3.5, 9.9.4.5 and 9.9.6.5 apply provided:

-	the UE is configured or pre-configured with measurement gaps or configured with concurrent measurement gaps

-	all positioning frequency layers are measured or associated with only one per-UE measurement gap, or

-	for the UE supporting independentGapConfigPRS-r17, all positioning frequency layers in the same FR are measured or associated with only one per-FR measurement gap in the corresponding FR.

-	if the measurement gap is pre-configured, the gap must be activated throughout the measurement period, and

-	if concurrent measurement gaps are configured, one of the gap combinations specified in clause 9.1.8.2 is configured, and

-	if the UE does not support independentGapConfigPRS-r17, the configured or pre-configured gap used to perform the PRS measurements must be of per-UE type, and

-	No active BWP switching occurs during the measurement gaps for PRS measurement, and

All measurement requirements specified in clause 9.9.2.5, 9.9.3.5, 9.9.4.5 and 9.9.6.5 shall apply without DRX as well as for any DRX configuration specified in TS 38.331 [2].

UE is only required to measure PRS resources that are fully or partially overlapped with measurement gaps, and the requirements in clause 9.9.2.5, 9.9.3.5, 9.9.4.5 and 9.9.6.5 are applicable to PRS resources that are fully or partially overlapped with measurement gaps.

A PRS resource is considered to be fully (partially) overlapped with measurement gaps if all (some) of its instances are overlapped with a measurement gap occasion. A PRS resource instance is considered to be overlapped with measurement gap occasion if the minimum number of unmuted repetitions of the instance taking into account nr-DL- PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD is fully covered by the MGL excluding RF switching time, where the minimum number is given in the accuracy requirements in clause 10.1.23, 10.1.24, 10.1.25 and 10.1.38 for RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP, respectively.

When UE is configured with measurement for more than one positioning requests, the measurement period for each request may be longer than measurement period when UE is configured with measurement for single positioning request.

If a positioning measurement gap is configured via PosGapConfig and activated by MAC CE, the measurement requirements in clause 9.9.2.5, 9.9.3.5, 9.9.4.5 and 9.9.6.5 apply provided that no other MGs are configured, and only one measurement gap configured via PosGapConfig is activated.

When UE supports [PRS bandwidth aggregation capability] and is configured with [PRS bandwidth aggregation configuration], requirements are based on common numerology across all configured intra-band contiguous PFLs to be aggregated, whereby

-	PRS resources to be aggregated from different PFLs may have equal or different number of PRS RBs.

-	PRS resources to be aggregated from different PFLs are transmitted in the same slot and in the same symbols.

-	PRS resources to be aggregated from different PFLs are transmitted by the same TRP and associated with a common Antenna Reference Point (ARP), as specified in TS 38.104.

- 	PRS resources to be aggregated from different PFLs are received with single RF chain and the same Antenna Reference Point (ARP), as specified in TS 38.101-1 for FR1 and TS 38.101-2 for FR2.

--- End of change 2 ---