3GPP TSG-RAN WG4 Meeting #109	R4-2321009

Chicago, US, Nov 13 – Nov 17, 2023

Agenda Item:	8.25.2.3

Source:	MediaTek Inc.

Title:	Discussion on NW A RRM requirements for MUSIM

Document for:	Discussion

# Introduction

In NR Rel-17 specification, RAN4 has introduced gap patterns particularly for MUSIM purpose. However, their corresponding RRM requirements were not specified. In Rel-18 MUSIM WI, RAN4 has discussed the RRM requirements for MUSIM and the open issues are captured in the WF [1]. In this paper, NW A requirements for MUSIM are further discussed.

# Discussion

The configured MUSIM gaps may collide with L3/L1 measurements in NW A while the UE is in RRC_CONNECTED state in NW A. This collision will impact the RRM requirements for NW A. In the previous meeting, the requirements of L3 and L1 measurements were updated to address the impact of MUSIM gaps on the measurement delay requirements including the scaling factors Kp , Kgap and Kgap_EUTRA.

However, it was also noticed that these requirements cannot be met for some cases. For example, in intra-frequency measurements without MG, when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions, requirements do not apply.

For the scenario when Navailable = 0 due to fully overlapping between SMTC occasions and the union of MUSIM gap and measurement gap occasions it is still FFS. In our view, this is a corner case and NW can avoid having such configuration. Therefore, it is preferred not to define special requirement for such scenario either and to simplify the specs by having a general rule for both cases (i.e., when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions or the union with MG occasions)

Proposal 1: Requirements do not apply for intra-frequency measurement without MG if Navailable = 0 due to fully overlapping between SMTC occasions and the union of MUSIM gap and measurement gap occasions.

In addition, MUSIM gaps can also impact L3/L1 measurements in NTN (SAN) and unlicenced bands. However, the impact of MUSIM gaps on NTN measurements was not discussed in RAN4, and the discussion of collision between MUSIM gaps and NTN resources (e.g., SMTC) can be different from the previous agreements made for the TN scenario. Therefore, given the limited time to finish the WI, we suggest to descope MUSIM gaps impact on NTN requirements.

Proposal 2: Descope MUSIM gaps impact on NTN requirements in R18.

# Summary

In this contribution, the remaining RRM requirements for NW A are discussed. The following proposals were approached:

Proposal 1: Requirements do not apply for intra-frequency measurement without MG if Navailable = 0 due to fully overlapping between SMTC occasions and the union of MUSIM gap and measurement gap occasions.

Proposal 2: Descope MUSIM gaps impact on NTN requirements in R18.

# References

- R4-2317425, WF on R18 MUSIM, vivo.