3GPP TSG-RAN4 Meeting #109	R4-2321124

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | 1 | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for ATG UE Demodulation Requirements – Applicability Rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Perf |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce ATG UE Demodulaiton Requirements – Applicability Rules |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce ATG UE Demodulaiton Requirements – Applicability Rules |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.1 5.1.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### 5.1.1.1	General

The minimum performance requirements are applicable to all FR1 operating bands defined in TS 38.101-1[6].

The minimum performance requirements in Clause 5 are mandatory for UE supporting NR operation, except test cases listed in Clauses 5.1.1.3, 5.1.1.4, 5.1.1.5, 5.1.1.6, 5.1.1.7, 5.1.1.8, 5.1.1.12.

If same test is listed for different UE features/capabilities in Clauses 5.1.1.3 and 5.1.1.4, then this test shall apply for UEs which support all corresponding UE features/capabilities.

End of Change 1

Start of Change 2

#### 5.1.1.12	Applicability of requirements for ATG

The performance requirements in Table 5.1.1.12-1 shall apply for ATG UEs [which support optional feature TBD].

Other performance requirements mandatory for UE supporting NR operation defined in Section 5 but not included in table 5.1.1.12-1 should not be considered applicable to ATG UEs.

Table 5.1.1.12-1: Requirements applicability for ATG UEs

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| ATG UE with 2RX | FR1 FDD | PDSCH | All tests in Clause 5.2.2.1.22 |  |
|  |  | PDCCH | Test number 1, 3 and 5 in Table 5.3.2.1.1 Test number 3 in Table 5.3.2.1.2 |  |
|  |  | PBCH | Clause 5.4.2.1 |  |
|  |  | SDR | Clause 5.5.1 |  |
|  | FR1 TDD | PDSCH | Tests 1-1, 1-2, 1-3 in Clause 5.2.2.2.23 | Only if ATG UE does not supports optional features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ |
|  |  |  | Tests 1-4, 1-5, 1-6 in Clause 5.2.2.2.23 | Only if ATG UE supports the optional features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ |
|  |  | PDCCH | All test cases in Table 5.3.2.2.1 All test cases in Table 5.3.2.2.2 |  |
|  |  | PBCH | Clause 5.4.2.2 |  |
|  |  | SDR | Clause 5.5.1 |  |
| ATG UE with 4RX | FR1 FDD | PDSCH | All tests in Clause 5.2.3.1.21 |  |
|  |  | PDCCH | Test number 1, 3 and 5 in Table 5.3.3.1.1 Test number 3 in Table 5.3.3.1.2 |  |
|  |  | PBCH | Clause 5.4.3.1 |  |
|  |  | SDR | Clause 5.5.1 |  |
|  | FR1 TDD | PDSCH | Tests 1-1, 1-2, 1-3 in Clause 5.2.3.2.22 | Only if ATG UE does not supports optional features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ |
|  |  |  | Tests 1-4, 1-5, 1-6 in Clause 5.2.3.2.22 | Only if ATG UE supports the optional features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ |
|  |  | PDCCH | All test cases in Table 5.3.3.2.1 All test cases in Table 5.3.3.2.2 |  |
|  |  | PBCH | Clause 5.4.3.2 |  |
|  |  | SDR | Clause 5.5.1 |  |

End of Change 2