3GPP TSG-RAN WG4 Meeting #110                                                                                        R4-2400041

Athens, GR, February 26 – March 01, 2024

Source: 	CATT

Title: 	Further discussion on SAN RF requirements for above 10GHz bands

Agenda item:	8.18.3

Document for:	Approval

- Introduction

This contribution provides our views on ICS requirement for SAN RF requirements for above 10GHz bands.

- Discussion

OTA in-channel selectivity requirement

Generally, the IoT level is achieved in SAN system due to the co-channel interference from other cells or beams. According to the simulation results for Ka-band NTN SAN dynamic range[1], the IoT at 95% CDF for FRF=2 agreed as co-existence parameter is as following table.

Table 2-1 IoT at 95% CDF for FRF=2

|  | XPR=20dB | XPR=30dB |
| --- | --- | --- |
| Cases | IoT [dB] @ 95% CDF | IoT [dB] @ 95% CDF |
| GEO | -14.65 | -24.65 |
| LEO-600 | 5.33 | -4.668 |
| LEO-1200 | -0.6906 | -10.7 |

For FRF=2, as shown in above Table 2-1, observed IoT level for SAN including GEO, LEO-600 and LEO-1200 is around [-24.65-5.33] dB, considering the legacy C/I 9dB assumed for interfering signal, then maximum ICS is about 14dBc, which is same as maximum ICS 14dBc for FR2-1 TN BS, where legacy C/I 9dB is from clause 10.9.3 of TR 38.817-2.

So it is reasonable that the OTA in-channel selectivity requirement for BS type 2-O for FR2-1 can be reused for Ka-band SAN type 2-O.

Proposal 1: Reuse OTA in-channel selectivity requirement for BS type 2-O for FR2-1 for Ka-band SAN type 2-O as below:

Table 10.9.3-1: OTA in-channel selectivity requirement for SAN type 2-O

| Frequency Range | BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-NTN | 50 | 60 | G-FR2-A1-4 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 32 RB |
|  | 100,200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 64 RB |
|  | 50 | 120 | G-FR2-A1-5 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 16 RB |
|  | 100,200,400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_50M is defined in clause 10.3.3. |  |  |  |  |  |  |

- Conclusion

This contribution provides our general consideration on SAN RF requirement for above 10GHz bands. The following observations and proposals are provided as follows:

Proposal 1: Reuse OTA in-channel selectivity requirement for BS type 2-O for FR2-1 for Ka-band SAN type 2-O as below:

Table 10.9.3-1: OTA in-channel selectivity requirement for SAN type 2-O

| Frequency Range | BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-NTN | 50 | 60 | G-FR2-A1-4 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 32 RB |
|  | 100,200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 64 RB |
|  | 50 | 120 | G-FR2-A1-5 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 16 RB |
|  | 100,200,400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_50M is defined in clause 10.3.3. |  |  |  |  |  |  |

- References

- R4-2318302，Simulation results for Ka-band NTN SAN dynamic range, CATT, RAN4#109