3GPP TSG-RAN WG4 Meeting #110                                                                                    R4-2400076

Athens, GR, Feb. 26 – Mar. 01, 2024

Source: 	CATT

Title: 	Discussion on 15 KHz SCS issue for NCR-MT

Agenda item:	8.20.1.2

Document for:	Approval

- Introduction

In RAN4 #109meeting, the Big CR [1] on TS 38.106 [2] was approved. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share our views on 15 KHz SCS related issue for NCR-MT.

- Discussion

Since the Big CR on TS 38.106 has been approved, the transmission bandwidth configuration of NCR-MT will reuse the UE approach as follows:

### 5.4.2 Transmission bandwidth configuration For NCR-MT, the transmission bandwidth configuration is the same as specified for UE in TS 38.101-1 [13] for FR1 in subclause 5.3.2 and in TS 38.101-2 [14] for FR2-1 in subclause 5.3.2.

More specifically, the transmission bandwidth configuration of UE (FR1 as an example) is shown as below:

### 5.3.2 Maximum transmission bandwidth configuration The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1. Table 5.3.2-1: Maximum transmission bandwidth configuration NRB SCS (kHz); 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 25; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; 11; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135.

It is obviously that for NCR-MT, the 15 KHz SCS will be supported. However, we noticed that for REFSENS, ACS and in-band blocking requirements of the WA NCR-MT, the requirements of WA IAB-MT are reused accordingly. As an example, the REFSENS requirement of NCR-MT was captured below:

### 6.16.1 NCR-MT reference sensitivity level #### 6.16.1.1 General The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector for NCR-MT type 1-C or TAB connector for NCR-MT type 1-H at which a throughput requirement shall be met for a specified reference measurement channel. #### 6.16.1.2 Minimum requirements for NCR-MT type 1-C and 1-H The Wide Area NCR-MT reference sensitivity level is specified the same as the Wide Area IAB-MT reference sensitivity level requirement for IAB-MT type 1-H in TS 38.174 [22], clause 7.2.2, where references to nominal channel bandwidth apply to NCR-MT passband. The Local Area NCR-MT reference sensitivity level is specified the same as specified in TS 38.101-1 [13] clause 7.3. Referenced requirements applying to NB IoT are not applicable to the NCR-MT.

The scheme of reusing IAB-MT requirement raised the issue that the definition of 15 KHz SCS is missing for IAB-MT REFSENS/ACS/IBB requirements. However, since LA NCR-MT reuses UE requirements, the related definition of 15 KHz SCS is included in UE spec. Therefore, the REFSENS/ACS/IBB requirement between WA NCR-MT and LA NCR-MT will be not compatible.

Observation 1: For WA NCR-MT, the 15 KHz SCS related components under REFSENS/ACS/IBB requirements is missing. Therefore, the REFSENS/ACS/IBB requirements between WA NCR-MT and LA NCR-MT are not compatible.

In RAN4 #108 meeting, a WF [3] on FRC parameters has been agreed:

Issue 3-2-1 REFSENS requirement - Agreement: - For LA NCR-MT: reusing legacy UE assumption on NF - Use the revised Table 2-1 for FRC parameters below for FR1 NCR-MT Table 2-1: FRC parameters for FR1 reference sensitivity level for NCR-MT. Reference channel; G-FR1-A1-21; G-FR1-A1-22; G-FR1-A1-23; G-FR1-A1-24; G-FR1-A1-25; G-FR1-A1-26. Subcarrier spacing (kHz); 15; 30; 60; 15; 30; 60. Allocated resource blocks; 25; 11; 11; 106; 51; 24. CP-OFDM Symbols per slot (Note 1); 9; 9; 9; 9; 9; 9. Modulation; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK. Code rate (Note 2); 1/3; 1/3; 1/3; 1/3; 1/3; 1/3. NOTE 1: DL-DMRS-config-type = 1 with DL-DMRS-max-len = 1, DL-DMRS-add-pos = pos2 with = 2, = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity.

It is obviously that the FRC parameters for 15 KHz SCS for NCR-MT was defined during the previous discussion, but not captured correctly in the TS 38.106. Therefore, we believed that the 15 KHz SCS related contents could be supplemented for WA NCR-MT.

Proposal 1: The 15 KHz SCS related contents for REFSENS/ACS/IBB requirement could be supplemented for WA NCR-MT.

However, since the REFENSE/ACS/IBB requirements of WA NCR-MT were reused from IAB-MT spec, it might be difficult to supplement the relevant contents. As an alternative, RAN4 could seek the views of operators on the need to retain the 15 KHz implementation for WA NCR-MT. By excluding the 15 KHz SCS from the transmission bandwidth configuration of NCR-MT, the REFSENS/ACS/IBB requirements between WA NCR-MT and LA NCR-MT are able to be compatible.

Proposal 2: As an alternative, RAN4 could seek the views of operators on the need to retain the 15 KHz implementation for WA NCR-MT. By excluding the 15 KHz SCS from the transmission bandwidth configuration of NCR-MT, the REFSENS/ACS/IBB requirements between WA NCR-MT and LA NCR-MT are able to be compatible.

- Conclusion

This contribution provides our consideration on 15 KHz SCS issue on NCR-MT. The following proposals are concluded as follows:

Observation 1: For WA NCR-MT, the 15 KHz SCS related components under REFSENS/ACS/IBB requirements is missing. Therefore, the REFSENS/ACS/IBB requirements between WA NCR-MT and LA NCR-MT are not compatible.

Proposal 1: The 15 KHz SCS related contents for REFSENS/ACS/IBB requirement could be supplemented for WA NCR-MT.

Proposal 2: As an alternative, RAN4 could seek the views of operators on the need to retain the 15 KHz implementation for WA NCR-MT. By excluding the 15 KHz SCS from the transmission bandwidth configuration of NCR-MT, the REFSENS/ACS/IBB requirements between WA NCR-MT and LA NCR-MT are able to be compatible.

- References

- R4-2321202, Big CR to TS 38.106 Introduction of NCR, ZTE, RAN4#109

- TS 38.106, NR Repeater Radio Transmission and Reception, 3GPP technical specification, V18.3.0

- R4-2313903, WF on NCR RF requirements, ZTE, RAN4#108