3GPP TSG-RAN WG4 Meeting #110		R4-2400178

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	10

Source: 	Apple

Title: 	On Release 18 feature list

Document for:	Discussion

- Introduction

In this document, we continue to share our views on Rel-18 NR UE features.

- NR_ENDC_RF_FR1_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NR_channel_raster_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NR_RF_FR2_req_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_RF_FR2_req_Ph3 | 29-1 | Beam correspondence in initial access and RRC_INACTIVE for PC3 UEs | R18 and onward UE shall support beam correspondence in initial access and RRC_INACTIVE and satisfy the corresponding spherical coverage requirement for initial access and RRC_INACTIVE as specified in 38.101-2 | 8-2 and 2-20 | No |  | UE performance in initial access and RRC_INACTIVE in FR2 cannot be guaranteed | Per UE | N/A | FR2 only |  |  | Mandatory without capability signaling |

- NR_FR2_multiRX_DL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_FR2_multiRX_DL | 30-1 | Supports scheduling restriction relaxation and measurement restriction relaxation | - Supports simultaneous reception of CSI-RS for layer 1 measurement and PDSCH with different QCL Type-D on overlapping OFDM symbols. - Supports Simultaneous layer 1 measurement of CSI-RS overlapping with another CSI-RS with different QCL Type-D on overlapping OFDM symbol(s). | 16-2c, 23-5-1, [at least one of 16-2a, 16-2b-1, 16-2b-2 and 16-2b-3] | Yes | N/A |  | Per FSPC | TDD only | FR2-1 only |  | [Note: It can be supported for all power classes excepted PC6. ] | Optional with capability signalling |
| 30. NR_FR2_multiRX_DL | [30-2] | Fast beam sweeping for layer 1 measurement | Supports beam sweeping factor reduction for SSB-based layer 1 measurement when the UE is in multi-RX operation. | 16-2c, 23-5-1 | [No] | N/A |  | Per band | N/A | FR2-1 only |  | Candidate values for Component 2: {2,4,6} for FR2-1 [Note: It can be supported for all power classes excepted PC6.] | [Optional with capability signalling] |

- NR_RRM_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | [32-1] | Concurrent gap with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Clause [9.1.12] of TS 38.133. | 19-2, and 19-3-1 or 19-3-2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | [32-2] | Concurrent gap with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | Network should not configure concurrent gap with NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | [32-3] | 2 NCSG configuration in a FR | Support configurations of 2 NCSG in the same FR | 32-4 | Yes | No | Network should not configure 2 NCSG in the same FR | Per UE | No | [Yes] | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | [32-4] | [Inter-RAT EUTRAN measurements without gap and outside active DL BWP] | 1. Support of requirements of inter-RAT EUTRAN measurements when the signal to measure is not completely within active DL BWP without gap with or without interruption. UE supporting [32-7] shall also support [32-9] |  | Yes | NA | Inter-RAT EUTRAN measurements when the signal to measure is not completely within active DL BWP has to be done within measurement gap | [Per UE] | No | No | N.A | Similar with NCSG/NFG, reporting of feasibility of case b-1 is based on network request. | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-5 | Inter-RAT EUTRAN measurement without gap [and within active DL BWP] | Support of inter-RAT EUTRAN measurements without gap when the signal to measure is contained within UE’s active DL BWP UE supporting [32-8] shall also support [32-9] |  | Yes | No | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | [32-9] | Effective measurement window for inter-RAT EUTRAN measurements | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity. | [32-7 or 32-8] | Yes | No | Undefined UE measurement behavior and when to allow scheduling restriction | Per UE | No | No | N.A | - A bitmap for 6 effective measurement window (EMW) patterns defined in TS 38.133. TBD which patterns(s) are mandatory supported | Optional with capability signalling |
| 32. NR_MG_enh2 | [32-10] | SimultaneousRxDataCRS-DiffNumerology | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with signal to measure contained within UE’s active DL BWP and PDCCH or PDSCH reception from the serving cell with a different numerology | 32-8, or 32-7 | Yes | No | scheduling restriction is applicable | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |

- NonCol_intraB_ENDC_NR_CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NR_HST_FR2_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NR_ATG

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- NR_demod_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 36. NR_demod_enh3 | 36-1 | MU-MIMO Interference Mitigation advanced receiver | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas. Note: This is the UE capability when modulation order is signalled via DCI | 3-4 | Yes | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO | Per FSPC | N/A | FR1 only |  | This is the UE capability when modulation order is signalled via DCI | Optional with capability signaling |
| # 36. NR_demod_enh3 | 36-2a | MU-MIMO Interference Mitigation advanced receiver with modulation order not signalled | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and 4RX | 36-1 | Yes | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO when modulation order is not signalled | Per FSPC | N/A | FR1 only |  |  | Optional with capability signaling |
| # 36. NR_demod_enh3 | 36-2b | MU-MIMO Interference Mitigation advanced receiver with modulation order not signalled | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX | 36-1 | Yes | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO when modulation order is not signalled | Per FSPC | N/A | FR1 only |  |  | Optional with capability signaling |
| # 36. NR_demod_enh3 | 36-3 | DMRS Configuration for MU-MIMO advanced receiver | DMRS configuration supported with R-ML for MU-MIMO (1) Type 1 with length 1 (2) Type 1 with length 2 (3) Type 2 with length 1 (4) Type 2 with length 2 | 36-1 | Yes |  | UE is not capable of advanced receiver to suppress inter-user inference in MU-MIMO with the DMRS configuration | Per FSPC | N/A | FR1 only |  |  | Optional with capability signaling |

- NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. NR_pos_enh2 | 37-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-1 | UL Tx switching across 3 bands for single-TAG | - UE to indicate support of dynamic UL Tx switching across 3 bands for inter-band UL CA, SUL or inter-band EN-DC - UE to declare the switching period for UL Tx switching across 3 bands for in inter-band EN-DC, inter-band UL CA or SUL band combinations. Switching period value to be from the set (35uSec, 140uSec, 210uSec) |  | yes | no | UE does not support Tx switching across 3 bands for inter-band EN-DC, inter-band UL CA and SUL band combinations, single-TAG | per band pair per BC | No | FR1 only | N/A | Agreed in RAN#98-e RP-223557 | Optional with capability signalling |
|  | 38-2 | UL Tx switching across 4 bands for single-TAG | - UE to indicate support of dynamic UL Tx switching across 4 bands for inter-band UL CA, SUL or inter-band EN-DC - UE to declare the switching period for UL Tx switching across 4 bands for in inter-band EN-DC, inter-band UL CA or SUL band combinations. Switching period value to be from the set (35uSec, 140uSec, 210uSec) |  | yes | no | UE does not support Tx switching across 4 bands for inter-band EN-DC, inter-band UL CA and SUL band combinations, single-TAG | per band pair per BC | No | FR1 only | N/A | Agreed in RAN#98-e RP-223557 | Optional with capability signalling |
|  | 38-3 | UL Tx switching across 2 bands for dual-TAG | - UE to indicate support of dynamic UL Tx switching across 2 bands for inter-band UL CA, SUL or inter-band EN-DC, dual-TAG mode - UE to declare the switching period for UL Tx switching across 2 bands for in inter-band EN-DC, inter-band UL CA or SUL band combinations, dual-TAG mode. Switching period value to be from the set (35uSec, 140uSec, 210uSec) |  | yes | no | UE does not support Tx switching across 2 bands for inter-band EN-DC, inter-band UL CA and SUL band combinations, dual-TAG | per band pair per BC | No | FR1 only | N/A | Agreed in RAN#101 RP-232418 | Optional with capability signalling |
|  | 38-4 | UL Tx switching across 3 bands for dual-TAG | - UE to indicate support of dynamic UL Tx switching across 3 bands for inter-band UL CA, SUL or inter-band EN-DC, dual-TAG mode - UE to declare the switching period for UL Tx switching across 3 bands for in inter-band EN-DC, inter-band UL CA or SUL band combinations, dual-TAG mode. Switching period value to be from the set (35uSec, 140uSec, 210uSec) | 38-1 and 38-2 | yes | no | UE does not support Tx switching across 3 bands for inter-band EN-DC, inter-band UL CA and SUL band combinations, dual-TAG | per band pair per BC | No | FR1 only | N/A | NOTE: Signalling structure is up to RAN2. RAN4 will specify for UL CA and EN-DC for which band combinations DL interruptions are Agreed in RAN#101 RP-232418 | Optional with capability signalling |
|  | 38-5 | UL Tx switching across 4 bands for dual-TAG | - UE to indicate support of dynamic UL Tx switching across 4 bands for inter-band UL CA, SUL or inter-band EN-DC, dual-TAG mode - UE to declare the switching period for UL Tx switching across 4 bands for in inter-band EN-DC, inter-band UL CA or SUL band combinations, dual-TAG mode. Switching period value to be from the set (35uSec, 140uSec, 210uSec) | 38-1 and 38-2 | yes | no | UE does not support Tx switching across 4 bands for inter-band EN-DC, inter-band UL CA and SUL band combinations, dual-TAG | per band pair per BC | No | FR1 only | N/A | NOTE: Signalling structure is up to RAN2. RAN4 will specify for UL CA and EN-DC for which band combinations DL interruptions are Agreed in RAN#101 RP-232418 | Optional with capability signalling |
|  | 38-6 | DL interruption for Tx switching across 3 bands | Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern - TDD+TDD EN-DC with the same UL-DL pattern | 38-1, 38-2, 38-3 | yes | no | UE not reporting this capability means DL interruption is not required | per band pair per BC | No | FR1 only | N/A |  | Optional with capability signalling |
|  | 38-7 | DL interruption for Tx switching across 4 bands | Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern - TDD+TDD EN-DC with the same UL-DL pattern | 38-1, 38-2, 38-3 | yes | no | UE not reporting this capability means DL interruption is not required | per band pair per BC | No | FR1 only | N/A |  | Optional with capability signalling |
|  | 38-8 | Optional UE TX switching capability | - For the band with the number of Tx chain unchanged due to switching, in addition to the baseline UE assumption agreed in RAN4 #104e, introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching. The swithing period is the switching period between band A and band B | 38-1 | yes | no | UE not reporting this capability means UE does not support optional Tx switching capability | per band pair per BC | No | FR1 only | N/A | RAN2 to define how the UE could signal to the network its support of this this optional feature. Agreed in RAN#98-e RP-223557 | Optional with capability signalling |

Additional UE optional capabilities have been proposed by proposed by few companies:

- UL Tx switching across 4 bands: In RAN4 Meeting#107, one company proposed a solution for solving the 4-band UL Tx switching ambiguity issue and improving the corresponding switching period: This proposal relies on introducing two new UE capabilities: The first one is the BandOrdering1T1Tto1T1T through which the UE indicates how the 4 bands (A, B, C, D) are mapped for (1,1,0,0) to (0,0,1,1) switching. The second capability is the preferredBandPairsTxChain which  allocates the UE’s preferred Tx switching band pairs to specific Tx chain. For example, for UL Tx 4-band switching across (A,C,B,D), if the UE declares the two capabilities described above with (A,C) the preferred band pair for TX chain#1 and (B,D) the preferred band band pair of TX chain#2, the following TX switching scenario will be realized for simultaneous dualUL switching: (A—>C) and (B—>D). If UE reports max(Tswitch_A-C, Tswitch_B-D)=SW1, and max(Tswitch_A-D, Tswitch_B-C)=SW2 with SW2>SW1, there will be an improvement of SW2-SW1 on the switching time.

- UL Tx switching acoss 3 and 4 bands with at least one unaffected band : In RAN4 Meeting #108, another company proposed to extend the concept to switching cases where one band is not involved in the switching process (the unaffected band cases) with the goal to reduced the switching period. Supporting that capacity means that the switching period could be  min{max(Tswitch_A-C, Tswitch_B-C), Tswitch_A-B)}for 3-band switching and min{max(Tswitch_A-C, Tswitch_B-C), Tswitch_A-B)} for 4-band switching. There was an issue with this proposal that was not addressed during the offline discussion: If UE declares Tswitch_A-C and Tswitch_B-D = 140uSec, and Tswitch_A-D and Tswitch_B-C = 35uSec, the resulting 4-band switching period would be 35uSec based on the formula above. We would prefer the UE to just declare Tswitch_A-C and Tswitch_C-D as the switching period for the 3-band and 4-band switching respectively.

- NR_Mob_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | [39-1] | SSB based L1-RSRP measurements for multiple cells with RTD > CP | Capability of SSB based L1-RSRP measurements for more than 1 Cells with RTD > CP in one frequency layer | 45-1, or 45-1a from RAN1 Rel-18 feature list | Yes | No | [NW does not know which requirements UE will follow] | [Per UE] | No | Yes | N/A | Note: the max number of cells is up to 39-3-1 or 39-3a-1 | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-2] | SSB based inter-frequency L1-RSRP measurements without gap | Capability of SSB based inter-frequency L1-RSRP measurements without gap for LTM | 45-1a and 9-4 | Yes | No | UE does not support inter-frequency L1-RSRP measurements without gap | [Per band/BC] | No | No | N/A | Note: UE supports inter-frequency with Type 1 measurement gap by default if UE reports supporting 45-1a but not 39-3. | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-3-1] | Number of neighbour cells to be measured per intra-frequency layer | Capability of number of neighbour cells to be measured for L1-RSRP per intra-frequency layer | 45-1 from RAN1 Rel-18 feature list | Yes | No | NW does not know the max number of cells UE can measure | [per UE / Per band] | No | [No] | N/A | Candidate values: 2, 3, [… TBD] | Mandatory with capability signaling if UE supports 45-1 |
| 39. NR_Mob_enh2 | [39-3-2] | [Number of SSB resources for L1-RSRP measurement within a slot] | The max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement | 45-1 from RAN1 Rel-18 feature list and/or 39-3 | Yes | No | NW does not know the max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement | Per band | No | Yes | N/A | Note: component 4 is also counted in FG 2-24 | Mandatory with capability signaling if UE supports 45-1 or 39-3 |
| 39. NR_Mob_enh2 | [39-3-3] | [Number of SSB resources for L1-RSRP measurement per intra-frequency layer] | The max number of SSB resources for L1-RSRP measurement that UE can measure per intra-frequency layer | 45-1 from RAN1 Rel-18 feature list | Yes | No | NW does not know the max number of SSB resources for L1-RSRP measurement that UE can measure per frequency layer | Per UE | No | Yes | No |  | Mandatory with capability signaling if UE supports 45-1 |
| 39. NR_Mob_enh2 | [39-3-4] | [Number of frequency layers for L1-RSRP measurement] | - The max number of frequency layers configured to measure for intra- and inter-frequency without gap L1-RSRP measurement - The max number of frequency layers configured to measure for inter-frequency L1-RSRP measurement with measurement gap | 45-1 and/or 39-2 or 45-a from RAN1 Rel-18 feature list | Yes | No | NW does not know the max number of frequency layers UE can measure | Per UE | No | Yes | No |  | Mandatory with capability signaling if UE supports 45-1, 45-1a, 39-2 |
| 39. NR_Mob_enh2 | [39-3a-1] | Number of neighbour cells to be measured for L1-RSRP inter-frequency layer | Capability of number of neighbour cells to be measured for L1-RSRP inter-frequency per frequency layer | 45-1a from RAN1 Rel-18 feature list | Yes | No | NW does not know the max number of cells UE can measure | [per UE / Per band] | No | [No] | N/A | Candidate values: 2, 3, [… TBD] | Mandatory with capability signaling if UE supports 45-1a |
| 39. NR_Mob_enh2 | [39-3a-2] | [Number of SSB resources for L1-RSRP measurement per inter-frequency layer] | The max number of SSB resources for L1-RSRP measurement that UE can measure per inter-frequency layer | 45-1, or 45-1a from RAN1 Rel-18 feature list | Yes | No | NW does not know the max number of SSB resources for L1-RSRP measurement that UE can measure per frequency layer | Per UE | No | Yes | No |  | Mandatory with capability signaling if UE supports 45-1a |
| 39. NR_Mob_enh2 | [39-4] | Interruption on DL slot(s) due to PDCCH- ordered RACH transmission | Capability on whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission | 45-5 | Yes | No | UE does not cause interruptions on UL&DL slots on serving cells due to PDCCH-ordered RACH transmission | [Per band pair] (between the target band for RACH transmission and band under UE’s current band combo) per band combination | No | No | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-4a] | Interruption due to RF retuning for PDCCH- ordered RACH | Indicates the interruption length (Y ms) due to RF re-tuning for PDCCH ordered RACH of which the resources are not fully contained in any of UE’s configured UL BWP(s) of active serving cells | 45-5 | Yes | No | Network does not know whether UE supports the case that RACH bandwidth is outside of any configured BWP and the corresponding length of the interruption | [Per band pair] (between the target band for RACH transmission and band under UE’s current band combo) | No | No | N/A | Candidate values for interruption length Y = 0.25, 0.5, 1 and 2 | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-5] | Early ASN.1 decoding and validity check before cell switch | Capability of early ASN.1 decoding and validity check before cell switch to reduce or skip TLTM_RRC-processing delay (refer to TS 38.133)]. | [Component 6 of 45-3 or 45-4] | Yes | No | TLTM_RRC-processing delay will not be skipped, i.e., 10ms | [Per UE] | No | No | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-6] | [Shorter UE processing time during cell switch] | Capability of reduced TLTM_processing delay (refer to TS 38.133)]. - Support of reduced TLTM_processing for cell switch from FR1 to FR1. - Support of reduced TLTM_processing for cell switch from FR2 to FR2. - Support of reduced TLTM_processing for cell switch from FR1/FR2 to FR2/FR1. | [Component 6 of 45-3 or 45-4] | Yes | No | TLTM_processing delay will not be reduced, i.e., 20ms for intra-FR cell switch and 40ms for inter-FR cell switch | [Per UE] | No | No | N/A | Candidate values: TBD | Optional with capability signaling |

- NR_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_NTN_enh | 40-1 | DMRS bundling for NTN coverage enhancement | The range of [maxDurationDMRS-Bundling-r17] for NTN bands for which RAN4 has introduced requirements is restricted as follows: - Up to [4] slots if ntn-ScenarioSupport-r17 is present and indicated as NGSO or only the IE field nonTerrestrialNetwork-r17 is present - Up to [16] slots if ntn-ScenarioSupport-r17 is present and indicated as GSO | [maxDurationDMRS-Bundling-r17], [ntn-ScenarioSupport-r17] | Yes | N/A | UE cannot support this feature | Per band |  |  | N/A |  | Optional with capability signalling |

RAN1 has assumed “pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side” in order for the UE to meet the maximum allowable phase difference requirements for DMRS bundling.  Because the side conditions of the phase continuity requirement for DMRS bundling do not allow the UE to perform pre-compensation during the configured DMRS bundling window, the UE has to perform this operation at the beginning of the bundle.  Thus, the range of the maximum duration of DMRS bundling needs to be restricted for NTN bands as follows:

- Up to [4] slots if ntn-ScenarioSupport-r17 is present and indicated as NGSO or only the IE field nonTerrestrialNetwork-r17 is present

- Up to [16] slots if ntn-ScenarioSupport-r17 is present and indicated as GSO

It is proposed to reflect these restrictions in the Rel-18 UE feature list, with the understanding that RAN2 can accommodate the restriction in the capability value within the TS38.306 specification.  Alternatively, if this approach is deemed too difficult, RAN4 can consider introducing new NTN-specific capabilities for DMRS bundling with these restrictions applied.

- NR_cov_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Netw_Energy_NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. # Netw_Energy_NR | 42-1 | FR1 Inter-band SSB-less SCell operation | Support of SSB-less SCell operation for inter-band CA for FR1 and co-located cells |  | Yes | N/A | UE does not support SSB-less SCell operation for inter-band CA for FR1 and co-located cells, and network cannot configure FR1 inter-band SSB-less SCell to the UE for CA. | Per FS | No | FR1 only | N/A | UE is required to meet the RRM requirement of FR1 inter-band SSB-less SCell operations in TS38.133. | Optional with capability signaling |

- NR_DualTxRx_MUSIM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. # NR_DualTxRx_MUSIM | 43-1 | Collision handling and signalling solution for handling collisions between MUSIM gaps and between measurement gaps and MUSIM gaps | UE supports indication of MUSIM gap priority; collision handling between MUSIM gaps and between measurement gaps and MUSIM gaps | musim-GapPreference-r17 | YES | N/A | UE is not capable to meet Rel-18 MUSIM requirement | Per UE | No | No | N/A |  | Optional with UE capability signalling |

- 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44. XX | 44-1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 44. XX |  |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. # NR_SL_enh2 | 45-1 |  |  |  |  |  |  |  |  |  |  |  |  |