3GPP TSG-RAN4 Meeting#110	 R4-2400206

# Athens, Greece, 26 Feb- 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1992 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | ❌ | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 Cat F CR for 38.101-1 Add the missing configurations for ULCA and TxD scenario with 3Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, TELUS, Bell Mobility, Verizon, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core |  |  |  |  |  | Date: |  |  | 2024-01-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CA_n25-n77 and CA_n5A-n77A were selected as PC1.5 example band combinations during the discussion for 3Tx WI, as below. These two combos have been include in Table 6.2H.3.1-1 of TS38.101-1 v0.4.0 which is for ULCA+UL MIMO scenario, while missing in clause 6.2L.3.1 for ULCA+TxD scenario (And the table header is also missing). (Please Note the PC2 for 3Tx CA_n5A-n77A and CA_n25A-n77A is proposed in the 3Tx basket WID in this meeting according to the request) |  |  |  |  |  |  |  |  |
| , |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the missing combination CA_25A-n77A and CA_n5-n77 to enable ULCA+TxD scenario with 3Tx. - Add the missing Table header in clause 6.2L.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ULCA+TxD 3Tx scenario is not introduced for CA_25A-n77A and CA_n5-n77. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2L.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### << Start of changes >>

6.2L.3	Transmitter power for inter-band UL CA with Tx Diversity

6.2L.3.1	UE maximum output power for inter-band UL CA with Tx Diversity

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors and all UL CCs, as specified in Table 6.2L.3.1-1. The period of measurement shall be at least one sub frame (1 ms).

Table 6.2L.3.1-1: UE Power Class for inter-band UL CA with Tx Diversity in one frequency band

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n5A-n77A | 293 | +2/-3 |  |  | 23 | +2/-3 |
| CA_n8A-n78A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n25A-n41A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n25A-n77A | 293 | +2/-3 |  |  | 23 | +2/-3 |
| CA_n26A-n78A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n41A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n78A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n71A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. NOTE 4: Power class 3 is the default power class unless otherwise stated. NOTE 5: FWA form factor is targeted unless otherwise stated. |  |  |  |  |  |  |

### << End of changes >>