3GPP TSG-RAN WG4 Meeting #110	R4-2400240

Athens, Greece Feb 26 – Mar 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Supporting Simulations for BS Demodulation on Less than 5 MHz

Agenda item:	8.10.6.2

Document for:	Information

Introduction

The introduction of Less than 5 MHz for Rel-18 has been outlined in the WID [1], with an aim to increase the utilization of 5G NR in bands that support less than 5MHz such as Rail communications, public protection and disaster relief (PPDR), utilities or other requirements.

RAN4 Demodulation will specify the performance of Less than 5MHz to enable this feature, specifically RAN4 will focus on the scope, channel scenario, rank, operating SNR, carrier bandwidth and other implementation aspects to define both the UE and BS demodulation performance of Less than 5MHz.

The main objectives from the WID [1] are as follows:

Objective of SI or Core part WI or Testing part WI The following objectives shall be included for dedicated FDD spectrum in FR1: - Identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN1]: - Restrict to subcarrier spacing of 15kHz and the use of normal cyclic prefix. - For SSB: - Reuse PSS/SSS specification without puncturing. - PBCH based on current design - Identify and specify necessary minimum changes to PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization. - Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n8, n26 and n28: - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum. - Minimize impact on RF requirements: - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator). - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n8, n26 and n28. - Specify RRM requirements while minimizing specification impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz.

In addition, the <5MHz work item defines the following (highlighted in green) objectives related to BS Demodulation ([1]):

“Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations from approximately 3MHz up to below 5MHz, corresponding to the core requirements: - Specify necessary RRM performance requirements (RAN4) - Specify necessary UE demodulation performance and CSI reporting requirements (RAN4) - Specify necessary BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4)”

In the following contribution we will provide simulation results to enable RAN4 to specify BS demodulation requirements related to less than 5MHz CBW.

Discussion

Background

This paper presents simulations to support our companion TDoc [2].

PUSCH

The following table represents the simulation campaign from Nokia on less than 5MHz to support the definition of requirements for PUSCH.

| Waveform | PRB | DMRS | Channel | MCS | SNR @ 70% TPUT |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 16 | 11.62 |
|  |  |  | TDLA 30-10 | 4 | 0.48 |
|  |  | 1+0 |  | 2 | -2.5 |
|  |  |  |  | 16 | - |
|  | 12 PRB |  |  | 4 | 1.56 |
|  |  |  | TDLB 100-400 | 2 | -0.72 |
|  |  |  |  | 16 | 13.17 |
|  |  | 1+1 |  | 4 | 1.13 |
| CP-OFDM |  |  |  | 2 | -2.34 |
|  |  |  |  | 16 | 11.75 |
|  |  |  | TDLA 30-10 | 4 | 0.36 |
|  |  | 1+0 |  | 2 | -2.52 |
|  |  |  |  | 16 | - |
|  | 15 PRB |  |  | 4 | 1.47 |
|  |  |  | TDLB 100-400 | 2 | -0.7 |
|  |  |  |  | 16 | 13.39 |
|  |  | 1+1 |  | 4 | 1.11 |
|  |  |  |  | 2 | -2.33 |

PUCCH

The following tables present the performance where ACK missed detection probability or DTX to ACK probability shall not exceed 1% at the SNR given for various PUCCH formats, in TDLC 300-100 channels with FH enabled.

Format 2

Number of PRBs when using 1 OFDM symbol is 4, when using 2 OFDM symbol is 9.

Number of UCI information bits when using 1 OFDM symbol is 4, when using 2 OFDM symbol is 22.

| Number of Tx Antenna | Number of Rx Antenna | Number of OFDM Symbol | 15 PRB (SNR) | 12 PRB (SNR) |
| --- | --- | --- | --- | --- |
| 1 | 2 | 1 | -1.44 | -1.22 |
| 1 | 2 | 2 | -3.31 | -3.31 |
| 1 | 4 | 1 | -6.16 | -6.16 |
| 1 | 4 | 2 | -7.97 | -7.97 |
| 1 | 8 | 1 | -9.98 | -10.09 |
| 1 | 8 | 2 | -11.92 | -11.92 |

Conclusion

Within this contribution we discuss the demodulation requirements for BS Demodulation on Less than 5 MHz

In the paper, no observations were made:

- References

- RP-231425 “Revised WID: NR support for dedicated spectrum less than 5 MHz”, Nokia, Nokia Shanghai

- R4-2400239  Discussion on BS Demodulation on Less than 5 MHz”, Nokia, Nokia Shanghai Bell, RAN WG4 #110, Athens, February 24