3GPP TSG-RAN WG4 Meeting # 110								                 R4-2400263

Athens, Greece, 26 Feb – 01 Mar, 2024

Title: 	Proposal for extended two DL with one or two UL co-existence study template

Source: 	Skyworks Solutions Inc.

Agenda Item:	14 Revision of the Work Plan

Document for:	Discussion

# 1	Background

In this meeting, we have shared an evaluation paper [1] that introduces a plan to provide enhanced templates for the band combination TPs which contain coexistence studies, and result in MSD specifications (1,2 and 3 DL band TPs). We also have proposed several additional technical papers that address the calculations and related tables for the different MSD types and rules in [2, 3, 4, 5, 6]. These technical contributions are where the actual calculations, rules, and interference cases to be analysed are discussed, while, in this contribution, we present a new two-DL band co-existence study template that covers one UL, along with one or two CC and two ULs that includes two or three CCs. Here we evaluate the structure, formats, and methods for using the template. The approach includes the new tables for all types of MSDs and annexes for guidelines based on [2, 3, 4, 5, 6]. It also includes the band combination from [7] as an example of a concrete use of the template.

# Discussion

This contribution is submitted for approval, but we do understand it needs some of the technical details to be agreed within the other submitted papers. Still, we think it is important as it provides an overall view of the template with examples and can be place holder to capture agreements on the structure.

## 2.1 proposed improvements to the 2DL/1or2UL coexistence analysis

In the discussion paper [1], we shared our plan for improved templates designed to improve and extend the block approval process in Release 19. Separately, we submitted discussion papers for the technical content of the proposed tables and guidelines of the template. The technical aspect of each of the MSD types can be discussed within these contributions. In this contribution, we focus on collecting the different elements to illustrate a complete template, its annexes, and an example based on CA_n1-n104. The objective is to present the structure, flow, and procedures for filling the template.

List of proposed enhancements for 2DL/1or2UL bands block approval TP template for Release 19 as discussed in [1]:

- Addition at the end of section “5.XX.1.1 Operating bands for CA” of a table to capture and justify potential frequency range restriction(s) for the co-existence analysis as discussed in [2]

- Addition at the end of section “5.XX.1.2 Channel bandwidths per operating band for CA” of:

  - A question related to the support of SimRx/Tx, or otherwise for TDD/TDD cases.

  - A table that sorts the applicable UL configuration and their related MSD studies

- For the 2DL/1UL section:

  - Addition of a specific section for “Co-existence studies for 1UL band with 1CC”

    - UL harmonic and harmonic mixing tables are updated in a matrix form with additional guidelines as discussed in [3]

    - A new calculation table for cross-band isolation MSD is added, as discussed in [4]

  - Addition of a specific section for “Co-existence studies for 1UL band with 2CC intra-band”

    - The IMD range table is updated and simplified as discussed in [5]

- For this meeting the delta T/R, REFSENS and OOB exception sections are not covered, However, these may be part of further guidelines/proposals on how to design MSD test points.

- For the 2DL/1UL section:

  - Slightly updated 2DL 2UL with 1CC/band IMD table, with an analysis and Note section

  - Added section “5.XX.2.2.1	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)”, with a calculation table that includes an analysis and Note section, as discussed in [6]

- For the annex section to be added to the TP template, and to the related TR (the annex section of each TP is not copied in the TR and can be omitted in the TP submission)

  - Addition of annex A, covering the applicable frequency range restrictions as discussed in [2]

  - Addition of annex B, covering band group definition and criteria as discussed in [4,6].

In future meetings, we may decide to propose further enhancement to the two DL band TP for other sections, most notably with a focus on guidelines for the design of the REFSENS test points related to the different MSD types.

Noted that a defined part of the template is relevant to derive the one and three DL band templates:

- The new calculation table for cross-band isolation MSD added as discussed in [4], is relevant to the FDD intra-band DLCA with 1UL CC.

- The section for “Co-existence studies for 1UL band with 2CC intra-band” with the IMD range table is updated and simplified, as discussed in [5], is relevant to the FDD intra-band DLCA with intra-band ULCA.

- The 2DL/1UL section, with the updated 2DL 2UL with 1CC/band IMD table is relevant to the 3DL/2ULwith 1CC/band case.

- Similarly, the section “5.XX.2.2.1 Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)”, as discussed in [6], is relevant to the 3DL/2UL 3CC case.

## 2.2 proposed template for 2DL/1or2UL block approval TPs

In the proposed template the fields to be completed by the proponent in are highlighted in grey, added parts are designated with blue text.

The note part of the table may become redundant for the different TPs captured in the TR. The rapporteur updating the TR may decide to omit the Note section of the tables when copying the TPs in the related TR.

************************* Start of template **********************************

## 5.XX		CA_nX-nY

### 5.XX.1	Common for 1 band UL and 2 bands UL CA

#### 5.XX.1.1 Operating bands for CA

Note: For band definition, relevant rows can be copied directly from Table 5.2-1 in 38.101-1 to Table 5.XX.1.1-1 below.

Table 5.XX.1.1-1: CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| nX | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nY | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |

Note: For certain band combinations, frequency range restrictions may be applicable when the band combination can be uniquely identifiable to a region or a country. Operator holding frequency range restrictions are not allowed and frequency ranges derived from additional emission requirements (NS) are not relevant. Related frequency range restrictions can be found in annex A. Such frequency range restriction(s) are captured in Table 5.XX.1.1-2, and are used for the coexistence study tables.

Table 5.XX.1.1-2: Applicable frequency range restrictions for coexistence study

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| nX | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| Justification | text |  |  |

#### 5.XX.1.2	Channel bandwidths per operating band for CA

Table 5.XX.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_nXA-nYA | CA_n1A-n102A | nX | Channel BWs | X |
|  |  | nY | Channel BWs |  |
| CA_nX(A/B/C(2A)-nY(A/B/C(2A) | CA_nX(A/B/C(2A)-nY(A/B/C(2A) | nX | Channel BW or CA BCS | X |
|  |  | nY | Channel BW or CA BCS |  |

To determine the coexistence study cases to be analyzed, the following question and UL configuration types table should be completed. The allowable UL configurations are listed in annex B.

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? XXX

Table 5.XX.1.2-2: Supported UL configurations and required coexistence studies

| Type of UL Configuration | UL Configuration | Power class | Condition | Coexistence analysis to be performed | Coexistence study Tables |
| --- | --- | --- | --- | --- | --- |
| 1UL band with 1CC | nX, nY | 3 3 | One band is FDD Or SimRx/Tx TDD/TDD | UL harmonic, harmonic mixing and cross-band MSD should be studied | 5.XX.1.3.1-1 5.XX.1.3.1-2 |
| 1UL band with 2CC | CA_nXornYB/C/(2A) | 3 |  | IMDs of the two intra-band UL CCs | 5.XX.1.3.2-1 |
| 2UL bands 1CC per band | CA_nXA-nYA | 3 | FDD/FDD Or FDD/TDD | IMDs of the two UL bands | 5.XX.2.2.1-1 |
| 2UL bands incl. 1UL band with 2CC | CA_nXA/B/C-nYA/B/C | 3 3 |  | Triple beat of the three UL CCs if the two bands are in adjacent band groups | 5.XX.2.2.2-1 |

#### 5.XX.1.3	Co-existence studies

#### 5.XX.1.3.1	Co-existence studies for 1UL band with 1CC

Table 5.XX.1.3.1-1 summarizes frequency ranges of UL harmonics and/or harmonic mixing collisions may occur for CA_nX-nY.

Table 5.XX.1.3.1-1: UL/DL harmonics collision table for CA_nX-nY

| UL/DL harmonics |  | nX | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | fULlow | 2*fULlow | 3*fULlow | 4*fULlow | 5*fULlow |  |
| nY | fLow | fHigh | fULhigh | 2*fULhigh | 3*fULhigh | 4*fULhigh | 5*fULhigh |  |
| DL1 | fDLlow | fDLhigh | N/A |  |  |  |  | UL Harmonic |
| DL22 | 2*fDLlow | 2*fDLhigh |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL32 | 3*fDLlow | 3*fDLhigh |  |  | N/A |  | N/A |  |
| DL42 | 4*fDLlow | 4*fDLhigh |  | N/A |  | N/A | N/A |  |
| DL52 | 5*fDLlow | 5*fDLhigh |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
|  |  | fLow | fULlow | 2*fULlow | 3*fULlow | 4*fULlow | 5*fULlow |  |
| nX | fLow | fhigh | fULhigh | 2*fULhigh | 3*fULhigh | 4*fULhigh | 5*fULhigh |  |
| DL1 | fDLlow | fDLhigh | N/A |  |  |  |  | UL Harmonic |
| DL22 | 2*fDLlow | 2*fDLhigh |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL32 | 3*fDLlow | 3*fDLhigh |  |  | N/A |  | N/A |  |
| DL42 | 4*fDLlow | 4*fDLhigh |  | N/A |  | N/A | N/A |  |
| DL52 | 5*fDLlow | 5*fDLhigh |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| Note 1: When a collision is detected with an overlap of UL(X) range with DL(Y) range, the UL(X)/DL(Y) cell is marked “D” for direct hit. When UL(X) range is less than an X*ULCBW away from DL(Y) range, the UL(X)/DL(Y) cell is marked “N” for Near miss, and only the orders where X+Y<5 are considered. Note 2: Conditions on UL frequency apply for some ULX/DLY harmonic mixing cases for PC3/5: UL>2GHz for UL3/DL2, all UL except 450MHz bands for UL4/DL3, UL>4.2GHz for UL1/DL4, UL>6.4GHz for UL3/DL4, UL>1.5GHz for UL2/DL5. |  |  |  |  |  |  |  |  |

Table 5.XX.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for CA_nX-nY.

Table 5.XX.1.3.1-2: Cross-band isolation analysis of CA_nXA-nYA with nX and nY UL

| Bands3 | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) |  |  |  |  |
| fDL (MHz) |  |  |  |  |
| CBW (MHz)2 |  |  |  |  |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) |  |  |  |  |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) |  |  |  |  |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) |  |  |  |  |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) |  |  |  |  |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) |  |  |  |  |
| Analysis | text |  |  |  |
| [Note 1: Even if there is no overlap up to ACLR5, MSD due to transmitter noise floor should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in annex B -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest harmonic 2 frequency -As an indicative threshold, if >45dB rejection at the DL band frequency can be guaranteed assuming -130dBm/Hz TX noise floor level, the MSD should be negligible] Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) and is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. [Note 3: If the band combination can be uniquely identified to a given region/country, UL or DL frequency range restriction may apply] |  |  |  |  |

5.XX.1.3.2	Co-existence studies for 1UL band with 2CC intra-band

Table 5.XX.1.3.2-1 provides the one UL band with intra-band ULCA IMD frequency ranges for the CA_nX/YB/C/(2A) UL configuration for DL CA_nX-nY.

Table 5.XX.1.3.2-1: Intra-band ULCA IMD overlap for CA_nXA/B/C/(2A)-nYA/B/C/(2A)/CA_nX/YB/C/(2A) DL/UL

| All in MHz | flow/Min | fhigh/Max | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| fUL5 |  |  | Order | flow | fhigh |
| 2CCBW1 |  |  | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| fDL |  |  |  |  |  |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  |  |  |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  |  |  | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  |  |  | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
| Analysis | text |  |  |  |  |
| Note 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow). Note 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, an IMD order up to 9/7 should be considered, with MPR assumed - For intra-band ULCA within a FDD band, an IMD order up to 13 should be considered, with MPR not assumed. Note 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) -IMD2 is not considered assuming CA with 450MHz bands is not considered -IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz [-IMD6 is considered case by case for FDD or SimRx/Tx TDD bands <1.68GHz.] Note 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band. [Note 5: If the band combination can be uniquely identified to a given region/country, the UL frequency range restriction may apply.] |  |  |  |  |  |

*************** Delta T/R and 2DL/1UL REFSENS sections omitted in this version of the proposal ****************

### 5.XX.2		Specific for 2 bands UL CA

#### 5.XX.2.1	Maximum output power for inter-band CA

Table 5.XX.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_nXA-nYA | 23 | +2/-3 |
| CA_nX/YA/B/C/(2A) | 23 | +2/-3 |
| CA_nXA/B/C-nXA/B/C | 23 | +2/-3 |

#### 5.XX.2.2		UE co-existence

#### 5.XX.2.2.1	Co-existence studies for 2UL band with 1CC per band

Table 5.XX.2.2.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_ nXA-nYA.

Table 5.XX.2.2-1: IMD analysis

| Bands | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low | fx_high | fy_low | fy_high |
| UL (MHz) |  |  |  |  |
| DL (MHz) |  |  |  |  |
| IMD2 products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD2 (MHz) |  |  |  |  |
| IMD3 products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD3 (MHz) |  |  |  |  |
| IMD3 products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD3 (MHz) |  |  |  |  |
| IMD4 products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD4 (MHz) |  |  |  |  |
| IMD4 products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD4 (MHz) |  |  |  |  |
| IMD4 products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD4 (MHz) |  |  |  |  |
| IMD5 products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD5 (MHz) |  |  |  |  |
| IMD5 products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD5 (MHz) |  |  |  |  |
| IMD5 products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD5 (MHz) |  |  |  |  |
| IMD5 products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD5 (MHz) |  |  |  |  |
| Analysis | text |  | text |  |
| Note: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

#### 5.XX.2.2.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.XX.2.2.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products interference analysis for CA_nXA/B/C-nYA/B/C with nX/YB/C transmitting with a XXXMHz maximum instantaneous bandwidth.

Table 5.XX.2.2.2-1: Triple beat IMD analysis of CA_nXA/B/C-nYA/B/C

| Band / CA1 | nX |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| F_UL (MHz)3 |  |  |  |  |
| F_DL (MHz)3 |  |  |  |  |
| 2CCBW (MHz)2 | N/A | N/A |  |  |
| IMD3 products | fx_low-max2CCBW | fx_low | fx_high | fx_high+max2CCBW |
| IMD3 (MHz) |  |  |  |  |
| Analysis | text |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in Annex D, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum/maximum separation BW is 0MHz / Min(fy_high-fy_low, maximum aggregated BW), respectively. [Note 3: If the band combination can be uniquely identified to a given region/country, the UL or DL frequency range restriction may apply.] |  |  |  |  |

*********************REFSENS section omitted in this version of the template *********

************************* End of template **********************************

## 2.2 proposed annex information for 2DL/1or2UL TPs and TR

The proposed annex sections are meant to be added to the TP template and to the associated TR. However, these sections should not be copied in the TR from the TPs. This section is provided as guidance, and to assist the proponent with filling in the TP. When submitting the TP, the annex section can be omitted.

These annexes and tables should not be captured in RAN4 TS and are solely for use in band combination TPs and TRs.

************************* Start of annex **********************************

# Annexes

Note: These annexes are provided as a reference for the coexistence studies to be performed for block approval TPs, for a 2DL 1or2UL band combination. The annexes are not meant to be copied in the TR, as there is already an annex to the related TR, and as such, the annex section may be removed when submitting the TP.

Annex A

For certain band combinations, frequency range restrictions may be applicable when the band combination can be uniquely identifiable to a region or a country. Operator holding frequency range restrictions are not allowed, and frequency ranges derived from additional emission requirements (NS) are not relevant. Related frequency range restrictions are captured in Table X, and are used for the coexistence study tables.

The table provides and the region(s)/countries where the restricted frequency range applies per bands:

- The nominal UL/DL frequency range for the band in the TS 38.101-1

- The restricted UL/DL frequency range applicable

- Which specific band(s) associated with the band in the first column can help determine if the band combination is unique to the given region, and in some cases which other condition may apply. This list of bands is indicative and may not be considered as exhaustive.

Table A: Applicable frequency range restrictions for coexistence studies

| NR Band | Uniquely Identifiable Region | Uplink (UL) band | Downlink (DL) band | Restricted range (UL) | Restricted range (DL) | Associated band or condition for region/country uniqueness |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Flow-Fhigh (MHz) |  |  |  |  |
| n24 | North America | 1626.5-1660.5 | 1525-1559 | 1627.5-1637.5 1646.5-1656.5 | 1526-1536 | Only used in North America |
| n28 | Europe | 703-748 | 758-803 | 703-733 | 758-788 | [(n)7,20,38,65,75/76,91,92,109] |
| n28 | Japan |  |  | 718-748 | 773-803 | [(n)11,18,19,21,74] |
| n40 | Japan | 2300-2400 | 2300-2400 | 2330-2370 | 2330-2370 | [(n)11,18,19,21,74] |
| n41 | China | 2496-2690 | 2496-2690 | 2515-2675 | 2515-2675 | [(n)34(95),39(98),50/51] |
| n41 | Japan |  |  | 2545-2645 | 2545-2645 | [(n)11,18,19,21,74 or when n90 is signalled] |
| n46 | North America | 5150-5925 | 5150-5925 | 5150-5350 5470-5850 | 5150-5350 5470-5850 | [(n)2,12,13,14,24,25,30,48/49,66(86),70,71,85] |
| n46 | Rest of the world |  |  | 5150-5350 5470-5730 | 5150-5350 5470-5730 | [(n)1,3,8,11,18,19,20,21,28,34(95), 39(98),40,50/51,74,75/76,91,92,93,94,109] |
| n77 | Japan | 3300-4200 | 3300-4200 | 3400-4100 | 3400-4100 | [(n)1,3,11,18,19,21,40,74] |
| n77 | North America |  |  | 3450-3980 | 3450-3980 | [(n)2,12,13,14,24,25,30,48/49,66(86),70,71,85] |
| n78 | China | 3300-3800 | 3300-3800 | 3300-3600 | 3300-3600 | [(n)34(95),39(98),50/51] |
| n78 | Europe, Asia except China |  |  | 3400-3800 | 3400-3800 | [(n)7,11,18,19,20,21,28,38,65,74,75/76,109] |
| n79 | China | 4400-5000 | 4400-5000 | 4800-5000 | 4800-5000 | [(n)34(95),39(98),50/51] |
| n79 | Japan |  |  | 4500-4900 | 4500-4900 | [(n)11,18,19,21,74] |
| n102 | N/A | 5925-6425 | 5925-6425 | 5945-6425 | 5945-6425 | Band definition for in band PSD |

Annex B

For some MSD cases, a criterion applies in terms of band group as defined in Table B:

- For 2DL 1UL/1CC cross band isolation, and for the Tx noise floor MSD to be considered, the two bands should be part of the same or adjacent band group.

- For 2DL 2UL/3CC triple beat MSD to be considered, the two bands should be part of the same or adjacent band group

- For 3DL 2UL/3CC triple beat MSD to be considered, the third band should be part of the same or adjacent band group, rather than one from the UL band

Table B: Band group definition for same or adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a (LB) | FR1-b (MB) | FR1-c (HB) | FR1-d (VHB) | FR1-e (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

************************* End of annex **********************************

## 2.3 Example of a resulting TP for CA_n1-n102

The chosen example is based on TP [7] on CA_n1-n102 and DC_n1-n102, as it contains all of the different types of UL configurations, and can be considered to be fairly complete in terms of MSD studies. Although it does not result in an actual MSD specified for all the MSD types, it provides a good example of how the proposed template could function for a concrete case.

Some of the text is color coded with highlights to provide indication on how to derive the information to be filled in:

- Yellow highlights correspond to data related to the bands definitions

- Green highlights correspond to data related to frequency range restrictions

- Cyan highlights correspond to data related to the BCS DL, UL, channel bandwidths, and intra-band CA BW from the BCS definitions

- Teal highlights correspond to data that can be calculated with the equations provided

- Pink highlights correspond to text that is provided in support of the analysis of the table results.

Some commentary text can be further added below each table. In this example, we have retained the Note section of the tables, as this information may be a useful reference for the TP reviewers during the flagging process.

************************* Start of example TP **********************************

## 5.XX		CA_n1-n102

### 5.XX.1	Common for 1 band UL and 2 bands UL CA

#### 5.XX.1.1 Operating bands for CA

Note: For band definition, relevant rows can be copied directly from Table 5.2-1 in 38.101-1 to Table 5.XX.1.1-1 below.

Table 5.XX.1.1-1: CA band combination of band n1+n102

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n102 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD |

Note: For certain band combinations, frequency range restrictions may be applicable when the band combination can be uniquely identifiable to a region or a country. Operator holding frequency range restrictions are not allowed, and frequency ranges derived from additional emission requirements (NS) are not relevant. Related frequency range restrictions can be found in annex A. Such frequency range restriction(s) are captured in Table 5.XX.1.1-2 and are used for the coexistence study tables.

Table 5.XX.1.1-2: Applicable frequency range restrictions for coexistence study

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n102 | 5945 MHz – 6425 MHz | 5945 MHz – 6425 MHz | TDD |
| Justification | In 38.101-1, n102 is defined as 5945-6425MHz for the in-band PSD requirement to account for the lower 20MHz guard-band to the ITS channels |  |  |

#### 5.XX.1.2	Channel bandwidths per operating band for CA

Table 5.XX.1.2-1: Supported bandwidths per CA band combination of band n1+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n102A | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n102(2A) | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n102B | CA_n1A-n102A CA_n1A-n102B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n102C | CA_n1A-n102A CA_n1A-n102C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n102D | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n102E | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

To determine the coexistence study cases to be analyzed, the following question and UL configuration types table should be completed. The allowable UL configurations are listed in annex B.

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? N-A

Table 5.XX.1.2-2: Supported UL configurations and required coexistence studies

| Type of UL Configuration | UL Configuration | Power class | Condition | Coexistence analysis to be performed | Coexistence study Tables |
| --- | --- | --- | --- | --- | --- |
| 1UL band with 1CC | n1, n102 | 3 3 | One band is FDD Or SimRx/Tx TDD/TDD | UL harmonic, harmonic mixing and cross-band MSD should be studied | 5.XX.1.3.1-1 5.XX.1.3.1-2 |
| 1UL band with 2CC | CA_n102B, CA_n102C in CA_n1A-n102B, CA_n1A-n102C | 3 |  | IMDs of the two intra-band UL CCs | 5.XX.1.3.2-1 |
| 2UL bands 1CC per band | CA_n1A-n102A | 3 | FDD/FDD Or FDD/TDD | IMDs of the two UL bands | 5.XX.2.2.1-1 |
| 2UL bands incl. 1UL band with 2CC | CA_n1A-n102B CA_n1A-n102C | 3 3 |  | Triple beat of the three UL CCs, if the two bands are in adjacent band groups | 5.XX.2.2.2-1 |

#### 5.XX.1.3	Co-existence studies

#### 5.XX.1.3.1	Co-existence studies for 1UL band with 1CC

Table 5.XX.1.3.1-1 summarizes frequency ranges of UL harmonics and/or harmonic mixing collisions may occur for CA_n1-n102.

Table 5.XX.1.3.1-1: UL/DL harmonics collision table for CA_n1-n102

| UL/DL harmonics |  | n102 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 5945 | 11890 | 17835 | 23780 | 29725 |  |
| n1 | fLow | fHigh | 6425 | 12850 | 19275 | 25700 | 32125 |  |
| DL1 | 2110 | 2170 | N/A | - | - | - | - | UL Harmonic |
| DL22 | 4220 | 4340 | - | N/A | - | N/A | N/A | Harmonic Mixing |
| DL32 | 6330 | 6510 | D | - | N/A | - | N/A |  |
| DL42 | 8440 | 8680 | - | N/A | - | N/A | N/A |  |
| DL52 | 10550 | 10850 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Direct hit n102 UL1 / n1 DL3 |  |  |  |  |  |
| UL/DL harmonics |  | n1 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
|  |  | fLow | 1920 | 3840 | 5760 | 7680 | 9600 |  |
| n102 | fLow | fHigh | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 5945 | 6425 | N/A | - | N | - | - | UL Harmonic |
| DL22 | 11890 | 12850 | - | N/A | - | N/A | N/A | Harmonic Mixing |
| DL32 | 17835 | 19275 | - | - | N/A | - | N/A |  |
| DL42 | 23780 | 25700 | - | N/A | - | N/A | N/A |  |
| DL52 | 29725 | 32125 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Near miss n1 UL3 / n102 DL1 (5MHz) |  |  |  |  |  |
| Note 1: When a collision is detected with an overlap of UL(X) range with DL(Y) range, the UL(X)/DL(Y) cell is marked “D” for direct hit. When UL(X) range is less than an X*ULCBW away from DL(Y) range, the UL(X)/DL(Y) cell is marked “N” for Near miss and only the orders where X+Y<5 are considered. Note 2: Conditions on UL frequency apply for some ULX/DLY harmonic mixing cases for PC3/5: UL>2GHz for UL3/DL2, all UL except 450MHz bands for UL4/DL3, UL>4.2GHz for UL1/DL4, UL>6.4GHz for UL3/DL4, UL>1.5GHz for UL2/DL5 |  |  |  |  |  |  |  |  |

Based on the above table:

- n1 3rd UL harmonic falls 5MHz short of the band n102 DL, and thus may be victim of the ACLR1 range of the third harmonic. Given that n102 is a shared spectrum access band, a REFSENS exclusion range can be defined.

- n1 3rd DL harmonic mixing falls on the band n102 UL, thus harmonic mixing MSD should be considered.

Table 5.XX.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for CA_n1-n102.

Table 5.XX.1.3.1-2: Cross-band isolation analysis of CA_n1A-n102A with n1 and n102 UL

| Bands3 | n1 |  | n102 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1920 | 1980 | 5945 | 6425 |
| fDL (MHz) | 2110 | 2170 | 5945 | 6425 |
| CBW (MHz)2 | 5 | 50 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1870 | 2020 | 5845 | 6525 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1820 | 2070 | 5745 | 6625 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1770 | 2120 | 5645 | 6725 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1720 | 2170 | 5545 | 6825 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1670 | 2220 | 5445 | 6925 |
| Analysis | There is no overlap of the band n1 ACLR range with the band n102 DL up to order 5 and there is sufficient rejection of the transmitter noise floor at band n102 so no cross-band MSD is needed. |  | There is no overlap of the band n102 ACLR range with the band n1 DL up to order 5 and there is sufficient rejection of the transmitter noise floor at band n1 so no cross-band MSD is needed. |  |
| [Note 1: Even if there is no overlap up to ACLR5, MSD due to transmitter noise floor should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in annex B -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest harmonic 2 frequency -As an indicative threshold, if >45dB rejection at the DL band frequency can be guaranteed, assuming -130dBm/Hz TX noise floor level, the MSD should be negligible] Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. [Note 3: If the band combination can be uniquely identified to a given region/country, the UL or DL frequency range restriction may apply] |  |  |  |  |

Based on the above table: There is no case with the UL band ACLR range overlapping with the DL range and the band 1 and band n102 UL filters, together with the diplexer isolation, provides sufficient rejection of the other band DL frequency range and justifies the conclusion to ignore the UL noise floor impact.

5.XX.1.3.2	Co-existence studies for 1UL band with 2CC intra-band

Table 5.XX.1.3.2-1 provides the one UL band with intra-band ULCA IMD frequency ranges for the CA_n102B and CA_n102C UL configurations, which are part of CA_n1-n102B and CA_n1-n102C requested cases.

Table 5.XX.1.3.2-1: Intra-band ULCA IMD overlap for CA_n1A-n102C/CA_n102C DL/UL

| All in MHz | flow/Min | fhigh/Max | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| fUL5 | 5945 | 6425 | Order | flow | fhigh |
| 2CCBW1 | 0 | 160 | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| fDL | 2110 | 2170 |  | 0 | 160 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 5785 | 6585 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 5625 | 6745 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 5465 | 6905 |  | 11730 | 13010 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 5305 | 7065 |  | 11570 | 13170 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 5145 | 7225 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 4985 | 7385 |  | 17675 | 19435 |
| Analysis | The DL victim band is lower than the TDD intra-band ULCA band, which is contiguous ULCA. Thus, only the close n102C UL IMD3 to 9 should be considered, and there is no overlap with the DL band n1 |  |  |  |  |
| Note 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow). Note 2: The close to UL IMD range is the most critical when the victim DL band is in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, an IMD order up to 9/7 should be considered and with MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered, with MPR not assumed. Note 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) -IMD2 is not considered assuming CA with 450MHz bands is not considered -IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz [-IMD6 is considered case-by-case basis for FDD or SimRx/Tx TDD bands <1.68GHz.] Note 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band. [Note 5: If the band combination can be uniquely identified to a given region/country, the UL frequency range restriction may apply.] |  |  |  |  |  |

Since this is a contiguous ULCA TDD case and the UL band is above the DL band, only up to IMD9 is considered for close to UL products while BB and close to harmonic products can be ignored. Based on the above table: There is no case with the 2CC intra-band IMD range overlapping with the DL range of band 1.

********* Delta T/R, 2DL/1UL REFSENS and OOB exception sections omitted in this version of the example *********

### 5.XX.2		Specific for 2 bands UL CA

#### 5.XX.2.1	Maximum output power for inter-band CA

Table 5.XX.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n102A | 23 | +2/-3 |
| CA_n1A-n102B | 23 | +2/-3 |
| CA_n1A-n102C | 23 | +2/-3 |

#### 5.XX.2.2		UE co-existence

#### 5.XX.2.2.1	Co-existence studies for 2UL band with 1CC per band

Table 5.XX.2.2.1-1 provides the two UL bands, with one CC per band IMD interference analysis for CA_ n1A-n102A.

Table 5.XX.2.2-1: IMD analysis

| Bands | n1 |  | n102 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low | fx_high | fy_low | fy_high |
| UL (MHz) | 1920 | 1980 | 5945 | 6425 |
| DL (MHz) | 2110 | 2170 | 5945 | 6425 |
| IMD2 products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD2 (MHz) | 3965 | 4505 | 7865 | 8405 |
| IMD3 products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD3 (MHz) | 2585 | 1985 | 9910 | 10930 |
| IMD3 products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD3 (MHz) | 9785 | 10385 | 13810 | 14830 |
| IMD4 products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD4 (MHz) | 665 | 5 | 15855 | 17355 |
| IMD4 products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD4 (MHz) | 9010 | 7930 | 15730 | 16810 |
| IMD4 products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD4 (MHz) | 11705 | 12365 | 19755 | 21255 |
| IMD5 products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD5 (MHz) | 23780 | 21800 | 1975 | 1255 |
| IMD5 products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD5 (MHz) | 15435 | 13875 | 5950 | 7090 |
| IMD5 products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD5 (MHz) | 25700 | 27680 | 13625 | 14345 |
| IMD5 products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD5 (MHz) | 21675 | 23235 | 17650 | 18790 |
| Analysis | IMD3 fall in FDD band n1 DL: MSD may occur |  | IMD5 fall in TDD band n102 DL: no MSD possible |  |
| Note: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the table 5.XX.2.2.1-1, one 3rd order intermodulation product fall inside the DL band of n1.

#### 5.XX.2.2.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.XX.2.2.2-1 provides the two UL bands with one band, with a 2CC intra-band uplink CA triple beat products interference analysis provided for CA_n1A-n102C including n102C transmitting with a 160MHz maximum instantaneous bandwidth.

Table 5.XX.2.2.2-1: Triple beat IMD analysis of CA_n1A-n102C

| Band / CA1 | n1 |  | CA_n102C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| F_UL (MHz)3 | 1920 | 1980 | 5945 | 6425 |
| F_DL (MHz)3 | 2110 | 2170 | 5945 | 6425 |
| 2CCBW (MHz)2 | N/A | N/A | 0 | 160 |
| IMD3 products | fx_low-max2CCBW | fx_low | fx_high | fx_high+max2CCBW |
| IMD3 (MHz) | 1760 | 1920 | 1980 | 2140 |
| Analysis | There is a triple beat product of n1 UL with n102C 2CC UL falling in n1 DL. However, n1 and n102 are not part of adjacent band groups and thus MSD is not required. |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in Annex D, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum/maximum separation BW is 0MHz / Min(fy_high-fy_low, maximum aggregated BW) respectively. [Note 3: If the band combination can be uniquely identified to a given region/country, the UL or DL frequency range restriction may apply] |  |  |  |  |

The uplinks of n1A and n102C have a 1st order triple beat falling inside n1 DL. As stated for figure 1 in [2] the triple beat case is not eligible for MSD if the two UL bands aren’t part of the same or adjacent band group.

*********************REFSENS section omitted in this version of the example *********

************************* End of example TP **********************************

# Conclusion

In this contribution, we provided a draft overview of the proposed template for two DL with 1 or 2UL TP to TR in the block approval process. It includes calculations and guidelines for the analysis of the co-existence studies for all the potential MSD types. The goal is to provide defined templates that are designed to improve the block approval process quality in Release 19.

These are the proposals for TP templates, and it annexes illustrated by the CA_n1-n102 example.

List of proposed enhancements for 2DL/1or2UL bands block approval TP template for Release 19 as discussed in [1]:

- Addition at the end of section “5.XX.1.1 Operating bands for CA” of a table to capture and justify potential frequency range restriction(s) for the co-existence analysis as discussed in [2]

- Addition at the end of section “5.XX.1.2 Channel bandwidths per operating band for CA” of:

  - A question related to the support of SimRx/Tx, or otherwise for TDD/TDD cases.

  - A table that sorts the applicable UL configuration and their related MSD studies

- For the 2DL/1UL section:

  - Addition of a specific section for “Co-existence studies for 1UL band with 1CC”

    - UL harmonic and harmonic mixing tables are updated in a matrix form with additional guidelines as discussed in [3]

    - A new calculation table for cross-band isolation MSD is added, as discussed in [4]

  - Addition of a specific section for “Co-existence studies for 1UL band with 2CC intra-band”

    - The IMD range table is updated and simplified as discussed in [5]

- For this meeting the delta T/R, REFSENS and OOB exception sections are not covered, However, these may be part of further guidelines/proposals on how to design MSD test points.

- For the 2DL/1UL section:

  - Slightly updated 2DL 2UL with 1CC/band IMD table, with an analysis and Note section

  - Added section “5.XX.2.2.1	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)”, with a calculation table that includes an analysis and Note section, as discussed in [6]

- For the annex section to be added to the TP template, and to the related TR (the annex section of each TP is not copied in the TR and can be omitted in the TP submission)

  - Addition of annex A, covering the applicable frequency range restrictions as discussed in [2]

  - Addition of annex B, covering band group definition and criteria as discussed in [4,6].

# References

[1] R4-2320868 Improved table template for 1UL/CC and 2UL/CC MSD studies, Skyworks Solutions Inc., Nokia, RAN4#110

[2] R4-2400262 On applicable UL-DL frequency range restrictions for co-existence studies, Skyworks Solutions Inc., RAN4#110

[3] R4-2400259 On harmonic mixing orders and analysis, Skyworks Solutions Inc., Nokia, RAN4#110

[4] R4-2400258 On cross band isolation MSD analysis, Skyworks Solutions Inc., Nokia, RAN4#110

[5] R4-2400260 On simplifying analysis for 2DL-1 band intra-band ULCA IMD products, Skyworks Solutions Inc., Nokia, RAN4#109

[6] R4-2400261 On simplifying analysis for triple beat products, Skyworks Solutions Inc., Nokia, RAN4#110

[7] R4-2321853 TP to TR 38.718-02-01 Addition to CA_n1-n102 and DC_n1-n102, Nokia, BT, RAN4#109