3GPP TSG-RAN4 Meeting #110	R4-2400264

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1122 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38101-3-hc0_s06-XX Bug correction for higherPowerLimitMRDC-r17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Power_Limit_CA_DC-Core |  |  |  |  |  | Date: |  |  | 2024-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Wrong Information Element higherPowerLimit-r17 (intended for NR-CA/DC) was used when introducing higherPowerLimitMRDC-r17 (intended for MR-DC (NE-DC and EN-DC) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replacing higherPowerLimit-r17 by higherPowerLimitMRDC-r17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bugs are still in the spec and higherPowerLimitMRDC-r17 cannot be supported as it is not correctly specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.1.3, 6.2B.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2B.1.3	Inter-band EN-DC within FR1

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  | 23 | +2/-3 |
| DC_1A_n5A |  |  | 23 | +2/-3 |
| DC_1A_n7A |  |  | 23 | +2/-3 |
| DC_1A_n8A |  |  | 23 | +2/-3 |
| DC_1A_n20A |  |  | 23 | +2/-3 |
| DC_1A_n28A |  |  | 23 | +2/-3 |
| DC_1A_n38A |  |  | 23 | +2/-3 |
| DC_1A_n40A |  |  | 23 | +2/-3 |
| DC_1A_n41A |  |  | 23 | +2/-3 |
| DC_1A_n50A |  |  | 23 | +2/-3 |
| DC_1A_n51A |  |  | 23 | +2/-3 |
| DC_1A_n71A |  |  | 23 | +2/-3 |
| DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |  |  | 23 | +2/-3 |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_1A_n79A DC_1A_n84A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_1A_n80A |  |  | 23 | +2/-3 |
| DC_2A_n5A |  |  | 23 | +2/-3 |
| DC_2A_n7A |  |  | 23 | +2/-3 |
| DC_2A_n12A |  |  | 23 | +2/-3 |
| DC_2A_n25A |  |  | N/A | N/A |
| DC_2A_n28A |  |  | 23 | +2/-3 |
| DC_2A_n30A |  |  | 23 | +2/-3 |
| DC_2A_n38A |  |  | 23 | +2/-3 |
| DC_2A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| DC_2A_n46A |  |  | 23 | +2/-3 |
| DC_2A_n48A |  |  | 23 | +2/-3 |
| DC_2A_n66A |  |  | 23 | +2/-3 |
| DC_2A_n71A |  |  | 23 | +2/-3 |
| DC_2A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_2A_n78A |  |  | 23 | +2/-3 |
| DC_3A_n1A DC_3C_n1A |  |  | 23 | +2/-3 |
| DC_3A_n5A DC_3C_n5A |  |  | 23 | +2/-3 |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A |  |  | 23 | +2/-3 |
| DC_3A_n8A |  |  | 23 | +2/-3 |
| DC_3A_n20A |  |  | 23 | +2/-3 |
| DC_3A_n28A DC_3C_n28A |  |  | 23 | +2/-3 |
| DC_3A_n38A |  |  | 23 | +2/-3 |
| DC_3A_n40A |  |  | 23 | +2/-3 |
| DC_3A_n41A, DC_3C_n41A, DC_3C_n41A, | 266 | +2/-3 | 23 | +2/-3 |
| DC_3A_n50A |  |  | 23 | +2/-3 |
| DC_3A_n51A |  |  | 23 | +2/-3 |
| DC_3A_n71A |  |  | 23 | +2/-3 |
| DC_3A_n77A DC_3C_n77A |  |  | 23 | +2/-3 |
| DC_3A_n78A DC_3C_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_3A_n79A DC_3C_n79A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n41 DC_3C_n80A_ULSUP-TDM_n41 |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n77A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_3A_n82A |  |  | 23 | +2/-3 |
| DC_3A_n84A |  |  | 23 | +2/-3 |
| DC_4A_n2A |  |  | 23 | +2/-3 |
| DC_4A_n5A |  |  | 23 | +2/-3 |
| DC_4A_n7A |  |  | 23 | +2/-3 |
| DC_4A_n28A |  |  | 23 | +2/-3 |
| DC_4A_n38A |  |  | 23 | +2/-3 |
| DC_4A_n41A |  |  | 23 | +2/-3 |
| DC_4A_n78A |  |  | 23 | +2/-3 |
| DC_5A_n2A |  |  | 23 | +2/-3 |
| DC_5A_n7A |  |  | 23 | +2/-3 |
| DC_5A_n12A |  |  | 23 | +2/-3 |
| DC_5A_n30A |  |  | 23 | +2/-3 |
| DC_5A_n38A |  |  | 23 | +2/-3 |
| DC_5A_n40A |  |  | 23 | +2/-3 |
| DC_5A_n48A |  |  | 23 | +2/-3 |
| DC_5A_n66A |  |  | 23 | +2/-3 |
| DC_5A_n71A |  |  | 23 | +2/-3 |
| DC_5A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n79A |  |  | 23 | +2/-3 |
| DC_7A_n1A |  |  | 23 | +2/-3 |
| DC_7A_n2A |  |  | 23 | +2/-3 |
| DC_7A_n3A |  |  | 23 | +2/-3 |
| DC_7A_n5A DC_7C_n5A |  |  | 23 | +2/-3 |
| DC_7A_n8A |  |  | 23 | +2/-3 |
| DC_7A_n20A |  |  | 23 | +2/-3 |
| DC_7A_n25A |  |  | 23 | +2/-3 |
| DC_7A_n28A |  |  | 23 | +2/-3 |
| DC_7A_n40A |  |  | 23 | +2/-3 |
| DC_7A_n51A |  |  | 23 | +2/-3 |
| DC_7A_n66A |  |  | 23 | +2/-3 |
| DC_7A_n71A |  |  | 23 | +2/-3 |
| DC_7A_n77A |  |  | 23 | +2/-3 |
| DC_7A_n78A DC_7C_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_7A_n79A |  |  | 23 | +2/-3 |
| DC_7A_n80A |  |  | 23 | +2/-3 |
| DC_8A_n1A |  |  | 23 | +2/-3 |
| DC_8A_n2A |  |  | 23 | +2/-3 |
| DC_8A_n3A |  |  | 23 | +2/-3 |
| DC_8A_n7A |  |  | 23 | +2/-3 |
| DC_8A_n20A |  |  | 23 | +2/-3 |
| DC_8A_n28A |  |  | 23 | +2/-3 |
| DC_8A_n34A |  |  | 23 | +2/-3 |
| DC_8A_n39A |  |  | 23 | +2/-3 |
| DC_8A_n40A |  |  | 23 | +2/-3 |
| DC_8A_n41A, |  |  | 23 | +2/-3 |
| DC_8A_n77A |  |  | 23 | +2/-3 |
| DC_8A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_8A_n79A DC_8A_n79C |  |  | 23 | +2/-3 |
| DC_8A_n80A |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n41 |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_11A_n1A |  |  | 23 | +2/-3 |
| DC_11A_n3A |  |  | 23 | +2/-3 |
| DC_11A_n28A |  |  | 23 | +2/-3 |
| DC_11A_n41A |  |  | 23 | +2/-3 |
| DC_11A_n77A |  |  | 23 | +2/-3 |
| DC_11A_n78A |  |  | 23 | +2/-3 |
| DC_11A_n79A |  |  | 23 | +2/-3 |
| DC_12A_n2A |  |  | 23 | +2/-3 |
| DC_12A_n5A |  |  | 23 | +2/-3 |
| DC_12A_n7A |  |  | 23 | +2/-3 |
| DC_12A_n25A |  |  | 23 | +2/-3 |
| DC_12A_n30A |  |  | 23 | +2/-3 |
| DC_12A_n38A |  |  | 23 | +2/-3 |
| DC_12A_n41A |  |  | 23 | +2/-3 |
| DC_12A_n66A |  |  | 23 | +2/-3 |
| DC_12A_n71A7 |  |  | 23 | +2/-3 |
| DC_12A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_12A_n78A |  |  | 23 | +2/-3 |
| DC_13A_n2A |  |  | 23 | +2/-3 |
| DC_13A_n5A |  |  | 23 | +2/-3 |
| DC_13A_n7A |  |  | 23 | +2/-3 |
| DC_13A_n25A |  |  | 23 | +2/-3 |
| DC_13A_n48A |  |  | 23 | +2/-3 |
| DC_13A_n66A |  |  | 23 | +2/-3 |
| DC_13A_n71A |  |  | 23 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_13A_n78A |  |  | 23 | +2/-3 |
| DC_14A_n2A |  |  | 23 | +2/-3 |
| DC_14A_n5A |  |  | 23 | +2/-3 |
| DC_14A_n30A |  |  | 23 | +2/-3 |
| DC_14A_n66A |  |  | 23 | +2/-3 |
| DC_14A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_18A_n3A |  |  | 23 | +2/-3 |
| DC_18A_n28A |  |  | 23 | +2/-3 |
| DC_18A_n41A |  |  | 23 | +2/-3 |
| DC_18A_n77A |  |  | 23 | +2/-3 |
| DC_18A_n78A |  |  | 23 | +2/-3 |
| DC_18A_n79A |  |  | 23 | +2/-3 |
| DC_19A_n1A |  |  | 23 | +2/-3 |
| DC_19A_n77A |  |  | 23 | +2/-3 |
| DC_19A_n78A |  |  | 23 | +2/-3 |
| DC_19A_n79A |  |  | 23 | +2/-3 |
| DC_20A_n1A |  |  | 23 | +2/-3 |
| DC_20A_n3A |  |  | 23 | +2/-3 |
| DC_20A_n7A |  |  | 23 | +2/-3 |
| DC_20A_n8A |  |  | 23 | +2/-3 |
| DC_20A_n38A |  |  | 23 | +2/-3 |
| DC_20A_n28A |  |  | 23 | +2/-3 |
| DC_20A_n41A |  |  | 23 | +2/-3 |
| DC_20A_n50A |  |  | 23 | +2/-3 |
| DC_20A_n51A |  |  | 23 | +2/-3 |
| DC_20A_n77A |  |  | 23 | +2/-3 |
| DC_20A_n80A |  |  | 23 | +2/-3 |
| DC_20A_n78A |  |  | 23 | +2/-3 |
| DC_20A_n82A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_20A_n83A |  |  | 23 | +2/-3 |
| DC_21A_n1A |  |  | 23 | +2/-3 |
| DC_21A_n28A |  |  | 23 | +2/-3 |
| DC_21A_n77A |  |  | 23 | +2/-3 |
| DC_21A_n78A |  |  | 23 | +2/-3 |
| DC_21A_n79A |  |  | 23 | +2/-3 |
| DC_25A_n41A |  |  | 23 | +2/-3 |
| DC_25A_n77A |  |  | 23 | +2/-3 |
| DC_25A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n25A |  |  | 23 | +2/-3 |
| DC_26A_n41A |  |  | 23 | +2/-3 |
| DC_26A_n77A |  |  | 23 | +2/-3 |
| DC_26A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n1A |  |  | 23 | +2/-3 |
| DC_28A_n2A |  |  | 23 | +2/-3 |
| DC_28A_n3A |  |  | 23 | +2/-3 |
| DC_28A_n5A |  |  | 23 | +2/-3 |
| DC_28A_n7A DC_28A_n7B |  |  | 23 | +2/-3 |
| DC_28A_n8A |  |  | 23 | +2/-3 |
| DC_28A_n40A |  |  | 23 | +2/-3 |
| DC_28A_n41A |  |  | 23 | +2/-3 |
| DC_28A_n50A |  |  | 23 | +2/-3 |
| DC_28A_n51A |  |  | 23 | +2/-3 |
| DC_28A_n66A |  |  | 23 | +2/-3 |
| DC_28A_n77A |  |  | 23 | +2/-3 |
| DC_28A_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_28A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n41A |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_30A_n2A |  |  | 23 | +2/-3 |
| DC_30A_n5A |  |  | 23 | +2/-3 |
| DC_30A_n66A |  |  | 23 | +2/-3 |
| DC_30A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_38A_n1A |  |  | 23 | +2/-3 |
| DC_38A_n3A |  |  | 23 | +2/-3 |
| DC_38A_n8A |  |  | 23 | +2/-3 |
| DC_38A_n28A |  |  | 23 | +2/-3 |
| DC_38A_n78A |  |  | N/A | N/A |
| DC_38A_n79A |  |  | 23 | +2/-3 |
| DC_39A_n40A |  |  | 23 | +2/-3 |
| DC_39A_n41A DC_39C_n41A | 265 | +2/-3 | 23 | +2/-3 |
| DC_39A_n78A |  |  | 23 | +2/-3 |
| DC_39A_n79A | 265 | +2/-3 | 23 | +2/-3 |
| DC_40A_n1A |  |  | 23 | +2/-3 |
| DC_40A_n41A DC_40C_n41A |  |  | 23 | +2/-3 |
| DC_40A_n77A |  |  | N/A | N/A |
| DC_40A_n78A |  |  | 23 | +2/-3 |
| DC_40C_n78A |  |  | 23 | +2/-3 |
| DC_40A_n79A |  |  | 23 | +2/-3 |
| DC_41A_n1A DC_41C_n1A |  |  | 23 | +2/-3 |
| DC_41A_n3A DC_41C_n3A |  |  | 23 | +2/-3 |
| DC_41A_n28A DC_41C_n28A |  |  | 23 | +2/-3 |
| DC_41A_n77A DC_41C_n77A |  |  | 23 | +2/-3 |
| DC_41A_n78A DC_41C_n78A |  |  | 23 | +2/-3 |
| DC_41A_n79A DC_41C_n79A | 265 | +2/-3 | 23 | +2/-3 |
| DC_42A_n1A DC_42C_n1A |  |  | 23 | +2/-3 |
| DC_42A_n3A DC_42C_n3A |  |  | 23 | +2/-3 |
| DC_42A_n28A DC_42C_n28A |  |  | 23 | +2/-3 |
| DC_42A_n51A |  |  | 23 | +2/-3 |
| DC_42A_n77A |  |  | N/A | N/A |
| DC_42A_n78A |  |  | N/A | N/A |
| DC_42A_n79A |  |  | N/A | N/A |
| DC_48A_n2A |  |  | 23 | +2/-3 |
| DC_48A_n5A |  |  | 23 | +2/-3 |
| DC_48A_n12A |  |  | 23 | +2/-3 |
| DC_48A_n25A |  |  | 23 | +2/-3 |
| DC_2A_n46A |  |  | 23 | +2/-3 |
| DC_48A_n66A |  |  | 23 | +2/-3 |
| DC_48A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n2A |  |  | 23 | +2/-3 |
| DC_66A_n5A |  |  | 23 | +2/-3 |
| DC_66A_n7A |  |  | 23 | +2/-3 |
| DC_66A_n12A |  |  | 23 | +2/-3 |
| DC_66A_n25A |  |  | 23 | +2/-3 |
| DC_66A_n28A |  |  | 23 | +2/-3 |
| DC_66A_n30A |  |  | 23 | +2/-3 |
| DC_66A_n38A |  |  | 23 | +2/-3 |
| DC_66A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| DC_66A_n46A |  |  | 23 | +2/-3 |
| DC_66A_n48A |  |  | 23 | +2/-3 |
| DC_66A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_66A_n78A DC_66A-66A_n78A |  |  | 23 | +2/-3 |
| DC_66A_n86A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_71A_n2A |  |  | 23 | +2/-3 |
| DC_71A_n5A |  |  | 23 | +2/-3 |
| DC_71A_n38A |  |  | 23 | +2/-3 |
| DC_71A_n41A |  |  | 23 | +2/-3 |
| DC_71A_n48A |  |  | 23 | +2/-3 |
| DC_71A_n66A |  |  | 23 | +2/-3 |
| DC_71A_n78A |  |  | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 7: Only single switched UL is supported. NOTE 8: The UE that supports a PC2 uplink EN-DC configuration with single carrier for each individual band and a composite of supporting PC3 within a TDD or FDD band PC2 within a second TDD band may signal a higherPowerLimitMRDC-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame):

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

If a UE supports a different power class than the default UE power class for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal tomaxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4;

else

–	shall apply all requirements for the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

### << Unchanged sections omitted >>

##### 6.2B.4.1.3	Inter-band EN-DC within FR1

For inter-band dual connectivity with one uplink serving cell or more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG and one uplink serving cell on the NR CG or more than one uplink serving cells configured for intra-band UL CA, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission power for EN-DC operation, PTotalEN-DC= 10log10(PtotalEN-DC) with PtotalEN-DC as specified in clause 7.6 of TS 38.213 [10]. For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, the PCMAX applies to the entire E-UTRA CG. For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the NR CG, the PCMAX applies to the entire NR CG.

For a UE configured with EN-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell and E-UTRA Pcell is FDD, the UE is not expected to be configured with more than one serving cells in the uplink.

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier(s) shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, EN-DC , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + tC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), PLTE, PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA}

For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for the E-UTRA CG as specified in TS 36.101 [4] clause 6.2.5A modified by PLTE as follows:

PCMAX_L_ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c  - TC , (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPR + A-MPR + ΔTIB,c + TC + TProSe, P-MPR ), PLTE, PPowerClass,EN-DC }

PCMAX H _ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c , PPowerClass,E-UTRA, PLTE, PPowerClass,EN-DC}

The configured maximum output power PCMAX,f,c,NR (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified as follows:

PCMAX_L,f,c,NR = MIN { PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MAX(MPRc, A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), PNR , PPowerClass,NR – ΔPPowerClass,NR }

For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the NR CG, PCMAX_L,f,c, NR and PCMAX_H,f,c, NR are the limits for the NR CG as specified in [2] subclause 6.2A.4 modified by PNR as follows:

PCMAX_L,f,c,NR  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA, PPowerClass,NR – MAX(MPR + A-MPR + ΔTIB,c + T_NR ,C + TRxSRS, P-MPR ), PNR, PPowerClass,EN-DC }

PCMAX_H,f,c,NR  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,NR, PNR, PPowerClass,EN-DC}

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PPowerClass refers to the maximum output power of the E-UTRA intra-band CA power class given in Table 6.2.2A-1 of TS 36.101 [4],

-	If more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG, PPowerClass refers to the maximum output power of the NR intra-band CA power class given in sub clause 6.2A.1 of [2],

-	PLTE is the value given by the field p-maxEUTRA-r15 of the RRCConnectionReconfiguration-v1510 IE as defined in TS 36.331 [8];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in clause 6.2.3A and clause 6.2.4A of TS 36.101 [4] respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX_ E-UTRA,c is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers within the E-UTRA CG.

-	If more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG, MPRc and A-MPRc are determined by subclause 6.2.2 of [2]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in TS 38.331 [9];

-	Δtc_E-UTRA, c = 1.5 dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0 dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0 dB;TC_NR,C is the highest value TC_NR,C among all serving cells c if more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.3 for inter-band EN-DC; if the UE indicates higherPowerLimitMRDC-r17 and ΔPPowerClass,EN-DC = 0, PPowerClass,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB;

-	∆PPowerClass,EN-DC = 3 dB for a power class 2 capable EN-DC UE when requirements of default power class had been applied as specified in sub-clause 6.2B.1; otherwise ∆PPowerClass,EN-DC = 0 dB;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the EN-DC combination as defined in clause 6.2.1 of 38.101-1 [2]; in case IE powerClassNRPart-r16 as defined in TS 38.331 [9] is indicated, PPowerClass,NR should use that value instead;

-	ΔPPowerClass,NR is 3 dB or 0 dB according to clause 6.2.4 of TS 38.101-1 [2] for a UE that supports power class 2 in the NR band of the EN-DC combination as defined in clause 6.2.1 of TS 38.101-1 [2];

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of 36.101 [4];

-	ΔPPowerClass,E-UTRA is 3 dB or 0 dB according to clause 6.2.5 of TS 36.101 [4] for a UE that supports power class 2 in the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of TS 36.101 [4];

-	ΔTIB,c specified in clause 6.2B.4.2.3 for EN-DC, the individual Power Class defined in table 6.2B.1.3 and any other additional power reductions parameters specified in clauses 6.2B.2 and 6.2B.3for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

-	∆TRxSRS is the highest value among all serving cells c.

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

PTotalEN-DC = 10log10(PtotalEN-DC) with PTotalEN-DC the configured maximum transmission power for EN-DC operation as specified in clause 7.6 of TS 38.213 [10].

The total configured maximum transmission power for both synchronous and non-synchronous operation is

PTotalEN-DC= MIN { PEMAX, EN-DC ,PPowerClass, EN-DC – ΔPPowerClass, EN-DC }

If the UE does not support dynamic power sharing,

PTotalEN-DC= MIN { PEMAX, EN-DC ,PPowerClass, EN-DC  – ΔPPowerClass, EN-DC } + 0.3 dB

If the EN-DC UE does not support dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalEN-DC applies.

When a UE supporting dynamic sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX,f,c,NR for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for EN-DC operation, PTotalEN-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

When an UL subframe transmission p from E-UTRA overlap with a physical-channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3-1 when same or different subframe and physical-channel durations are used in aggregated carriers. The lesser of PPowerClass ,EN-DC and PEMAX,EN-DC shall not be exceeded by the UE during any evaluation period of time where PPowerClass ,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB if the UE indicates higherPowerLimitMRDC-r17.

Table 6.2B.4.1.3-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | E-UTRA Subframe on all aggregated cells of E-UTRA | Min(Tno_hopping, Physical Channel Length) on all aggregated cells of NR |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ EN-DC _H (p,q) , PCMAX_ EN-DC _H (p,q+1), … , PCMAX_ EN-DC _H (p,q+n) }

where PCMAX_ EN-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with E-UTRA subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ EN-DC _L (p,q) , PCMAX_ EN-DC _L (p,q+1), … , PCMAX_ EN-DC _L (p,q+n)}

where PCMAX_EN-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with E-UTRA subframe p,

With

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

And:

a= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PTotalEN-DC

b= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > PTotalEN-DC

If a= FALSE

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

ELSE If (a=TRUE) AND (b=FALSE)

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q) /X_scale ], PEMAX, EN-DC ,PPowerClass, EN-DC}

ELSE If b= TRUE

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, EN-DC ,PPowerClass, EN-DC}

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.3-1 for inter-band EN-DC; if the UE indicates higherPowerLimitMRDC-r17, PPowerClass,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB;

-	X_scale is the linear value of X dB which is configured by RRC and can only take values [0 , 6]

-	pCMAX_ E-UTRA,c (p) is the linear value of PCMAX_ E-UTRA,c (p), the configured max power for E-UTRA. If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PCMAX_ E-UTRA,c (p) will be replaced by PCMAX(p) which is the configured maximum power for the entire E-UTRA CG.

-	pCMAX,f,c,NR (q) is the linear value of PCMAX,f,c,NR (q), the configured max power of NR, If more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG,  PCMAX_ NR,c (q) will be replaced by PCMAX(q) which is the configured maximum power for the entire NR CG.

Table 6.2B.4.1.3-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

When E-UTRA and NR transmissions overlap and the condition (If (a=TRUE) AND (b=FALSE)) is met, SCG shall be transmitted and the following supplementary minimum requirement apply for the measured SCG power, PUMAX,f,c,NR (q), under nominal conditions.

10log(pCMAX L,f,c,NR (q)/X_scale)  –  TLOW (10log(pCMAX L,f,c,NR (q)/X_scale) )}  ≤  PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,NR (q)) + THIGH (10log(pCMAX H, f,c,NR (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.