# 3GPP TSG-RAN WG4 Meeting #110                                                             R4-240032

# Athens, GR, 26th February - 1st March 2024

# Title:	TP for TR37.718-21-11 ENDC for FR1

# Source:	Softbank, LG Electronics

# Agenda Item:	7.4.2

# Document for:	Approval

- Introduction

This contribution is a text proposal for TR 38.718-21-11[5] combinations as requested in RAN4#101.

- EN-DC 3-11_n79 (2 bands LTE inter-band Carrier aggregation (CA) (2 Downlink (DL) / 1 Uplink (UL)) and 1 NR band (1 DL / 1 UL))

- EN-DC 8-42_n79 (2 bands LTE inter-band Carrier aggregation (CA) (2 Downlink (DL) / 1 Uplink (UL)) and 1 NR band (1 DL / 1 UL))

- Reference

[1] 3GPP TR 21.905 Vocabulary for 3GPP Specifications (Release 17) V17.1.0

[4] 3GPP TR37.863-01-01 E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of LTE 1 Down Link (DL) / 1 Up Link (UL) and 1 NR band (Release 15) V15.4.0

[5] 3GPP TS37.718-21-11 Dual Connectivity of 2 bands LTE inter-band CA and 1 NR band (Release 18) V0.9.0

- Text Proposal

- -- Start of TP –

-- Unaffected parts omitted –

## 5.x	DC_8-42_n79

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A-42A_n79A | DC_8A_n79A |

### 5.x.2	Co-existence studies

Based on co-existence studies of DC_8_n79 and DC_42_n79 captured in TR37.863-01-01[4], own Rx impact of the 3bands is as follows.

- 2nd order IMD generated by dual uplink of band 8 and band n79 may fall into own Rx of band 42.

- 3rd order IMD generated by dual uplink of band 8 and band n79 may also fall into own Rx of band n79. However, TDD band does not transmit and receive simultaneously in a single band, thus the own band of n79 would not be interfered.

- 3rd order IMD generated by dual uplink of Band 42 and Band n79 may also fall into own Rx of Band 42. However, UL of Band 42 is not used in this configuration. So, there is no own Rx impact of 3rd band for this configuration.

### 5.x.3	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_8-42_n79 can be applied.

Table 5.xx.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8-42_n79 | 0.3 | 0.8 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.xx.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8-42_n79 | - | 0.5 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.x.4	REFSENS requirements

Based on above co-existence study result, 2nd order IMD generated by UL_8_n79 fall into band 42.

Table 5.x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-42A_n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n79 | 4470 | 40 | 216 | 4470 | N/A | N/A |
|  | 42 | N/A | 5 | N/A | 3443 | 33.1 | IMD2 |

## 5.x	DC_3-11_n79

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-11A_n79A | DC_3A_n79A |

### 5.x.2	Co-existence studies

Based on co-existence studies of DC_3_n79 and DC_11_n79 captured in TR37.863-01-01[4], own Rx impact of the 3bands is as follows.

- 3rd order IMD generated by dual uplink of Band 3 and Band n79 may also fall into own Rx of Band 3 and Rx Band of 11.

### 5.x.3	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_3-11_n79 can be applied.

Table 5.xx.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-11_n79 | 0.3 | 0.8 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.xx.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-11_n79 | - | 0.5 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.x.4	REFSENS requirements

3rd order IMD generated by dual uplink of Band 3 and Band n79 may fall into own Rx of Band 3 and Rx Band of 11.

Table 5.x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-11A_n79A | 3 | 1767 | 5 | 25 | 1862 | N/A | IMD5 |
|  | 11 | 1443 | 5 | 25 | 1491 | 16.1 | IMD3 |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |

-- Unaffected parts omitted –

-- End of TP –