| 3GPP TR 38.896 V1.2.0 (2024-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High power UE (power class 2) for NR FR1 FDD single band; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	8

4.1	TR Maintenance	8

5	High Power UE for FDD single band	8

5.1	NR band n8	8

5.1.1	UE maximum output power	8

5.1.2	A-MPR requirements	8

5.1.2.1 NS_43	8

5.1.2.1.1 A-MPR simulation results from Huawei	8

5.1.2.1.2 A-MPR simulation results from Apple	14

5.1.2.1.3 A-MPR requirements	15

5.1.2.2 NS_43U	16

5.1.3	Reference sensitivity requirements	17

5.2	NR band n28	17

5.2.1	UE maximum output power	17

5.2.2	A-MPR requirements	18

5.2.2.1 NS_18	18

5.2.2.1.1 A-MPR simulation results from Huawei	18

5.2.2.1.2 A-MPR simulation results from Apple	26

5.2.2.1.3 A-MPR requirements	28

5.2.2.2 NS_17	29

5.3	NR band n25	30

5.3.1	UE maximum output power	30

5.3.2	A-MPR requirements	30

5.3.3	Reference sensitivity requirements	31

5.4	NR band n66	31

5.4.1	UE maximum output power	31

5.4.2	A-MPR requirements	31

5.4.3	Reference sensitivity requirements	32

5.5	NR band n71	32

5.5.1	UE maximum output power	32

5.5.2	A-MPR and ∆MPR requirements	32

5.5.3	Reference sensitivity requirements	33

5.6	NR band n85	34

5.6.1	UE maximum output power	34

5.6.2	A-MPR requirements	34

5.6.3	Reference sensitivity requirements	35

5.7	NR band n70	35

5.7.1	UE maximum output power	35

5.7.2	A-MPR requirements	35

5.7.3	Reference sensitivity requirements	36

5.8	NR band n2	36

5.8.1	UE maximum output power	36

5.8.2	A-MPR requirements	36

5.8.3	Reference sensitivity requirements	36

5.9	NR band n14	37

5.9.1	UE maximum output power	37

5.9.2	A-MPR requirements	37

5.9.2.1	NS_06	37

5.9.2.1.1	A-MPR simulation results from Huawei	37

5.9.2.1.2	A-MPR simulation results from Apple	39

5.9.2.1.3	A-MPR requirements	42

5.9.3	Reference sensitivity requirements	42

Annex <A> (informative): Change history	43

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for High power UE (power class 2) for NR FR1 FDD single band under Rel-18 time-frame. The purpose is to gather the relevant background information and studies in order to complete the band specific requirements for the newly requested band combinations for power class 2 UE under FDD mode.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

DL				Downlink

FDD				Frequency Division Duplex

MPR 				Allowed maximum power reduction

MSD				Maximum Sensitivity Degradation

REFSENS				Reference Sensitivity power level

UE				User Equipment

UL				Uplink

4	Background

At 3GPP RAN4#97-e meeting, a basket Work Item on “High power UE (power class 2) for NR FR1 FDD single band”  was approved for Rel-18. The objectives of the core part are as follows:

The objectives of the core part are as follows:

- Scope of this basket WI includes NR FDD bands

- Specify the band specific RF requirements for all listed power class 2 NR FDD bands including

  - UE maximum output power and Tx power tolerance

  - A-MPR requirements if needed

  - PC2 sensitivity degradation requirements if needed

Note 1: Ensure that the UE RF requirements of power class 2 UEs shall comply with those of power class 3 when the maximum transmit power is limited to 23dBm by gNB configuration.

Note 2: The ability to define 1Tx and 2Tx requirements is subject to data availability.

Note 3: Both 1Tx and 2Tx UE architecture are considered in this WI.

The present document is a technical report for this basket Work Item.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

5	High Power UE for FDD single band

5.1	NR band n8

5.1.1	UE maximum output power

Table 5.1.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n8 | 26 | +2/-33 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

5.1.2	A-MPR requirements

5.1.2.1 NS_43

5.1.2.1.1 A-MPR simulation results from Huawei

The simulation results are obtained for NS_43 for both PC2 1Tx and PC3 1Tx, which are shown in Table 5.1.2.1.1-1 below. The absolute PC2 A-MPR values are plotted in the 3rd column, while the differences between PC2 and PC3 are plotted in the 2nd column, for which 0dB is assigned if A-MPR ≤ PC2 MPR.

Table 5.1.2.1.1-1: A-MPR simulation results for NS_43 with 1Tx, SCS=15kHz

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 15MHz |  |  |
| 10MHz |  |  |
| 5MHz |  |  |

Additional simulation results are obtained for NS_43 PC2 1Tx with BW=10MHz and SCS=30kHz, which are shown in Table 5.1.2.1.1-2 below. The figures in the 2nd column show the differences of the A-MPR simulation results between PC2 and PC3, and the figures in the 3rd column plot only the PC2 A-MPR, for which 0dB is assigned if A-MPR ≤ PC2 MPR.

Table 5.1.2.1.1-2: A-MPR simulation results for NS_43 with 1Tx, SCS=30kHz

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 10MHz |  |  |

It can be seen from the results in Table 1 that some A-MPR is needed for PC2 NS_18 for BW=10MHz with SCS=30kHz, although the PC3 A-MPR in the current specification is only allowed for SCS=15kHz.

5.1.2.1.2 A-MPR simulation results from Apple

Simulations have been conducted for all channel bandwidth from 5MHz, 10MHz and 15MHz. The A-MPR regions proposed in [3] were used as starting point. Selected simulation results can be found in Table 5.1.2.1.2-1 where allocations for DFT-s-OFDM and CP-OFDM for QPSK are provided.

Table 5.1.2.1.2-1: A-MPR simulation results for NS_43

5.1.2.1.3 A-MPR requirements

Based on the simulation results from previous sections, the PC2 A-MPR requirements for NS_43 are proposed below.

The existing A-MPR regions for PC3 as defined in TS38.101-1 are adapted for PC2, which are shown as the table below. The new table is applicable for both power classes without impact to the PC3 requirements.

Table 6.2.3.6-1: A-MPR regions for NS_43

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 15 | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | ≤ 1.8 MHz /12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  | ≥ 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/SCS | A7 |
|  |  |  |  |  | ≥ 6.12 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2. NOTE 2: Only 15 kHz SCS is applicable for power class 3; both 15kHz and 30kHz SCS are applicable for power class 2. NOTE 3: Void |  |  |  |  |  |  |  |

The A-MPR values for PC2 1Tx are proposed in the tables below.

Table 6.2.3.6-2a: A-MPR for NS_43 (Power Class 2, 1Tx)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | [1] | N/A | ≤ [1.5] | N/A |  |  |  | N/A |  | N/A |
|  | QPSK | ≤ [3.5] |  |  |  |  |  | ≤ [3.5] |  | ≤ [5.5] |  |
|  | 16 QAM | ≤ [3.5] |  |  |  |  |  | ≤ [3.5] |  | ≤ [5.5] |  |
|  | 64 QAM | ≤ [3.5] |  | ≤ [4.5] |  | ≤ [5] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | ≤ [4] |  |  |  |  |  | ≤ [4] |  | ≤ [6] |  |
|  | 16 QAM | ≤ [4] |  |  |  |  |  | ≤ [4.5] |  | ≤ [6] |  |
|  | 64 QAM | ≤ [4] |  | ≤ [5] |  | ≤ [6.5] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.6-2b: A-MPR for NS_43 (Power Class 2, 1Tx) Cont.

| Modulation/Waveform |  | A6 | A7 |  |
| --- | --- | --- | --- | --- |
|  |  | Outer / Inner (dB) | Outer (dB) | Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [12] | ≤ [4] | [3] |
|  | QPSK | ≤ [12] | ≤ [4] | [3] |
|  | 16 QAM | ≤ [12] | ≤ [5] | [4] |
|  | 64 QAM | ≤ [12] | ≤ [5.5] | [5.5] |
|  | 256 QAM | ≤ [12] | ≤ [6.5] | [5.5] |
| CP-OFDM | QPSK | ≤ [12] | ≤ [6] | [4.5] |
|  | 16 QAM | ≤ [12] | ≤ [6] | [5] |
|  | 64 QAM | ≤ [12] | ≤ [6.5] | [6] |
|  | 256 QAM | ≤ [12] | ≤ [7] | [7] |

5.1.2.2 NS_43U

FFS.

5.1.3	Reference sensitivity requirements

For PC2 single band n8, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n8 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in Table:

Table 5.1.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | Skyworks(R4-2300652, R4-2307239) | 0.3 | 0.4 | 0.7 | 1.9 | 2.1 |  | 2.6 |  |  |  |
|  | Apple(R4-2300362) | 0 | 1.2 | 0.9 | 1.9 | N/A | N/A | 4.4 | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715, R4-2307916) | 1.2 | 1.2 | 1.4 | 3.3 | 3.4 | - | 3.8 | - | - | - |
|  | Murata(R4-2305393, R4-2308436) | 0.4 | 0.3 | 0.3 | 2.4 | 2.8 | - | 2.9 | - | - | - |
|  | Qualcomm(R4-2302709) | 0.8 | 0.6 | 0.7 | 1.8 | - | - | 1.8 |  |  |  |
| Average |  | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | - | 3.1 |  |  |  |

For PC2 single band n8 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.1.3-2:

Table 5.1.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | Skyworks(R4-2300652, R4-2307239) | 0.8 | 1.0 | 1.5 | 5.1 | 5.6 |  | 6.6 |  |  |  |
|  | Apple(R4-2305364) | 0.5 | 0.5 | 0.9 | 6.2 | N/A | N/A | 8.1 | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715, R4-2307916) | 3.1 | 3.2 | 3.8 | 6.2 | 6.5 | - | 7.1 | - | - | - |
|  | Murata(R4-2305393, R4-2308436) | 1.8 | 1.8 | 2.2 | 5.6 | 6.1 | - | 6.8 | - | - | - |
|  | Qualcomm(R4-2302709) | 1.5 | 1.5 | 2.2 | 6.5 | - | - | 5.9 |  |  |  |
|  | MTK (R4-2219869) | 0 | 0 | 1 | 5.4 | - | - | 7.2 | - | - | - |
|  | ZTE, CU (R4-2215893) | 0.5 | 0.6 | 1.2 | 5.0 | - | - | 6.7 | - | - | - |
|  | Spreadtrum(R4-2304084) | 2.2 | 2.2 | 3.8 | 6.5 |  |  | 7.3 |  |  |  |
| Average |  | 1.3 | 1.4 | 2.1 | 5.8 | 6.1 | - | 7.0 |  |  |  |

5.2	NR band n28

5.2.1	UE maximum output power

Table 5.2.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n28 | 26 | [FFS] |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

5.2.2	A-MPR requirements

5.2.2.1 NS_18

5.2.2.1.1 A-MPR simulation results from Huawei

The simulation results are obtained for NS_18 for both PC2 1Tx and PC3 1Tx, which are shown in Table 5.2.2.1.1-1 below. The figures in the 2nd column show the differences in the A-MPR simulation results between PC2 and PC3, and the figures in the 3rd column plot only the PC2 A-MPR, for which 0dB is assigned if A-MPR ≤ PC2 MPR. It can be seen that similar A-MPR value distributions are shown for BW=15/20/25/30MHz.

Table 5.2.2.1.1-1: A-MPR simulation results for NS_18 with 1Tx, SCS=15kHz

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 5MHz |  |  |
| 10MHz |  |  |
| 15MHz |  |  |
| 20MHz |  |  |
| 25MHz |  |  |
| 30MHz |  |  |

5.2.2.1.2 A-MPR simulation results from Apple

Simulations have been conducted for all channel bandwidth from 5MHz, 10MHz, 15MHz, 20MHz, 25MHz and 30MHz. The A-MPR regions proposed in [3] were used as starting point. Selected simulation results can be found in Table 5.2.2.1.2-1 where allocations for DFT-s-OFDM and CP-OFDM for QPSK are provided.

Table 5.2.2.1.2-1: A-MPR simulation results for NS_43

5.2.2.1.3 A-MPR requirements

Based on the simulation results from previous sections, the PC2 A-MPR requirements for NS_18 are proposed below.

The existing A-MPR regions for PC3 as defined in TS 38.101-1 are adapted for PC2, which are shown as in the tables below. The new tables are applicable for both power classes without impact to the PC3 requirements.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 for PC3; Table 6.2.3.13-2, A1 for PC2 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 for PC3; Table 6.2.3.13-2, A2 for PC2 |
|  |  |  | 25, 30 |  | Table 6.2.3.13-1, A3, A4, A5 for PC3; Table 6.2.3.13-2, A3, A4, A5, A6 for PC2 |

Table 6.2.3.13-0: Band n28 and n83 30MHz A-MPR regions for NS_18

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

The A-MPR values for 1Tx PC2 are proposed in the table below.

Table 6.2.3.13-2: A-MPR for NS_18 (Power Class 2, 1Tx)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [3.5] | N/A | ≤ [8] | [3.5] | [10] | [4.5] | [2] |
|  | QPSK | ≤ [3.5] |  | ≤ [8] | [3.5] | [10] | [4.5] | [2] |
|  | 16 QAM | ≤ [4] |  | ≤ [9] | [3.5] | [11] | [5] | [2] |
|  | 64 QAM | ≤ [4.5] |  | ≤ [10] | [3.5] | [11] | [5.5] | [2.5] |
|  | 256 QAM | ≤ [6.5] |  | ≤ [11] | [3.5] | [11] | [5.5] |  |
| CP-OFDM | QPSK | ≤ [5.5] |  | ≤ [9.5] | [5] | [11.5] | [6] | [4] |
|  | 16 QAM | ≤ [5.5] |  | ≤ [10] | [5] | [11.5] | [6] | [4] |
|  | 64 QAM | ≤ [6] |  | ≤ [11.5] | [5] | [11.5] | [6] | [4] |
|  | 256 QAM | ≤ [9] |  | ≤ [11.5] | [5] | [11.5] | [7.5] |  |

Note that BW=25MHz is missing for NS_18 in Table 6.5.3.3-1 of TS38.101-1 and needs to be added.

Table 6.5.3.3.3-1: Additional requirements for "NS_18"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 25, 30 |  |
| 692-698 | -26.2 | 6 MHz |

5.2.2.2 NS_17

FFS

5.3	NR band n25

5.3.1	UE maximum output power

Table 5.3.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n25 | 26 | +2/-33 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

5.3.2	A-MPR requirements

Re-use PC3 A-MPR.

5.3.3	Reference sensitivity requirements

For PC2 single band n25, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n25 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in Table:

Table 5.3.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 | Apple(R4-2300362) | 0 | 0.6 | 1.0 | 1.4 | 2.1 | 2.4 | 3.0 | 4.7 | 5.1 | N/A |
|  | Skyworks(R4-2300652) | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 2.1 | 2.1 | 2.3 | 2.5 |  |
|  | Huawei, HiSilicon(R4- 2300715) | 1.4 | 1.4 | 1.4 | 1.9 | 1.9 | 3.5 | 3.5 | 3.7 | 3.8 | - |
|  | Murata(R4-2305393) | 1.0 | 0.7 | 0.6 | 0.6 | 0.5 | 2.8 | 2.6 | 3.2 | 3.5 |  |
| Average |  | 0.8 | 0.8 | 0.9 | 1.1 | 1.3 | 2.7 | 2.8 | 3.5 | 3.7 |  |

For PC2 single band n8 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.3.3-2:

Table 5.3.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 | Apple(R4-2305364) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 6.9 | 6.8 | 7.5 | 7.9 | N/A |
|  | Skyworks(R4-2300652) | 1.0 | 1.3 | 1.4 | 1.5 | 1.7 | 5.6 | 5.5 | 6.1 | 6.4 |  |
|  | Huawei, HiSilicon(R4- 2300715) | 3.6 | 3.6 | 3.6 | 3.6 | 3.6 | 6.7 | 6.7 | 6.9 | 7.0 | - |
|  | Murata(R4-2305393) | 0.7 | 0.6 | 0.7 | 0.9 | 0.8 | 4.8 | 5.9 | 6.1 | 6.9 |  |
| Average |  | 1.5 | 1.5 | 1.6 | 1.6 | 1.7 | 6.0 | 6.2 | 6.7 | 7.1 |  |

5.4	NR band n66

5.4.1	UE maximum output power

Table 5.4.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n66 | 26 | +2/-3 |

5.4.2	A-MPR requirements

Re-use PC3 A-MPR.

5.4.3	Reference sensitivity requirements

For PC2 single band n66, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n66 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in Table:

Table 5.4.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n66 | Skyworks(R4-2300652) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
|  | Huawei, HiSilicon(R4- 2304517) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Average |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |

For PC2 single band n8 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.4.3-2:

Table 5.4.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n66 | Skyworks(R4-2300652) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
|  | Huawei, HiSilicon(R4- 2304517) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Average |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |

5.5	NR band n71

5.5.1	UE maximum output power

Table 5.5.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n71 | 26 | +2/-3 |

5.5.2	A-MPR and ∆MPR requirements

For PC2 A-MPR re-use the A-MPR region from PC1 and set the allowed power back-off to 3dB DFT-s-OFDM and 4.5dB for and CP-OFDM.

For ∆MPR, PC2 will use the same ∆MPR as PC3 in Table 6.2.2-3.

5.5.3	Reference sensitivity requirements

For PC2 single band n71, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n71 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in the following tables:

Table 5.5.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity with UL ≤ 20 MHz

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | Skyworks(R4-2300652) | 0.1 | 0.1 | 0.9 | 2.0 | 2.2 | 2.3 | 2.4 |  |  |  |
|  | Apple(R4-2300362) | 0 | 1.6 | 0.6 | 1.6 | 1.8 | 2.0 | 2.8 | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715) | 1.2 | 1.2 | 1.4 | 3.2 | 3.4 | 3.5 | 3.6 | - | - | - |
|  | Murata(R4-2305393) | 0.7 | 0.8 | 0.6 | 2.1 | 2.1 | 2.0 | 2.8 | - | - | - |
| Average |  | 0.5 | 0.9 | 0.9 | 2.2 | 2.4 | 2.5 | 2.9 |  |  |  |

Table 5.5.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity with symmetric UL/DL > 20 MHz

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | Skyworks(R4- R4-2311119) |  |  |  |  | 3.7 | 3.8 | 3.8 |  |  |  |
|  | Apple(R4-2311251) |  |  |  |  | 1.7 | 1.5 | 1.5 |  |  |  |
|  | Murata(R4-2308456 ,R4-2311441) |  |  |  |  | 2.2 | 2.0 | 3.9 |  |  |  |
| Average |  |  |  |  |  | 2.5 | 2.4 | 3.1 |  |  |  |

For PC2 single band n71 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.5.3-2:

Table 5.5.3-3: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity with UL ≤ 20 MHz

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | Skyworks(R4-2300652) | 0.3 | 0.4 | 1.5 | 4.7 | 5.1 | 5.3 | 5.6 |  |  |  |
|  | Apple(R4-2305364) | 0.5 | 0.5 | 1.0 | 6.2 | 6.7 | 7.1 | 7.5 |  |  |  |
|  | Huawei, HiSilicon(R4- 2300715) | 2.9 | 2.9 | 3.6 | 6.2 | 6.4 | 6.6 | 6.8 |  |  |  |
|  | Murata(R4-2305393) | 1.6 | 1.8 | 2.0 | 5.6 | 6.0 | 6.5 | 6.8 |  |  |  |
|  | MediaTek(R4-2302353) | 0 | 0 | 0.5 | 4.8 | 5.5 | 5.6 | 5.6 |  |  |  |
| Average |  | 1.1 | 1.1 | 1.7 | 5.5 | 5.9 | 6.2 | 6.5 |  |  |  |

Table 5.5.3-4: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity with symmetric UL/DL > 20 MHz

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | Skyworks(R4- R4-2311119) |  |  |  |  | 5.8 | 6.2 | 6.2 |  |  |  |
|  | Apple(R4-2311251) |  |  |  |  | 7.4 | 7.7 | 7.8 |  |  |  |
|  | Murata(R4-2308456 ,R4-2311441) |  |  |  |  | 7.4 | 7.8 | 7.8 |  |  |  |
| Average |  |  |  |  |  | 6.9 | 7.2 | 7.3 |  |  |  |

5.6	NR band n85

5.6.1	UE maximum output power

Table 5.6.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n85 | 26 | +2/-3 |

5.6.2	A-MPR requirements

[NS_06 A-MPR TBD]

5.6.3	Reference sensitivity requirements

For PC2 single band n85, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n85 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in Table:

Table 5.6.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n85 | Skyworks(R4-2300652) | 0.6 | 0.6 | 2.4 |  |  |  |  |  |  |  |
|  | Apple(R4-2300362) | 0 | 1.1 | 2.8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715) | 1.2 | 1.2 | 2.7 | - | - | - | - | - | - | - |
| Average |  | 0.6 | 1.0 | 2.6 |  |  |  |  |  |  |  |

For PC2 single band n8 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.6.3-2:

Table 5.6.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n85 | Skyworks(R4-2300652) | 1.3 | 1.3 | 6.2 |  |  |  |  |  |  |  |
|  | Apple(R4-2305364) | 0.5 | 0.5 | 7.6 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715) | 3.1 | 3.2 | 6.9 | - | - | - | - | - | - | - |
|  | MediaTek(R4-2302353) | 0 | 0.5 | 4.8 |  |  |  |  |  |  |  |
| Average |  | 1.2 | 1.4 | 6.4 |  |  |  |  |  |  |  |

5.7	NR band n70

5.7.1	UE maximum output power

Table 5.1.1-1: UE output power for PC2

| NR Band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n70 | 26 | +2/-3 |

5.7.2	A-MPR requirements

On network signalling NS_03, for the NR waveforms DFT-s-OFDM and CP-OFDM there is no need for additional power back-off and the PC3 table could be re-used.

5.7.3	Reference sensitivity requirements

For PC2 single band n70, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n70 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in Table:

Table 5.7.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n70 | Skyworks(R4-2311152) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
|  | Apple(R4-2311251) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | N/A | N/A | N/A | N/A | N/A |
|  | Mediatek(R4-2312178) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
|  | Qualcomm (R4-2313553) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
| Average |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |

For PC2 single band n70 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.7.3-2:

Table 5.7.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n70 | Skyworks(R4-2311152) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
|  | Apple(R4-2311251) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | N/A | N/A | N/A | N/A | N/A |
|  | Mediatek(R4-2312178) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
|  | Qualcomm (R4-2313553) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |
| Average |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |  |  |

5.8	NR band n2

5.8.1	UE maximum output power

Table 5.8.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n2 | 26 | +2/-33 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

5.8.2	A-MPR requirements

Re-use PC3 A-MPR.

5.8.3	Reference sensitivity requirements

For PC2 single band n2, both 1Tx and 2Tx requirements are considered. Therefore, Reference Sensitivity Degradation requirements for not supporting Tx diversity (i.e., 1Tx) and for supporting Tx diversity (i.e., 2Tx) are needed to be defined.

For PC2 single band n2 not supporting Tx diversity (i.e., 1Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.8.3-1:

Table 5.8.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n2 | Apple(R4-2300362) | 0 | 0.8 | 1.6 | 1.8 | 2.5 | 2.8 | 3.5 | 5.1 |  |  |
|  | Skyworks(R4-2300652) | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 2.0 | 2.0 | 2.2 |  |  |
|  | Huawei, HiSilicon(R4- 2300715) | 0.9 | 0.9 | 1.0 | 1.3 | 1.3 | 3.2 | 3.2 | 3.4 |  |  |
|  | Murata(R4-2305393) | 1.3 | 0.9 | 0.8 | 0.7 | 0.6 | 2.8 | 2.6 | 3.2 |  |  |
| Average |  | 0.8 | 0.9 | 1.1 | 1.2 | 1.3 | 2.7 | 2.8 | 3.5 |  |  |

For PC2 single band n2 supporting Tx diversity (i.e., 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.8.3-2:

Table 5.8.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n2 | Apple(R4-2305364) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 6.7 | 6.6 | 7.3 |  |  |
|  | Skyworks(R4-2300652) | 1.4 | 1.7 | 1.8 | 1.3 | 1.5 | 5.4 | 5.4 | 5.9 |  |  |
|  | Huawei, HiSilicon(R4- 2300715) | 2.0 | 2.0 | 2.0 | 2.0 | 2.0 | 6.2 | 6.2 | 6.5 |  |  |
|  | Murata(R4-2305393) | 0.9 | 0.7 | 0.7 | 0.9 | 0.9 | 4.8 | 5.9 | 6.1 |  |  |
| Average |  | 1.2 | 1.2 | 1.3 | 1.2 | 1.2 | 5.8 | 6.0 | 6.5 |  |  |

5.9	NR band n14

5.9.1	UE maximum output power

Table 5.9.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n14 | 26 | +2/-3 |

5.9.2	A-MPR requirements

5.9.2.1	NS_06

The introduction of PC2 to NR band n14 requires A-MPR evaluation for NS_06. The A-MPR evaluation applies equally to NR Bands n12, n13, n14, and n85.

5.9.2.1.1	A-MPR simulation results from Huawei

The SEM of NS_06 is like that of NS_03 and only relaxed in delta_f_OOB=0.1~1MHz. No PC3 A-MPR is specified for NS_06.

The simulation results are obtained for NS_06 based on 1Tx PC2, which are shown in Table 5.9.2.1.1-1 below. The figures in the 2nd column show the differences in the A-MPR simulation results between PC2 and PC3, and the figures in the 3rd column plot only the PC2 A-MPR, for which 0dB is assigned if A-MPR ≤ PC2 MPR.

Table 5.9.2.1.1-1: A-MPR simulation results for NS_06

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 5MHz |  |  |
| 10MHz |  |  |
| 15MHz |  |  |

It can be seen from the simulation results that in the most demanding region, the needed back-off for PC2 is increased by around 0.5dB compared with PC3.

5.9.2.1.2	A-MPR simulation results from Apple

Simulations have been conducted for all channel bandwidth from 3MHz to 15MHz.

The following assumptions and requirements are used for the simulations:

-	Power Class 2

-	Fixed Bias

-	NR Waveform

-	Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

-	Carrier Leakage: 28dBc

-	Image: 28dBc

-	CIM3: 60dBc

-	CIM5: 70dBc

The NS_06 flag defines an alternative SEM mask for channel bandwidths up to 15MHz. While the NS_06 mask shows similarities to general SEM the additional requirements are tighter and therefore require more power back-off. The NS_06 Additional SEM is specified in clause 6.5.2.3.4 of 3GPP 38.101-1 [2].

In the following, the NS_06 requirements are used and channel bandwidth of 3MHz, 5MHz, 10MHz and 15MHz are simulated. The simulations results are provided for QPSK with DFT-s-OFDM and CP-OFDM in Table 5.9.2.1.2-1.

Table 5.9.2.1.2-1: Power back-off for QPSK and NS_06

| DFT-s-OFDM QPSK | CP-OFDM QPSK |
| --- | --- |

For CP-OFDM the results indicate that the MPR is sufficient. However, there is a tendency for some outer allocations bordering the channel edge to require slightly more power back-off. For DFT-s-OFDM there exist certain outer allocation which need 1.5dB which is 0.5dB increased to MPR. Including the results and observations from clause 5.9.2.1.1, it might be considered to define 0.5dB relaxation for outer allocations.

Therefore, it is proposed to define 0.5dB relaxation for outer allocations.

5.9.2.1.3	A-MPR requirements

Based on the simulation results from the previous section, the PC2 A-MPR requirements for NS_06 are proposed below in Table 5.9.2.1.3-1.

Table 5.9.2.1.3-1: A-MPR for NS_06 (Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.0 |
|  | QPSK | ≤ 1.5 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3.0 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 |
|  | 64 QAM | ≤ 4.0 |
|  | 256 QAM | ≤ 6.5 |

5.9.3	Reference sensitivity requirements

For PC2 single band n14, both 1Tx and 2Tx requirements are considered. Therefore, Reference Sensitivity Degradation requirements for not supporting Tx diversity (i.e., 1Tx) and for supporting Tx diversity (i.e., 2Tx) are needed to be defined.

For PC2 single band n14 not supporting Tx diversity (i.e., 1Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.9.3-1:

Table 5.9.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n14 | Skyworks (R4-2311152) | 0.4 | 0.8 |  |  |  |  |  |  |  |  |
|  | Apple (R4-2311251) | 0.3 | 0.6 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 0.9 | 1.0 |  |  |  |  |  |  |  |  |
|  | Murata (R4-2318253) | 0.7 | 0.8 |  |  |  |  |  |  |  |  |
| Average |  | 0.6 | 0.8 |  |  |  |  |  |  |  |  |

For PC2 single band n14 supporting Tx diversity (i.e., 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.9.3-2:

Table 5.9.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n14 | Skyworks (R4-2311152) | 0.8 | 1.3 |  |  |  |  |  |  |  |  |
|  | Apple (R4-2311251) | 0.8 | 0.8 |  |  |  |  |  |  |  |  |
|  | Huawei, HiSilicon(R4- 2316472) | 2.0 | 2.0 |  |  |  |  |  |  |  |  |
|  | Murata (R4-2318253) | 0.7 | 1.0 |  |  |  |  |  |  |  |  |
| Average |  | 1.1 | 1.3 |  |  |  |  |  |  |  |  |

Annex <A> (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-10 | RAN4 #104bis-e | R4-2215852 |  |  |  | TR skeleton | 0.0.1 |
| 2022-11 | RAN4 #105 | R4-2218618 |  |  |  | Included TPs/pCRs: R4-2220482 TP for TR 38.896 to include HPUE single band n8 | 0.1.0 |
| 2023-05 | RAN4 #107 | R4-2307311 |  |  |  | Included TPs/pCRs: R4-2310244 TP for TR 38.896 to include n8 PC2 RSD values R4-2310242 TP for TR38896 on the PC2 A-MPR requirements for band n8 R4-2310243 TP for TR38896 on the PC2 A-MPR requirements for band n28 | 0.2.0 |
| 2023-08 | RAN4 #108 | R4-2311522 |  |  |  | Included TPs/pCRs: R4-2314642 TP for 38.896 Addition of PC2 for n25 R4-2314642 TP for 38.896 Addition of PC2 for n66 R4-2314642 TP for 38.896 Addition of PC2 for n71 R4-2314642 TP for 38.896 Addition of PC2 for n85 R4-2314642 TP for 38.896 Addition of PC2 for n70 | 0.3.0 |
| 2023-09 | RAN#101 | RP-232224 |  |  |  | Provided for information to RAN | 1.0.0 |
| 2024-03 | RAN4#110 | R4-2400354 |  |  |  | Included TPs/pCRs: R4-2403625 TP for TR 38.896 to add PC2 for n14 R4-2400700 TP for TR 38.896 to add PC2 for n2 | 1.2.0 |