3GPP TSG-RAN WG4 Meeting # 110									          R4-2400368

Athens, Greece, 16th February – 1st March 2024

Title: 	CA_n26(2A) A-MPR CA_NC_NS_12-15

Agenda Item:	6.1

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Introduction

This document presents measurements to verify the agreed-upon [20]dB A-MPR for CA_n26(2A) for CA_NC_NS_12, 13, 14, and 15. We also introduce CR[1] to align the A-MPR requirements with the supported component carrier (CC) channel bandwidth (CBW) for BCS0 .

# Discussion

## 2.1 On CA_NC_NS_12,13,14,15 vs CA_n26(2A) BCS0

Table 1 reproduces the supported CC CBW for CA_n26(2A) BCS0. Only 5, 10 and 15MHz CBW are supported per CC for a maximum aggregated bandwidth (BW) of 30MHz.

Table 1: CA_n26(2A) BCS0 CBW per CC – extracted from Table 5.5A.1-1.

| NR CA Configuration | Uplink CA Configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | CA_n26(2A)7 | 5,10,15 | 5,10,15 |  |  | 30 | 0 |
| NOTE 7: Unless otherwise stated, only RF requirements for dual PA architecture are applicable for UL CA_n26(2A). The UE shall indicate the dualPA-Architecture for UL CA_n26(2A). |  |  |  |  |  |  |  |

The CA_NC_NS_12, 13, 14, and 15 requirements captured in clause 6.5A.3.3.2.2, 6.5A.3.3.2.3, 6.5A.3.3.2.4 and 6.5A.3.3.2.5 specify that for CA_n26(2A), the one UL NS_12, 13, 14, and 15 requirements apply. Table 2 lists these 1UL emission requirements and observe that 1UL requirements are specified for UL CBW. However, not all requirements are supported by CA_n26(2A) BCS0. For example, NS_12 is specified for 3MHz CBW, but this CBW is not supported for CA_n26(2A) BCS0. Several similar observations can be made for NS_13, 14, and 15.

Table 2: NS_12, 13, 14, and 15 requirements – extracted from sub-clause 6.5.3.3

Observation 1: One-UL NS_12, 13, 14, and 15 emission requirements apply to dual-UL CA_n26(2A) BCS0. However, several CBWs specified in NS_12, 13, 14, and 15 are not supported by CA_n26(2A) BCS0. Corrections are needed in the CA_NC_NS_12, 13, 14, and 15 requirements to reflect the supported CBW specified in CA_n26(2A) BCS0.

To account for observation 1, we present in CR [1] the corrections summarized in Proposal 1 below.

Proposal 1: We suggest RAN4 adopts the CR [1] following changes to the CA_NC_NS_12, 13, 14, and 15 requirements.

## 2.2 Background on CA_NC_NS_12,13,14,15 A-MPR

Table 2 defines the RAN4 meeting #109 requirements and the A-MPR agreement of [20]dB for uplink (UL) CA_n26(2A) Power Class 3 (PC3) for CA_NC_NS_12, 13, 14, and 15. As specified in Table 5.5A.1-1, Note 7, RAN4 assumes CA_n26(2A) is supported with a dualPA architecture.

Table 2: CA_NC_NS_12, 13, 14, and 15 requirements from Table 6.2A.3.1.2-1.

RAN4 does not preclude a UE from supporting this combination with a single PA. Thus, we present A-MPR measurements for 2PA and for 1PA front-end architecture. The intent is to verify if [20]dB is sufficient to meet the band n26 emission requirements.

## 2.1 Power Amplifier Calibration

- Local Oscillator (LO) leakage: -28 dBc, IQ Image rejection: -28 dB, C-IM3 -60dBs, C-IM5 -70dBc.

- PC3 operation and 4dB post PA insertion losses.

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge, with 1 dB MPR- PC3 ACLR -30 dBc.

- The PA Vcc is varied at large output back-off (OBO) to account for commercial UE performance.

## 2.2 Measurement Results

The OBO measurements are performed both for dualPA and one PA architecture using four waveforms:

  - PCC/SCC UL CBW configuration: 5MHz+5MHz and Wgap=5MHz.

  - Using QPSK DFT-s-OFDM and QPSK CP-OFDM:

    - PCC/SCC UL RB allocation: 1RB + 1RB to verify the dualUL IMD3 emission levels.

    - Full RB allocation to verify the close-in PA emission levels.

- For dualPA architecture, we have verified that [20]dB is sufficient to pass the NS_12, NS_13, NS_14 and NS_15 requirements. CP-OFDM 1RB+1RB waveforms are the configuration that require the highest OBO.

- For one PA architecture, the forward IMD products lead to a worse OBO than the dual PA architecture: we observe that more than [20]dB OBO is needed.

We therefore emphasize that the [20]dB A-MPR allowance is only valid for dualPA architecture by using a copy of Table 5.5A.1-1 NOTE 7 into Table 6.2A.3.1.2-1.

Observation 2: For CA_n26(2A), 20dB A-MPR is needed for UE signaling dualPA architecture. A higher back-off is needed for UEs with 1PA architecture. Only dualPA architecture is assumed for UL CA_n26(2A). It is important to emphasize this assumption in the A-MPR Table 6.2A.3.1.2-1.

Proposal 2: For ULCA_n26(2A), A-MPR, we recommend adding the footnote highlighted in green to Table 6.2A.3.1.2-1.

“NOTE X: Only applicable for dual PA architecture. The UE shall indicate the dualPA-Architecture for UL CA_n26(2A).”

Table 2: Footnote addition to Table 6.2A.3.1.2-1 for CA_n26(2A)

| CA Network Signalling value | Requirements (clause) | Uplink CA Configuration | A-MPR for sub-blocks in order of increasing uplink carrier frequency |
| --- | --- | --- | --- |
|  |  |  | A-MPR [dB] (clause) |
| CA_NC_NS_01 | 6.5A.2.2.2 6.5A.3.2.2 | All applicable NR CA configurations | N/A |
| CA_NC_NS_04 | 6.5A.2.3.2.1 6.5A.3.3.2.1 | CA_n41(2A) | 6.2A.3.1.2.1 |
| CA_NC_NS_12X | 6.5A.3.3.2.2 | CA_n26(2A) | [20] dB |
| CA_NC_NS_13X | 6.5A.3.3.2.3 | CA_n26(2A) | [20] dB |
| CA_NC_NS_14X | 6.5A.3.3.2.4 | CA_n26(2A) | [20] dB |
| CA_NC_NS_15X | 6.5A.3.3.2.5 | CA_n26(2A) | [20] dB |
| CA_NC_NS_55, CA_NC_NS_57 | See CA_NC_NS_01 | CA_n77(2A) | See CA_NC_NS_01 |
| CA_NC_NS_100X | 6.5A.2.4.2.4 | CA_n26(2A) | [20] dB |
| NOTE X: Only applicable for dual PA architecture. The UE shall indicate the dualPA-Architecture for UL CA_n26(2A). |  |  |  |

# Conclusion

In this contribution, we propose changes to the CA_NC_NS_12, 13, 14, and 15 requirements based on observation 1, and captured in proposal 1, CR [1]. Our measurements confirm that [20]dB A-MPR is needed for dualPA architecture. We propose to add a footnote, ensuring that this A-MPR applies only for UEs that indicate the dualPA-Architecture for UL CA_n26(2A).

Observation 1: One-UL NS_12, 13, 14, and 15 emission requirements apply to dual-UL CA_n26(2A) BCS0. However, several CBWs specified in NS_12, 13, 14, and 15 are not supported by CA_n26(2A) BCS0. Corrections are needed in the CA_NC_NS_12, 13, 14, and 15 requirements to reflect the supported CBW specified in CA_n26(2A) BCS0.

Proposal 1: We suggest that RAN4 adopts the CR [1], following changes to the CA_NC_NS_12, 13, 14, and 15 requirements.

Observation 2: For CA_n26(2A), 20dB A-MPR is needed for UE signalling dualPA architecture. A higher back-off is needed for UEs with 1PA architecture. Only dualPA architecture is assumed for UL CA_n26(2A). It is important to emphasize this assumption in the A-MPR Table 6.2A.3.1.2-1.

Proposal 2: For UL CA_n26(2A) A-MPR, consider adding the footnote highlighted in green to Table 6.2A.3.1.2-1.

“NOTE X: Only applicable for dual PA architecture. The UE shall indicate the dualPA-Architecture for UL CA_n26(2A).”

Table 2: Footnote addition to Table 6.2A.3.1.2-1 for CA_n26(2A)

| CA Network Signalling value | Requirements (clause) | Uplink CA Configuration | A-MPR for sub-blocks in order of increasing uplink carrier frequency |
| --- | --- | --- | --- |
|  |  |  | A-MPR [dB] (clause) |
| CA_NC_NS_01 | 6.5A.2.2.2 6.5A.3.2.2 | All applicable NR CA configurations | N/A |
| CA_NC_NS_04 | 6.5A.2.3.2.1 6.5A.3.3.2.1 | CA_n41(2A) | 6.2A.3.1.2.1 |
| CA_NC_NS_12X | 6.5A.3.3.2.2 | CA_n26(2A) | [20] dB |
| CA_NC_NS_13X | 6.5A.3.3.2.3 | CA_n26(2A) | [20] dB |
| CA_NC_NS_14X | 6.5A.3.3.2.4 | CA_n26(2A) | [20] dB |
| CA_NC_NS_15X | 6.5A.3.3.2.5 | CA_n26(2A) | [20] dB |
| CA_NC_NS_55, CA_NC_NS_57 | See CA_NC_NS_01 | CA_n77(2A) | See CA_NC_NS_01 |
| CA_NC_NS_100X | 6.5A.2.4.2.4 | CA_n26(2A) | [20] dB |
| NOTE X: Only applicable for dual PA architecture. The UE shall indicate the dualPA-Architecture for UL CA_n26(2A). |  |  |  |

# References

- R4-2400362, CR to TS 38.101-1 Rel-18 CA_NC_NS_12-15, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions Inc.