3GPP TSG-RAN WG4 Meeting # 110									          R4-2400369

Athens, Greece, 16th February – 1st March 2024

Title: 	PC2 n13 NS_07

Agenda Item:	7.20

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Introduction

This contribution presents power amplifier (PA) measurements to verify Band n13 PC2 NS_07 simulation results [1, 2, 3].

# Discussion

Our measurements focus on verifying the 10MHz channel bandwidth (CBW) Region A1 A-MPR proposals from [1, 2, 3]. These are summarized in Table 1.

Table 1: Band n13 PC2 region A1 A-MPR proposals

| Modulation/Waveform | PC3 ref. | A1 [1] | A1 [2] | A1 [3] |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM QPSK | 12 | 15: PC3 + 3 | PC3 +2 to +2.5 | 12 (=PC3) |
| CP-OFDM QPSK | 14 | 17: PC3 + 3 | PC3 +2 to +3 | 14 (=PC3) |

The evaluation of the PA output-back-off (OBO) to meet the NS_07 requirements present at least three challenges:

- For some RB allocations, the simulated PC3 raw OBO exceeds the agreed upon PC3 A-MPR [1, 2]. Thus, it was proposed to evaluate the PC2 A-MPR, by comparing the difference between the simulated PC3 results and the simulated PC2 results, rather than formulate absolute OBO results,

- When OBO exceeds 6 to 8dB, measurements obtained by keeping the PA supply voltage constant may not be representative of worst-case smartphone performance,

- It is challenging to simulate the impact of PA-to-PA cross-coupling/reverse IMD for transmit diversity (TxD) 23+23dBm architectures.

It is important to note that RAN4 introduced an extra [2]dB A-MPR allowance. This accounts for the TxD challenges for Band n1 PC2 A-MPR. This allowance is captured in Note 7 of Table 6.2.3.1-1, and is reproduced in Table 2 below.

Table 2: Band n1 PC2 TxD NS_05, NS_05U, NS_48, NS_49 allowance – Table 6.2.3.1-1.

Observation 1: For band n1 PC2 A-MPR, an additional [2] dB allowance was agreed upon to account for TxD performance.

This paper consolidates measurements that can be used to compare the measured PC3 performance, vs 1x26dBm PC2, vs 23+23dBm PC2 architectures.

## 2.1 Power Amplifier Calibration

- Local Oscillator (LO) leakage: -28 dBc, IQ Image rejection: -28 dB.

- C-IM3 -60dBc, C-IM5 -70dBc.

- Power Class 3 (PC3) and PC2 operation and 4dB post PA insertion losses.

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, and 100 RB at the lower channel edge with 1 dB MPR, and:

  - for 1xPC3 and 2xPC3 (23+23dBm): ACLR -30 dBc,

  - for 1xPC2 (1x26dBm): ACLR -31 dBc.

- Variable Vcc whenever required.

## 2.2 Measurement Results

The measurement results are captured in Table 3, assuming 10dB antenna-to-antenna isolation for 23+23dBm.

Table 3: Measured Band n13 PA OBO for Region A1 – 10MHz CBW

| Modulation/Waveform | UL allocation | 1x26dBm | 23+23dBm |
| --- | --- | --- | --- |
| DFT-s-OFDM QPSK | 50(RBstart=0) | PC3 + 2.7dB | PC3 + 4dB |
| CP-OFDM QPSK | 52(RBstart=0) | PC3 + 2.9dB | PC3 + 4.5dB |

Observation 2: For region A1, 10MHz CBW, the measurements confirm simulation results from [1, 2]: For 1x26dBm PC2 PA: PC3+2.7dB and PC3 +2.9dB OBO is needed to meet the NS_07 requirements. For 2x23dBm PC3+PC3 (TxD) architectures, an additional 1.4dB to 1.6dB OBO is needed.

Based on observation 1 and 2, we suggest adopting the following proposal.

Proposal: In Table 6.2.3.1-1, attach Note 7 to the Band n13 NS_07 A-MPR requirement.

# Conclusion

In this contribution, we have presented measurement data to verify the Band n13 10MHz CBW output power back-off required to meet the NS_07 requirements. We make the following observations and proposal.

Observation 1: For band n1 PC2 A-MPR, an additional [2] dB allowance was agreed upon to account for TxD performance.

Observation 2: For region A1, 10MHz CBW, the measurements confirm simulation results from [1, 2]: For 1x26dBm PC2 PA: PC3+2.7dB and PC3 +2.9dB OBO is needed to meet the NS_07 requirements. For 2x23dBm PC3+PC3 (TxD) architectures, an additional 1.4dB to 1.6dB OBO is needed.

Proposal: In Table 6.2.3.1-1, attach Note 7 to the Band n13 NS_07 A-MPR requirement.

# References

- R4-2318750, A-MPR discussion for FDD HPUE NS_07, 3GPP TSG-RAN WGA4 Meeting # 109, Chicago, United States, Apple.

- R4- 2316475, PC2 A-MPR for NS_07, 3GPP TSG-RAN WGA4 Meeting # 108-bis, Xiamen, China, Huawei, HiSilicon.

- R4-2320652, PC2 A-MPR for bands n7, n13 and n28, 3GPP TSG-RAN WGA4 Meeting # 109, Chicago, United States, Qualcomm Inc.